3GPP TSG-RAN WG2 Meeting #127bis	R2-2407978

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.3.3

Source:	OPPO

Title:	Discussion on measurement event prediction

Document for:	Discussion, Decision

Introduction

In RAN2#125bis meeting, the measurement event prediction is briefly discussed and come to following agreements:

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

This use case is closely related to RRM measurement prediction and also RLF prediction use case. Now with the progress of both RRM measurement prediction and RLF prediction in past RAN2 meetings, some further clarification of this use case is needed.

Discussion

Definition

At RAN2#127 meeting the discussion on direct and indirect RLF prediction is rather controversial. The issue is similar for direct and indirect measurement event prediction considering they are both classification issue. During simulation, measurement event may relatively be easy to trigger hence the model could be a bit easier to be trained. But compared to measurement results, measurement event is still less possible case. While for indirect measurement event prediction, what have been evaluated for RRM measurement prediction use case can be re-used as much as possible. In order to make progress on measurement event prediction use case, RAN2 should focus on indirect measurement event prediction evaluation.

Proposal 1: RAN2 focus on evaluation of indirect measurement event prediction

The indirect measurement event prediction could be illustrated in Figure 2.1-1:

Figure 2.1-1 Indirect measurement event prediction

In Figure 2.1-1, the model is used to predicted L3 RSRP based on actual measurement results of neighbouring cell and serving cell respectively for intra-frequency case. And the post processing unit will process the L3 RSRP results from both serving cell and neighbouring cell. It follows the specified way to judge measurement event A3. Because actual and/or predicted L3 RSRP measurement results will be available just like legacy L3 RSRP measurement results in time domain, the predicted measurement event should be at one time instance in future.

Proposal 2: The output of the post processing unit is a predicted measurement event at one time instance

Considering the intention is to predict measurement event in advance so that network can learn and prepare the handover procedure in network side before the right time to switch the UE from source to target cell, the RRM measurement prediction model in Figure 2.1-1 should be intra-frequency temporal domain case A. As for current inter-frequency prediction, the prediction can be only done in frequency domain i.e. the predicted measurement results of another frequency can’t be predicted in advance. In order to support inter-frequency neighbouring cell in Figure 2.1-1, the prediction of inter-frequency neighbouring cell should also occur within the same neighbouring frequency. Then technically for inter-frequency neighbouring cell, the prediction itself is still done in the same frequency. But it also means models of two frequencies should run in parallel. One more frequency demands one more data set and they likely need be trained separately. For the sake of progress, RAN2 should focus on intra-frequency prediction first.

Proposal 3: Model of RRM measurement prediction for intra-frequency temporal domain case A is reused for RRM measurement event prediction

Proposal 4: RAN2 focus on intra-frequency neighbouring cell for measurement event prediction

Ideally if prediction window (PW) is long enough e.g. longer than TTT, then the measurement event can be predicted purely based on predicted measurement result as illustrated in Figure 2.1-2:

Figure 2.1-2

The post processing model in Figure 2.1-1 need collect enough predicted L3 RSRP to judge whether measurement event A3 occur or not. Considering the PW may be shorter than TTT (as illustrated in Figure 2.1-3), there are two ways for the post processing unit to collect the L3 RSRP measurement results, which could be predicted by model and later on replaced by actual measurement:

Option 1: To replace predicted L3 RSRP measurement results, if actually L3 RSRP measurement results are available when observation window (OW) is moving forward

Option 2: To collect predicted L3 RSRP measurement results only, even if actually L3 RSRP measurement results are available when observation window (OW) is moving forward

Considering actual L3 RSRP measurement results are most likely more accurate than predicted measurement results, option 1 is better choice.

Figure 2.1-3

Proposal 5: Measurement event is derived based on predicted L3 RSRP results and optional actual historical L3 RSRP results from both serving cell and one neighbouring cell

The Figure 2.1-1 only show how to predict measurement event for one neighbouring cell. In order to improve the simulation efficiency, it is also possible that multiple neighbouring cells can be predicted so that measurement events can be derived simultaneously by the post processing unit in Figure 2.1-1. But it is sort of implementation issue and can leave it to company’s implementation.

Proposal 6: How to predict measurement event for more than one neighbouring cells could be company’s implementation.

Methodology

For event A3, the (predicted) measurement results from serving cell must be there otherwise it doesn’t work at all. During simulation a parallel hypothetical HO procedure should run so that serving cell can be identified properly. The hypothetical HO procedure could run based on measurement event A3, which is purely based on actual measurement result of serving cell and some neighbouring cells. To avoid simulating any detail HO signalling procedure over the Uu interface, it could be assumed that an HO will occur with fixed preparation time and execution time after a real measurement event A3 is triggered.

Proposal 7: A hypothetical HO procedure based on measurement event A3 is running to help identify serving cell during simulation

Metrics

The agreed metrics for indirect RLF at RAN2#127 meeting is as following:

Agreements on KPI

For indirect

-	SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction.

Table 2.3-1

The metrics for indirect measurement event can be similar considering there are similar classification issue. But the metrics in table 2.3-1 is bit messy. Actually, those metrics can be combined within same one i.e. F1 score and the rest metrics can be seen as component of the F1 score.

| actual event\predicted event | Negative | Positive |
| --- | --- | --- |
| Negative | n0 | n1 |
| Positive | n2 | n3 |

Table 2.3-2 Illustration of the prediction results

In table 2.3-2, there are 4 counters defined as follows:

- Counter n0: number of cases, where no event is predicted and no event occurs in baseline

- Counter n1: number of cases, where an event is predicted but no event occurs in baseline

- Counter n2: number of cases, where no event is predicted but an event occurs in baseline

- Counter n3: number of cases, where an event is predicted and an event occurs in baseline

For measurement event prediction, counter n1 could be interpreted as “False measurement event detection”. Counter n2 could be interpreted as “Missed measurement event prediction” Counter n3 could be interpreted as “True measurement event prediction”.

If an event is predicted but far away from the time when the event occurs in baseline case, then it doesn’t make sense because the time point when event occur is the critical part of the event. On the other hand, it will be very difficult for a model to predict a measurement event at the exact same time instance as real event occurs in baseline scenario. So, a true measurement event is predicted only when the predicted time point and the time point when it occurs in baseline case is very close e.g., less than a single sampling period or measurement period. Based on this understanding, the counter n1/n2/n3 could be illustrated with following Figures:

Figure 2.3-1 counter n3           Figure 2.3-2 counter n2			Figure 2.3-3 counter n1

In Figure 2.3-1 when the time gap between baseline event and predicted event is lower than allowed maximum time gap (say MATG), counter n3 increases by one. In Figure 2.3-2, if a baseline event occurs and there is no predicted event within MATG before and after this event, counter n2 increases by one. In Figure 2.3-3, if an event is predicted and there is no baseline event within MATG before and after this predicted event, counter n1 increases by one. Obviously here the time gap is the similar to the metric “time difference of true time RLF and predicted RLF” for RLF event.

The accuracy formulas are:

- Precision = n3/(n1+n3)

- Recall = n3/(n2+n3)

- F1 score = 2*Precision*Recall/ (Precision+ Recall)

So clearly if F1 score is taken as metrics, the time gap and counter n1/n2/n3 can be component of F1 score and hence not necessary to define them as separate metrics.

Proposal 8: Measurement event prediction take F1 score as metric

Proposal 9: To calculate F1 score, the related counter n1(i.e. false measurement event detection), n2(i.e. missed measurement event prediction) and n3 (i.e. true measurement event prediction) should be defined as following:

- Counter n1: the counter increased by one when no real measurement event occurs within a predefined gap around a predicted measurement event

- Counter n2: the counter increased by one when no measurement event is predicted within a predefined gap around a real measurement event

- Counter n3: the counter increased by one when a real measurement event occurs within a predefined gap around a predicted measurement event

Simulation assumptions

As proposal 3 suggests the simulation assumption of RRM measurement prediction for temporal domain case A (FR2) can be reused for measurement event prediction.

Proposal 10: The Simulation assumption of RRM measurement prediction for temporal domain case A (FR2) can be reused.

During handover, it is crucial to consider the RSRPs during the Time To Trigger (TTT). According to TR 36.839, the typical TTT can be set as 40ms, 80ms, 160ms, and 480ms. If we adopt the non-sliding filtering for measurement event prediction, we only have one sample every 400ms, which is larger than 40ms/80ms/160ms/320ms. It is unclear how we can determine if the A3 event is satisfied during a 40ms ,80ms ,160ms or 320ms TTT. This issue does not arise with sliding filtering, as it can make predictions on a smaller scale and the minimum one is 80ms i.e. one sampling period

Proposal 11: The sliding filtering option is adopted as the baseline for measurement event prediction.

In our paper [1] it is shown that sub-case 2 outperforms sub-use case 1 and 3 in temporal domain case A. Since we propose to reuse model of temporal domain case A , we propose that RAN2 can focus on sub-case 2 for measurement event prediction.

Proposal 12: To focus on sub-case 2 for measurement event prediction

In addition, the post processing unit should be configured with parameters to derive measurement event A3. And those parameters can be also used for the hypothetical HO procedure in proposal 7. At RAN2#127 meeting many companies propose to reuse those parameters from 36.839. In TR 36.839, specific parameters given for A3-offset are -1 dB, 0 dB, 1 dB, 2 dB, and 3 dB. Most companies seem to agree that 2 dB is a good choice. TTT is recommended to be 40ms, 80ms, 160ms, and 480ms in the TR. To make the HO easier to happen, a small TTT is better. Therefore, we can use 80ms (as FR2 sampling period of sliding filtering is 80ms). In TR 36.839, handover preparation time (50ms) and handover execution time (40ms) are discussed separately. In measurement event prediction, that is not necessary and we can combine them into one parameter (i.e., handover preparation and execution time, HPET). Given that the sample period for FR2 is redefined as 80ms for sliding filtering, HPET is better to be an integer multiple 80ms. Therefore, we can simply use 80ms for HPET. The recommend value for each parameter are listed in Table 2.4-1.

| Parameters | values |
| --- | --- |
| A3 event offset | 2 db (baseline), 0dB |
| TTT | 80ms (baseline), 160ms, 320ms, 480ms |
| Preparation and execution time | 80ms |

Table 2.4-1

Proposal 13: To agree on HO parameters in table 2.4-1 as simulation parameters for measurement event prediction

Conclusion

Proposal 1: RAN2 focus on evaluation of indirect measurement event prediction

Proposal 2: The output of the post processing unit is a predicted measurement event at one time instance

Proposal 3: Model of RRM measurement prediction for intra-frequency temporal domain case A is reused for RRM measurement event prediction

Proposal 4: RAN2 focus on intra-frequency neighbouring cell for measurement event prediction

Proposal 5: Measurement event is derived based on predicted L3 RSRP results and optional actual historical L3 RSRP results from both serving cell and one neighbouring cell

Proposal 6: How to predict measurement event for more than one neighbouring cells could be company’s implementation.

Proposal 7: A hypothetical HO procedure based on measurement event A3 is running to help identify serving cell during simulation

Proposal 8: Measurement event prediction take F1 score as metric

Proposal 9: To calculate F1 score, the related counter n1(i.e. false measurement event detection), n2(i.e. missed measurement event prediction) and n3 (i.e. true measurement event prediction) should be defined as following:

- Counter n1: the counter increased by one when no real measurement event occurs within a predefined gap around a predicted measurement event

- Counter n2: the counter increased by one when no measurement event is predicted within a predefined gap around a real measurement event

- Counter n3: the counter increased by one when a real measurement event occurs within a predefined gap around a predicted measurement event

Proposal 10: The Simulation assumption of RRM measurement prediction for temporal domain case A (FR2) can be reused.

Proposal 11: The sliding filtering option is adopted as the baseline for measurement event prediction.

Proposal 12: To focus on sub-case 2 for measurement event prediction

Proposal 13: To agree on HO parameters in table 2.4-1 as simulation parameters for measurement event prediction

Reference

[1] R2-2407977 Discussion on simulation result of RRM measurement OPPO