3GPP TSG-RAN WG2 Meeting #127bis	R2-2407979

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	Simulation results for RRM measurement prediction

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#126 meeting, RAN2 agreed on the baseline of simulation assumptions and the methodology for the RRM measurement prediction. As to the potential scenarios, cases 2,3,4 in the following table are prioritized.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

At the RAN2#127 meeting, more detailed simulation assumptions for RRM measurement prediction were investigated.

In this contribution, we will provide initial simulation results on the agreed prioritized scenarios and agreed assumptions.

- Discussion

  - Methodology of simulation

    - Clarification of Case B and filtering option

At the last meeting, RAN2 discussed and agreed on two L1 filtering options, i.e., sliding and non-sliding L1 filtering. The option of non-sliding filtering corresponds to the maximum measurement period requirement specified by RAN4, which means that one Layer 1/3 filtered RSRP value can be derived from every 5 L1 samples, e.g., for FR2, the maximum length of measurement period can be 5*80ms=400ms.

However, this maximum requirement cannot meet the need in certain scenarios that require more real-time monitoring of channel signal quality, e.g., TTT=320ms. Therefore, the option of sliding L1 filtering with smaller intervals to generate L3 measurement results is necessary.

Agreements 1 To keep two filtering options on the table and up to company to report.

Figure 1 Sliding L1/L3 filtering Figure 2: non-sliding L1/L3 filtering In sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every sample period. In non-sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every measurement period. In both L1/L3 filtering options, the filtered L1 measurement result is obtained based on the non-filtered L1 measurement results within one measurement period. And the filtered L3 measurement result is obtained as specified in section 5.5.3.2 of TS 38.331

In addition, for the measurement reduction Case B, two patterns of input and output relationships have been introduced. In example 1, the input instances within the observation window are temporally continuous measurements; While in example 2, the observation window includes interleaved instances that are skipped for measurement. To our understanding, the difference between example 1 and example 2 is the length of the observation window, i.e., example 2 has the longer observation window which covers the previous prediction window(s). Since example 2 has better prediction accuracy and the pattern is similar to the relaxed measurement, example 2 is selected as the baseline in our simulation.

Figure 2.1-1 temporal domain case example 1 Figure 2.1-2 temporal domain case example 2 Proposal 1: Both example 1 in Figure 2.1-1 and example 2 in Figure 2.1-2 can be used for temporal domain case B prediction. Example 2 in Figure 2.1-2 is recommended as baseline and company can report if they adopt example 1 pattern. Proposal 2: To agree on following definition: Intra-frequency temporal domain case B: In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding Note: The historical measurement results in observation window are at least actual measurement results. And company can report it, if they use predicted measurement results (if any) in observation window as input of model also.

However, we found that certain combinations of L1/L3 filtering options and the sub-use cases may lead to feasibility issues.

Sliding L1/L3 filtering

Figure 2.1-1 Sub-use case 1 (input & output: L1 sample without L1 filtering)

In sub-use case 1, L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated. For this sub-use case, the sliding L1/L3 filtering is feasible. Taking the L3 filtered RSRP instances 05 and 06 in Figure 2.1-1 as examples:

- For the pattern of input and output, the input is the measured L1 samples in even numbers (i.e., 00, 02, 04, 06…), and the output is the predicted L1 samples in odd numbers (i.e., 01, 03, 05, 07…). Note that both input and output are L1 beam-level measurements without L1 filtering.

- The L3 filtered RSRP in instance 05 can be derived based on measured L1 samples 02&04 and predicted L1 samples 01&03&05.

- The L3 filtered RSRP in instance 06 can be derived based on measured L1 samples 02&04&06 and predicted L1 samples 03&05.

Figure 2.1-2 Sub-use case 2

In sub-use case 2, cell-level measurement result(s) is predicted based on actual cell-level measurement result(s). For this sub-use case, the sliding L1/L3 filtering is not feasible. Taking the L3 filtered RSRP instances 05 and 06 in Figure 2.1-2 as examples:

- For the pattern of input and output, the input is the L3 filtered RSRP instances in even numbers (i.e., 00, 02, 04, 06…), and the output is the predicted L3 filtered RSRP instances in odd numbers (i.e., 01, 03, 05, 07…).

- If the L3 filtered RSRP measurement in instance 05 is skipped, the L1 samples 01-05 measurements can be skipped accordingly.

- However, to derive the L3-filtered RSRP in instance 06, the measurement of L1 samples 02-05 is still needed other than the measurement of L1 sample 06. Similarly, for the L3 filtered RSRP instance 08, the measurement of L1 samples 04-08 is still needed. As a result, no measurement of the L1 sample can be skipped.

Figure 2.1-3 Sub-use case 3

In sub-use case 3, Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s). For this sub-use case, the sliding L1/L3 filtering is not feasible either. Taking the L3 filtered RSRP instances 05 and 06 in Figure 2.1-3 as examples:

- For the pattern of input and output, the input is the measured L1 samples in even numbers (i.e., 00, 02, 04, 06…), and the output is the predicted L3 filtered RSRP instances in odd numbers (i.e., 01, 03, 05, 07…).

- The L3 filtered RSRP in instance 05 can be predicted based on L1 samples 00&02&04, and the L1 samples 01&03&05 can be skipped accordingly.

- However, the L3 filtered RSRP in instance 06 can not be derived based on L1 samples 02&04&06 since the measurement of L1 samples 03&05 is skipped.

Proposal 1: For measurement reduction Case B, sliding L1/L3 filtering is only applicable for sub-use case 1 when both input and output are L1 beam-level measurements without L1 filtering.

Non-sliding L1/L3 filtering

Figure 2.1-4(a) Sub-use case 1 (input & output: L1 sample without L1 filtering)

Figure 2.1-4(b) Sub-use case 1 (input & output: L1 filtered RSRP)

In sub-use case 1, L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated. For this sub-use case, when the input and output are L1 samples, the non-sliding L1/L3 filtering is applicable in Figure 2.1-4(a):

- For the pattern of input and output, the input is the measured L1 samples in even numbers (i.e., 00, 02, 04, 06…), and the output is the predicted L1 samples in odd numbers (i.e., 01, 03, 05, 07…).

- The L3 filtered RSRP in instance 04 can be derived based on measured L1 samples 00&02&04 and predicted L1 samples 01&03.

When the input and output are L1 filtered RSRP, the combination is still feasible and the pattern is in Figure 2.1-4(b).

Figure 2.1-5 Sub-use case 2 (input: L3 filtered RSRP)

In sub-use case 2, Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s). For this sub-use case, the non-sliding L1/L3 filtering is feasible. Taking the L3 filtered RSRP instances 19 in Figure 2.1-5 as examples:

- For the pattern of input and output, the input is the L3 filtered RSRP instances in N*10+4 (i.e., 04, 14, …), and the output is the predicted L3 filtered RSRP instances in N*10+9 (i.e., 09, 19, …).

- For the skipped L3 filtered RSRP measurement instances, the measurement of L1 samples can be skipped accordingly. For instance, the L3 filtered RSRP is predicted based on L3 filtered RSRP instances 04 and 14, and the measurement of L1 samples instances 15-19 can be skipped.

Figure 2.1-6 Sub-use case 3 (input: L1 filtered RSRP)

In sub-use case 3, Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s). For this sub-use case, the non-sliding L1/L3 filtering is feasible. Taking the L3 filtered RSRP instances 19 in Figure 2.1-5 as examples:

- For the pattern of input and output, the input is the L1 filtered RSRP instances in N*10+4 (i.e., 04, 14, …), and the output is the predicted L3 filtered RSRP instances in N*10+9 (i.e., 09, 19, …).

- For the skipped L1 filtered RSRP measurement instances, the measurement of L1 samples can be skipped accordingly. For instance, the L3 filtered RSRP can be predicted based on L1 filtered RSRP instances 04 and 14. And the measurement of L1 samples instances 15-19 can be skipped.

Proposal 2: For measurement reduction Case B, non-sliding L1/L3 filtering is applicable for all sub-use cases.

    - Pattern of inter-frequency prediction

At RAN2#127 meeting, for pattern of temporal domain case B prediction, it was clarified that the prediction is always after the measurement and is at future time instance(s).

Agreements

=>	For temporal domain case B prediction the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s).

For frequency domain prediction with measurement reduction purposes, the pattern/ timing relationship between the measurement (on frequency 1) and prediction (on frequency 2) should also be clarified.

From our understanding, the following two options can be considered:

-	Option 1: The input is historical measurement values on Frequency 1(e.g., instances 1-10) and the output is the value on Frequency 2 at the last time instance (e.g., instances 10) of measurement on Frequency 1.

-	Option 2: The input is historical measurement values on frequency 1 (e.g., instances 1-10) and the output is the values on Frequency 2 at the same time instances (e.g., instances 1-10) of measurement on Frequency 1.

Figure 2.1-7(a) Pattern of inter-frequency prediction of Option 1

Figure 2.1-7(b) Pattern of inter-frequency prediction of Option 2

The advantage of Option 2 is that multiple instances on Frequency 2 can be out for one single model inference. On the contrary, multiple model inferences are needed for Option 1 to obtain multiple predicted instances on Frequency 2. However, the typical usage of inter-frequency prediction is inter-frequency handover/redirection, in which Option 2 is not applicable. For instance, extra latency of historical prediction at Frequency 2 may lead to too-late handover.

Therefore, the pattern of Option 1 should be the baseline for inter-frequency prediction.

Proposal 3: For frequency domain prediction for measurement reduction, the input is a set of historical measurement instances on Frequency 1 and the output is the predicted value on Frequency 2 at the time of the last measurement instance on Frequency 1.

    - Clarification of spatial domain prediction

In the previous RAN2 meetings, an issue was raised that, it is unclear how to derive cell quality based on both predicted beam qualities and measured beam qualities in the case of spatial RRM prediction. From our understanding, predicted beam qualities are different from measured beam qualities considering predicted beam qualities may be inaccurate compared to measured beam qualities. It is unclear whether predicted beam qualities can directly be used together with measured beam qualities for beam consolidation/selection. We therefore suggest RAN2 to discuss this issue.

Proposal 4: For spatial domain prediction, RAN2 to discuss how to derive cell-level quality based on predicted beam qualities and measured beam qualities.

  - FR1 to FR1 intra-frequency (temporal domain)

Based on the analysis in section 2.1.1, example 2 is selected as the pattern for measurement reduction in our simulation. Additionally, both sliding L1/L3 filtering and non-sliding L1/L3 filtering are considered in our simulations for the applicable sub-use cases.

Furthermore, in the email discussion [2], it was agreed that for intra-frequency temporal domain case B, the value range of observation window (OW) is {40ms~2000ms} and the value range of prediction window (PW) is {40ms~800ms}. The following simulation will select reasonable values from the candidates based on the characteristics of sub-use cases.

    - Sliding L1/L3 filtering (Sub-use case 1)

For sub-use case 1 with sliding L1/L3 filtering, we select the pattern of inputs and outputs shown in Figure 2.1-1, where the PW length is the L1 sample period multiplied by the number of L1 samples, i.e., 40ms*N. For MRRT of 1/2, N=1; for MRRT of 4/5, N=4.

Our simulation focuses on the impact on prediction accuracy from three aspects: UE speed, measurement reduction rate (MRRT), and OW length.

The results of prediction accuracy (L1 sample RSRP) for different combinations of UE speed and OW length under MRRT of 1/2 and 4/5 are shown in Table 2.2-1 and Table 2.2-2.

Table 2.2-1 L1 RSRP difference of sub-use case 1 (sliding L1/L3 filtering, MRRT=1/2)

| OW length (ms) | 6*40 | 8*40 | 10*40 | 20*40 |
| --- | --- | --- | --- | --- |
| Number of samples within OW | 6 | 8 | 10 | 20 |
| Number of samples as input | 3 | 4 | 5 | 10 |
| Prediction accuracy (30km/h) | 0.18 | 0.171 | 0.173 | 0.16 |
| Prediction accuracy (60km/h) | 0.352 | 0.342 | 0.34 | 0.329 |
| Prediction accuracy (90km/h) | 0.465 | 0.454 | 0.456 | 0.452 |

Table 2.2-2 L1 RSRP of sub-use case 1 (sliding L1/L3 filtering, MRRT =4/5)

| OW length (ms) | 10*40 | 20*40 | 25*40 | 50*40 |
| --- | --- | --- | --- | --- |
| Number of samples within OW | 10 | 20 | 25 | 50 |
| Number of samples as input | 2 | 4 | 5 | 10 |
| Prediction accuracy (30km/h) | 0.47 | 0.411 | 0.414 | 0.413 |
| Prediction accuracy (60km/h) | 0.797 | 0.711 | 0.705 | 0.706 |
| Prediction accuracy (90km/h) | 1.134 | 1.028 | 1.028 | 1.027 |

It can be observed that for sliding L1 filtering, the prediction accuracy is mostly quite small, i.e., less than 1dB. This is because the RSRP change is relatively small within a short prediction window. In this case, although the prediction accuracy value is relatively small, it does not necessarily indicate a high-quality AI/ML prediction.

Therefore, when providing the simulation results of the prediction accuracy, the RSRP change between the actual RSRP at the prediction time (within the prediction window) and the last RSRP measurement in the observation window should also be provided to assess the prediction accuracy.

Proposal 5: For temporal domain RRM prediction, the actual RSRP difference between the actual RSRP within the prediction window and the last RSRP measurement in the observation window should be provided to assess the prediction accuracy.

The actual RSRP difference of MRTT=1/2 and 4/5 is in Table 2.2-3.

Table 2.2-3 Actual L1 RSRP difference of Sliding L1/L3 filtering

| UE speed | Actual L1 RSRP difference (MRTT= 1/2) | Actual L1 RSRP difference (MRTT= 4/5) |
| --- | --- | --- |
| 30km/h | 0.184 | 0.451 |
| 60km/h | 0.350 | 0.801 |
| 90km/h | 0.529 | 1.136 |

From Table 2.2-1 to Table 2.2-3, it can be seen that the prediction accuracy is quite close to the actual RSRP difference. This is because the L1 sample values are unstable and the corresponding rule of RSRP change is not clear, so a longer observation window does not provide more information to improve prediction accuracy. That is, the AI model primarily relies on the last RSRP measurement in the observation window to generate the prediction.

Observation 1a: For temporal domain prediction Case B, if the model input is L1 samples (without filtering), the prediction accuracy is quite close to the actual RSRP difference.

    - Non-Sliding L1/L3 filtering (Sub-use cases 1/2/3)

For the non-sliding L1/L3 filtering scenario, all the Cases are considered:

- For Sub-use case 1, the pattern of inputs and outputs is shown in Figure 2.1-4(b), and the PW length is the measurement of L1 filtered RSRP measurement period multiplied by the number of L1 filtered RSRP, i.e., 200ms*N, For MRRT of 1/2, N=1; for MRRT of 4/5, N=4.

- For Sub-use case 2, the pattern of inputs and outputs is shown in Figure 2.1-5, and the PW length is the measurement of L3 filtered RSRP measurement period multiplied by the number of L3 filtered RSRP, i.e., 200ms*N, For MRRT of 1/2, N=1; for MRRT of 4/5, N=4.

- For Sub-use case 3, the pattern of inputs and outputs is shown in Figure 2.1-6, and the PW length is the measurement of L3 filtered RSRP measurement period multiplied by the number of L3 filtered RSRP, i.e., 200ms*N, For MRRT of 1/2, N=1; for MRRT of 4/5, N=4.

For Case 1, the simulation results of prediction accuracy for different combinations of UE speed and OW length under MRRT of 1/2 and 4/5 are shown in Table 2.2-4 and Table 2.2-5. And the actual L3 RSRP is in Table 2.2-6.

Table 2.2-4 Simulation results of sub-use case 1 (Non-Sliding L1/L3 filtering, MRTT=1/2)

| OW length (ms) | 4*200 | 6*200 | 8*200 | 10*200 |
| --- | --- | --- | --- | --- |
| Number of samples within OW | 4 | 6 | 8 | 10 |
| Number of samples as input | 2 | 3 | 4 | 5 |
| Prediction accuracy (30km/h) | 0.590 | 0.553 | 0.512 | 0.503 |
| Prediction accuracy (60km/h) | 1.138 | 1.050 | 0.952 | 0.914 |
| Prediction accuracy (90km/h) | 1.465 | 1.332 | 1.208 | 1.117 |

Table 2.2-5 Simulation results of sub-use case 1 (Non-Sliding L1/L3 filtering, MRTT=4/5)

| OW length (ms) | 10*200 |
| --- | --- |
| Number of samples within OW | 10 |
| Number of samples as input | 2 |
| Prediction accuracy (30km/h) | 2.482 |
| Prediction accuracy (60km/h) | 4.147 |
| Prediction accuracy (90km/h) | 5.088 |

Table 2.2-6 Actual L3 RSRP difference of Non-Sliding L1/L3 filtering

| UE speed | Actual RSRP difference (MRTT= 1/2) | Actual RSRP difference (MRTT= 4/5) |
| --- | --- | --- |
| 30km/h | 0.828 | 2.038 |
| 60km/h | 1.448 | 3.483 |
| 90km/h | 1.930 | 4.676 |

For sub-use case 1 with non-sliding L1/L3 filtering, unlike the sliding approach, L1 filtered RSRP is selected as input and output. From the above Table 2.2-4, it can be observed that the increase in OW length can improve prediction accuracy, and the prediction accuracy is smaller than the actual RSRP difference in Table 2.2-6. Compared to L1 sample, L1 filtered RSRP is more stable, therefore, a longer observation window can provide more information to improve prediction accuracy.

Observation 1b: For temporal domain prediction Case B, if the model input is L1 filtered RSRP, increasing OW length can improve prediction accuracy.

Proposal 6: The L1 sample (without filtering) is not expected to be the model input for RRM measurement prediction.

In addition, under the same OW length, prediction accuracy decreases with an increase in UE speed. This is because, with a higher UE speed, the variation in RSRP becomes more pronounced (as in Table 2.2-6).

Observation 2: For temporal domain prediction Case B, as the UE speed increases, prediction accuracy decreases.

By comparing Table 2.2-4 and Table 2.2-5, it can be seen that as the MRRT increases, i.e., more skipped measurement samples, there is a significant decrease in prediction accuracy. For example, in sub-use case 1, corresponding to speeds of 30km/h to 90km/h, the prediction accuracy range is 2.482~5.088 dB, which is quite close to the actual RSRP difference (2.038~4.676 dB). This is mainly due to two reasons: first, as the MRRT increases, the prediction window becomes longer, and the gap between the predicted value and the actual measured value in the temporal domain also increases; second, as the MRRT increases, the actual measurements within the same observation window length decrease, resulting in a decrease in the input information.

In beam management, the typical prediction accuracy threshold is 1dB. RAN2 needs to discuss how to determine whether prediction results with an accuracy exceeding 1dB are acceptable. We believe there are two possible methods:

- RAN4 evaluates the threshold for prediction accuracy.

- RAN2 introduces system-level performance simulation and the performance degradation threshold can be provided by the operators. The system-level performance KPIs may include short ToS rate and HOF rate to reflect too-early HO/HO to wrong cell and too-late HO.

Observation 3: For temporal domain prediction Case B, as the MRRT increases, prediction accuracy decreases.

Proposal 7: RAN2 to discuss how to determine acceptable prediction accuracy when RRM measurement prediction is introduced for measurement reduction. The following two options can be considered:

- Option 1: Request RAN4 to evaluate the requirement for prediction accuracy.

- Option 2: RAN2 introduces system-level performance (e.g., short ToS rate and HOF rate) simulation and the performance degradation threshold can be provided by the operators.

The simulation results for sub-use case 2 and sub-use case 3 are shown in Tables 2.2-7 to Tables 2.2-10.

Table 2.2-7 Simulation results of sub-use case 2 (Non-Sliding L1/L3 filtering, MRTT=1/2)

| OW length (ms) | 4*200 | 6*200 | 8*200 | 10*200 |
| --- | --- | --- | --- | --- |
| Number of samples within OW | 4 | 6 | 8 | 10 |
| Number of samples as input | 2 | 3 | 4 | 5 |
| Prediction accuracy (30km/h) | 0.959 | 0.868 | 0.823 | 0.806 |
| Prediction accuracy (60km/h) | 1.763 | 1.639 | 1.564 | 1.528 |
| Prediction accuracy (90km/h) | 2.404 | 2.209 | 2.131 | 2.022 |

Table 2.2-8 Simulation results of sub-use case 2 (Non-Sliding L1/L3 filtering, MRTT=4/5)

| OW length (ms) | 10*200 |
| --- | --- |
| Number of samples within OW | 10 |
| Number of samples as input | 2 |
| Prediction accuracy (30km/h) | 2.480 |
| Prediction accuracy (60km/h) | 4.236 |
| Prediction accuracy (90km/h) | 5.481 |

Table 2.2-9 Simulation results of sub-use case 3 (Non-Sliding L1/L3 filtering, MRTT=1/2)

| OW length (ms) | 4*200 | 6*200 | 8*200 | 10*200 |
| --- | --- | --- | --- | --- |
| Number of samples within OW | 4 | 6 | 8 | 10 |
| Number of samples as input | 2 | 3 | 4 | 5 |
| Prediction accuracy (30km/h) | 0.660 | 0.625 | 0.594 | 0.571 |
| Prediction accuracy (60km/h) | 1.280 | 1.206 | 1.143 | 1.093 |
| Prediction accuracy (90km/h) | 1.745 | 1.628 | 1.552 | 1.493 |

Table 2.2-10 Simulation results of sub-use case 3 (Non-Sliding L1/L3 filtering, MRTT=4/5)

| OW length (ms) | 10*200 |
| --- | --- |
| Number of samples within OW | 10 |
| Number of samples as input | 2 |
| Prediction accuracy (30km/h) | 1.856 |
| Prediction accuracy (60km/h) | 3.353 |
| Prediction accuracy (90km/h) | 4.337 |

For the same OW length, the performance of sub-use case 1 and sub-use case 3 is better than that of sub-use case 2. This is mainly because the input for sub-use cases 1 and 3 is beam level, while the input for sub-use case 2 is at the cell level. That is to say, the input information for sub-use cases 1 and 3 is more extensive than that of sub-use case 2, leading to more accurate predictions.

Observation 4: For temporal domain prediction Case B, the performance of sub-use case 1 and sub-use case 3 is better than that of sub-use case 2.

Proposal 8: For each sub-use case of temporal domain prediction Case B, capture the following aspects in the TR regarding their impact on prediction accuracy: UE speed, MRRT, and OW length.

  - FR1 to FR1 inter-frequency (frequency domain)

For the inter-frequency simulation, we adopt the pattern of inputs and outputs shown in Figure 2.1-7(a).

    - Co-located scenario

Our simulation focuses on the impact on prediction accuracy from three aspects: UE speed and OW length.

For the prediction from low-frequency (2GHz) to high-frequency (4GHz), the results of prediction accuracy for different combinations of UE speed and OW length are shown in Table 2.3-1.

Table 2.3-1 Simulation results of prediction from 2GHz to 4GHz

| OW length (ms) | 5*40 | 10*40 | 20*40 | 30*40 | 40*40 | 50*40 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of samples within OW | 5 | 10 | 20 | 30 | 40 | 50 |
| Prediction accuracy (30km/h) | 1.746 | 1.483 | 1.153 | 0.932 | 0.823 | 0.721 |
| Prediction accuracy (60km/h) | 2.218 | 1.980 | 1.599 | 1.364 | 1.215 | 1.099 |
| Prediction accuracy (90km/h) | 2.419 | 2.186 | 1.867 | 1.603 | 1.418 | 1.356 |

Similar to the time-domain simulation in section 2.2, under the same OW length, prediction accuracy decreases with an increase in UE speed. At the same time, for the same UE speed, the increase in OW length can improve prediction accuracy.

Observation 5: For frequency domain prediction, increasing OW length can improve prediction accuracy.

Observation 6: For frequency domain prediction, as the UE speed increases, prediction accuracy decreases.

Another potential influencing factor is the direction of prediction, i.e., prediction from low-frequency cell to high-frequency cell or prediction from high-frequency cell to low-frequency cell. As seen from Table 2.3-2, for the same UE speed and OW length, the prediction accuracy of prediction from low frequency to high frequency is slightly better than that of prediction from high frequency to low frequency, but the difference in performance between the two is not significant.

Table 2.3-2 Simulation results of prediction from 4GHz to 2GHz

| OW length (ms) | 5*40 | 10*40 | 20*40 | 30*40 | 40*40 | 50*40 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of samples within OW | 5 | 10 | 20 | 30 | 40 | 50 |
| Prediction accuracy (30km/h) | 1.855 | 1.600 | 1.277 | 1.071 | 0.937 | 0.818 |
| Prediction accuracy (60km/h) | 2.375 | 2.094 | 1.725 | 1.516 | 1.350 | 1.213 |
| Prediction accuracy (90km/h) | 2.507 | 2.264 | 1.892 | 1.715 | 1.553 | 1.436 |

Observation 7: For frequency domain prediction within FR1, the prediction accuracy of prediction from low frequency to high frequency and prediction from high frequency to low frequency is almost the same.

Proposal 9: For frequency domain prediction, capture the following aspects in the TR regarding their impact on prediction accuracy: UE speed and OW length.

    - Consideration of non-colocated

With the UE moves, the typical RSRP values of the co-located inter-frequency cells are shown in the figure below. The sudden changes in RSRP are due to the transition from Line-of-Sight (LOS) to Non-Line-of-Sight (NLOS). During LOS or NLOS periods, the RSRP difference between the two cells is relatively stable. Therefore, it is easy to achieve good performance of prediction accuracy for the co-located scenario. To expand the application scope of inter-frequency prediction, we think that non-colocated inter-frequency prediction should also be considered.

Figure 2.3-1 L3 cell-level RSRP difference(dB) between 4GHz and 2GHz

Observation 8: For Inter_F_C (inter-frequency inter-cell), the RSRP difference of the co-located cells is relatively stable.

Proposal 10: For frequency domain prediction, non-colocated neighbouring cells should also be considered.

  - FR2 to FR2 intra-frequency (temporal domain)

In the email discussion [1], it was agreed that for intra-frequency temporal domain case A, the ratio between OW and PW is at least limited to the value range {5,4,3,2,1,1/2,1/3,1/4,1/5}.

As the PW length is 400 ms and the sampling rate is 80 ms for FR2, the applicable value range of OW length is {80,160,240,320,400,800,1200,1600,2000} ms. And the corresponding ratio between OW and PW is {1/5, 2/5, 3/5, 4/5, 1, 2, 3, 4, 5}. Therefore, we propose:

Proposal 11: Update the ratio between OW and PW to the value range {1/5, 2/5, 3/5, 4/5, 1, 2, 3, 4, 5}.

The simulation results of FR2 intra-frequency for the 3 sub-use cases are illustrated in the following Table 2.4-1 to Table 2.4-3:

Table 2.4-1 Simulation results of sub-use case 1

| OW length (ms) | 80 | 160 | 240 | 320 | 400 | 800 | 1200 | 1600 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OW length / PW length | 1/5 | 2/5 | 3/5 | 4/5 | 1 | 2 | 3 | 4 | 5 |
| Prediction accuracy (60km/h) | 1.845 | 1.253 | 1.222 | 1.204 | 1.211 | 1.286 | 1.314 | 1.339 | 1.416 |
| Prediction accuracy (90km/h) | 2.344 | 1.736 | 1.632 | 1.662 | 1.647 | 1.721 | 1.769 | 1.842 | 1.852 |
| Prediction accuracy (120km/h) | 2.647 | 2.034 | 1.954 | 1.965 | 1.962 | 1.977 | 2.099 | 2.11 | 2.164 |

Table 2.4-2 Simulation results of sub-use case 2

| OW length (ms) | 80 | 160 | 240 | 320 | 400 | 800 | 1200 | 1600 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OW length / PW length | 1/5 | 2/5 | 3/5 | 4/5 | 1 | 2 | 3 | 4 | 5 |
| Prediction accuracy (60km/h) | 2.094 | 1.302 | 1.198 | 1.155 | 1.146 | 1.091 | 1.068 | 1.042 | 1.022 |
| Prediction accuracy (90km/h) | 2.562 | 1.753 | 1.591 | 1.538 | 1.531 | 1.441 | 1.441 | 1.456 | 1.434 |
| Prediction accuracy (120km/h) | 2.784 | 1.986 | 1.807 | 1.766 | 1.723 | 1.697 | 1.676 | 1.65 | 1.67 |

Table 2.4-3 Simulation results of sub-use case 3

| OW length (ms) | 80 | 160 | 240 | 320 | 400 | 800 | 1200 | 1600 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OW length / PW length | 1/5 | 2/5 | 3/5 | 4/5 | 1 | 2 | 3 | 4 | 5 |
| Prediction accuracy (60km/h) | 2.045 | 1.655 | 1.609 | 1.575 | 1.554 | 1.591 | 1.666 | 1.694 | 1.752 |
| Prediction accuracy (90km/h) | 2.541 | 2.041 | 1.899 | 1.88 | 1.873 | 1.87 | 2.002 | 2.034 | 2.069 |
| Prediction accuracy (120km/h) | 2.763 | 2.28 | 2.151 | 2.092 | 2.098 | 2.135 | 2.139 | 2.165 | 2.223 |

Similar to the time-domain simulation in section 2.2, under the same OW length, prediction accuracy decreases with an increase in UE speed. At the same time, within a certain range of OW length, the increase in OW length can improve prediction accuracy. However, when the OW length is further increased than the PW length, the improvement in accuracy is not significant and may even decrease.

Furthermore, for the same OW length (e.g., 400ms), the performance of sub-use case 2 is better than that of sub-use case 1 and sub-use case 3. Our understanding is that, for FR2, the beam-level RSRP is not stable as input.

Observation 9: For temporal domain prediction Case A, within a certain range of OW length, increasing OW length can improve prediction accuracy.

Observation 10: For temporal domain prediction Case A, as the UE speed increases, prediction accuracy decreases.

Observation 11: For temporal domain prediction Case A in FR2, the performance of sub-use case 2 is better than that of sub-use case 1 and sub-use case 3.

Proposal 12: For each sub-use case of temporal domain prediction Case A, capture the following aspects in the TR regarding their impact on prediction accuracy: UE speed and OW length.

For all the scenarios, RAN2 may discuss whether to down-select the sub-use cases based on simulation results.

Proposal 13: For all the scenarios, RAN2 to discuss whether to down-select the sub-use cases based on simulation results.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Methodology of simulation

Proposal 1: For measurement reduction Case B, sliding L1/L3 filtering is only applicable for sub-use case 1 when both input and output are L1 beam-level measurements without L1 filtering.

Proposal 2: For measurement reduction Case B, non-sliding L1/L3 filtering is applicable for all sub-use cases.

Proposal 3: For frequency domain prediction for measurement reduction, the input is a set of historical measurement instances on Frequency 1 and the output is the predicted value on Frequency 2 at the time of the last measurement instance on Frequency 1.

Proposal 4: For spatial domain prediction, RAN2 to discuss how to derive cell-level quality based on predicted beam qualities and measured beam qualities.

FR1 to FR1 intra-frequency (temporal domain)

Observation 1a: For temporal domain prediction Case B, if the model input is L1 samples (without filtering), the prediction accuracy is quite close to the actual RSRP difference.

Observation 1b: For temporal domain prediction Case B, if the model input is L1 filtered RSRP, increasing OW length can improve prediction accuracy.

Observation 2: For temporal domain prediction Case B, as the UE speed increases, prediction accuracy decreases.

Observation 3: For temporal domain prediction Case B, as the MRRT increases, prediction accuracy decreases.

Observation 4: For temporal domain prediction Case B, the performance of sub-use case 1 and sub-use case 3 is better than that of sub-use case 2.

Proposal 5: For temporal domain RRM prediction, the actual RSRP difference between the actual RSRP within the prediction window and the last RSRP measurement in the observation window should be provided to assess the prediction accuracy.

Proposal 6: The L1 sample (without filtering) is not expected to be the model input for RRM measurement prediction.

Proposal 7: RAN2 to discuss how to determine acceptable prediction accuracy when RRM measurement prediction is introduced for measurement reduction. The following two options can be considered:

- Option 1: Request RAN4 to evaluate the requirement for prediction accuracy.

- Option 2: RAN2 introduces system-level performance (e.g., short ToS rate and HOF rate) simulation and the performance degradation threshold can be provided by the operators.

Proposal 8: For each sub-use case of temporal domain prediction Case B, capture the following aspects in the TR regarding their impact on prediction accuracy: UE speed, MRRT, and OW length.

FR1 to FR1 inter-frequency (frequency domain)

Observation 5: For frequency domain prediction, increasing OW length can improve prediction accuracy.

Observation 6: For frequency domain prediction, as the UE speed increases, prediction accuracy decreases.

Observation 7: For frequency domain prediction within FR1, the prediction accuracy of prediction from low frequency to high frequency and prediction from high frequency to low frequency is almost the same.

Observation 8: For Inter_F_C (inter-frequency inter-cell), the RSRP difference of the co-located cells is relatively stable.

Proposal 9: For frequency domain prediction, capture the following aspects in the TR regarding their impact on prediction accuracy: UE speed and OW length.

Proposal 10: For frequency domain prediction, non-colocated neighbouring cells should also be considered.

FR2 to FR2 intra-frequency (temporal domain)

Observation 9: For temporal domain prediction Case A, within a certain range of OW length, increasing OW length can improve prediction accuracy.

Observation 10: For temporal domain prediction Case A, as the UE speed increases, prediction accuracy decreases.

Observation 11: For temporal domain prediction Case A in FR2, the performance of sub-use case 2 is better than that of sub-use case 1 and sub-use case 3.

Proposal 11: Update the ratio between OW and PW to the value range {1/5, 2/5, 3/5, 4/5, 1, 2, 3, 4, 5}.

Proposal 12: For each sub-use case of temporal domain prediction Case A, capture the following aspects in the TR regarding their impact on prediction accuracy: UE speed and OW length.

Proposal 13: For all the scenarios, RAN2 to discuss whether to down-select the sub-use cases based on simulation results.

- Reference

- R2-2407849	summary of [POST127][030][AI mobility] RRM simulation assumptions - Phase 1 (OPPO)

- R2-2407885	summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO)_phase 2