3GPP TSG-RAN WG2 Meeting #127bis	                                                                              R2-2407989

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on Conditional Intra-CU LTM

Agenda Item:    8.6.4

Document for:  Discussion and Decision

Introduction

According to the latest WID of NR mobility enhancements [1], it is confirmed to support the conditional Intra-CU LTM in non-DC scenario. This paper gives our initial consideration on the conditional intra-CU LTM. In detail, the following aspects are covered by our discussion,

- Conditional LTM Preparation phase

- Conditional LTM Early sync phase

- Conditional LTM Evaluation and execution phase

- Conditional LTM completion phase

- Stage-2 conditional LTM procedure

The detailed discussion is provided in Section 2, and followed by Section 3 that summarizes the contribution of this paper.

Discussion

- Conditional LTM Preparation phase

Upon the preparation phase, besides the candidate configuration used for LTM, the associated execution condition is needed. It seems straightforward to use the RRCReconfiguration to include the conditional LTM configuration.

Proposal 1: Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

Then the first issue is which events can be configured as execution condition for the evaluation of conditional LTM.

Generally speaking, we can take the event-triggered L1 measurement reporting for reference, as the purpose of which is also for cell switch.

For the event-triggered measurement reporting, it had been agreed that,

- Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Event LTM1 is not defined.

Take CHO as example, only condEventA3 and condEventA5 were agreed as execution condition, which takes both serving cell channel quality and candidate cell channel quality into consideration. We can follow the CHO, i.e., only take event LTM3 like and event LTM5 like as the execution condition for conditional LTM.

Proposal 2: Event LTM3-like and LTM5-like are used as the conditional LTM execution condition.

Different from CHO, the conditional LTM supports the subsequent conditional LTM, thus how to determine the execution condition for each candidate cell should be discussed. Considering the serving cell is changed with each conditional LTM execution, similar as the event triggering measurement report, it should be the Source cell to determine the event for initial evaluation of conditional LTM and the candidate cell to determine the event for subsequent evaluation of conditional LTM when the candidate cell becomes the serving cell. Considering the execution condition is based on L1 measurement results, and it is DU that has the acknowledge of the L1 measurement results of the candidate cells and serving cell, so it is reasonable to determine the execution condition by DU.

Proposal 3a: To support subsequent conditional LTM, the execution conditions provided to UE includes the execution conditions for initial conditional LTM and the execution conditions for subsequent conditional LTM.

Proposal 3b: Source cell provides the initial execution condition for each conditional LTM candidate for initial conditional LTM, and each candidate cell prepares the execution condition for each other conditional LTM candidate for subsequent conditional LTM when it becomes the serving cell.

Proposal 3c:  It is DU to generate the execution condition.

- Conditional LTM Early sync phase

The main benefit of the LTM is reduction on latency due to early sync to achieve RACH-less LTM, if RACH-less procedure is not supported in conditional LTM, the main benefit of LTM on latency reduction would be unable to be achieved, and the benefit on conditional LTM would be no more than the robustness for which support of legacy CHO is already enough. Hence in order to achieve both benefits of LTM (improvement on latency) and UE triggered handover (robustness), conditional RACH-less LTM should be supported.

Proposal 4: RACH-less Conditional LTM is supported.

One may think RACH based conditional LTM should be performed if TA is not available when conditional LTM is triggered, i.e., to follow the same principle as R18 LTM. However, the latency for RACH-based conditional LTM may longer than the NW based LTM, so whether RACH based conditional LTM is supported should be discussed.

Observation 1: The latency of RACH-based conditional LTM is longer than R18 NW-controlled LTM.

Proposal 5:RAN2 discuss whether to support RACH based conditional LTM.

Early UL sync

In Rel-18 LTM, UE can perform early TA acquisition via PDCCH ordered early RACH or via UE-based TA measurement.

For conditional LTM, we need to consider whether the R18 early UL sync mechanisms can be reused.

R18 UE based TA measurement

As Rel-18 UE based TA measurement is performed at UE side itself, so it is straightforward to reuse it directly for conditional LTM without any needs to enhancement. However, it is worth to note that Rel-18 UE based TA measurement can only be used in a limited case that the candidate cell has same DL timing as serving cell.

Proposal 6: R18 UE based TA measurement mechanism is reused for conditional LTM.

PDCCH ordered early TA acquisition

According to the WID, conditional LTM is supported only for intra-CU case. However, it is still possible that cells under the same CU may have different DL timing. Hence, supporting of only R18 UE based TA measurement is not sufficient for early TA acquisition in conditional LTM. PDCCH ordered early TA is needed in conditional LTM.

Observation 2: In conditional LTM, PDCCH ordered early TA is needed as UE based TA measurement can only be used if the candidate cell has same DL timing as serving cell).

Therefore, it is proposed that,

Proposal 7a: PDCCH ordered early TA acquisition is supported for conditional LTM.

For PDCCH ordered early TA, the R18 mechanism cannot be reused directly. In R18 LTM, NW selects the candidates based on LTM L1 measure results reported from UE and performs PDCCH ordered early TA on these candidates. Later TA value acquired via PDCCH ordered early RACH is sent to UE via cell switch command MAC CE upon the LTM execution. But for conditional LTM, LTM L1 measurement reporting may not be reported to NW from UE, and there is no cell switch command MAC CE from NW to UE and UE needs to know the TA value when UE decides to trigger the conditional LTM execution (i.e., the TA value needs to be sent to UE in advance).UE also needs to maintain the received TA values until the triggering of condition LTM execution.

So, generally there are following issue to address,

-	How NW can determine candidate cells on which to perform early TA acquisition

-	How to deliver the TA value from NW to UE before the triggering of conditional LTM

-	How to maintain the TA value at UE side before conditional LTM execution

Observation 3: To support PDCCH ordered early TA in conditional LTM, the following issues are to be addressed,

-	How NW can determine candidate cells on which to perform early TA acquisition

-	How to deliver the TA value from NW to UE before the conditional LTM execution

-	How to maintain the TA value at UE side before conditional LTM execution

Issue 1: How NW can determine candidate cells on which to perform early TA acquisition

There are some potential Solutions:

- Option 1: UE needs to be configured with legacy L3 measurement reporting or LTM L1 measurement reporting if conditional LTM is configured

In option 1, NW needs to configure L3 measurement reporting or LTM l1 measurement reporting if conditional LTM is configured to UE. Then R18 PDCCH ordered early TA mechanism can be reused.

- Option 2: UE requests NW to trigger  PDCCH ordered early TA on certain candidate cells

For option 2, UE can selects the candidate cells based on L1 measure results and then request NW to perform PDCCH order early TA on these candidates.

Proposal 7b: On how NW can determine candidate cells on which to perform early TA acquisition, down selection between the following options is needed,

- Option 1: UE needs to be configured with legacy L3 measurement reporting or LTM L1 measurement reporting if conditional LTM is configured

- Option 2: UE requests NW to trigger PDCCH ordered early TA on certain candidate cells

Issue 2: How to deliver the TA value from NW to UE before the triggering of conditional LTM

To deliver the TA value from NW to UE before the triggering of conditional LTM, only the candidate ID and TA value is needed, it seems obvious if to reuse the legacy MAC CE cell switch command, redundant information, e.g., TCI state index will be included. How to handle this redundant information and how to distinguish it used for conditional LTM or legacy LTM will cause complexity for spec. New MAC CE could be introduced to include the TA value used for conditional LTM.

Proposal 7c: For PDCCH ordered early TA in conditional LTM, a new MAC CE is introduced to deliver the TA value and the corresponding candidate configuration ID to UE before the triggering of conditional LTM execution.

Issue 3: How to maintain the TA value at UE side before conditional LTM execution

As indicated in issue 2, the TA value needs to be sent to UE before the triggering of conditional LTM, UE needs to maintain the TA value and ensures that the stored TA value is a valid one when to use it upon conditional LTM execution.

The potential Solutions are as follows,

-	Option 1: TAT-like timer is maintained at UE side

In option 1, a TAT-like timer is started when UE receives the TA value of a candidate cell. UE treat the TA value of the target cell as valid if the TAT is still running when conditional LTM is triggered by UE

-	Option 2: UE determines the TA value as valid if the L1 measurement result change is not above a threshold.

In option 2, a TA-specific threshold is configured to UE.UE stores the TA value when receiving it. If the L1 measurement result change is above a threshold, the UE consider the stored TA is invalid.

RAN2 needs to discuss and down selects between the potential options above.

Therefore, it is proposed that,

Proposal 7d: RAN2 discusses how to maintain the TA value at UE side before conditional LTM execution,

-	Option 1: TAT-like timer is maintained at UE side

-	Option 2: UE determines the TA value as valid if the L1 measurement result change is not above a threshold.

Early DL sync

In R18 LTM, the NW can do early TCI state activation to one candidate cell based on L1 measurement reporting. This allows the UE to be DL synchronized with this candidate cell. For conditional LTM, early DL sync needs to be supported as well but we need to consider whether it is feasible to reuse the R18 LTM early TCI state activation.

Proposal 8a: Early candidate TCI State activation/deactivation is supported for conditional LTM.

For conditional LTM, UE may be not configured to report the LTM L1 measurement results to NW, so it is not clear how NW can select the TCI state of one candidate cell to perform the early activation.

Potentially there are two options to address this issue.

Option 1: R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

For option 1, to select the suitable TCI state to activate, NW needs to have the L1 measure results of candidate beams/candidate cells. It can be a) based on L3 measurement results, or b) based on L1measurement result, e.g. NW is mandated to additionally configure R18 LTM L1 measurement or R19 event triggered L1 measurement to UE when configuring conditional LTM to UE.

Observation 4: If R18 candidate TCI State activation/deactivation design is reused in conditional LTM, NW cannot determine candidate cells on which to perform TCI State activation/deactivation if UE does not report candidate measure results.

Option 2: UE determines the TCI state to activate based on L1 measurement results. The detailed procedure is up to UE implementation.

For Option 2, it is done by UE itself based on the L1 measurement results at UE side, and NW is not involved at all. UE can determine when and to which TCI state of the candidate cell to perform the early activation/deactivation.

Proposal 8b: To perform early candidate TCI State activation/deactivation in conditional LTM, RAN2 down-selects between the following options,

- Option 1: R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

- Option 2: UE determines the TCI state to activate based on L1 measurement results without NW involvement. The detailed procedure is up to UE implementation.

- Conditional LTM Evaluation and execution phase

Conditional LTM Evaluation

The UE evaluate the execution condition for each candidate cell/beam, and when at least one of the candidate cell/beam fulfills the associated execution condition, the UE should trigger the LTM cell switch procedure.

Proposal 9: UE evaluates the execution condition for each candidate beam and triggers the LTM cell switch to the corresponding candidate cell when at least one beam of the candidate cell fulfills the associated execution condition.

In the event-triggered measurement reporting, it had been agreed that,

- MAC layer handles the event evaluation and measurement report triggering.

We prefer a unified solution for conditional LTM and event-triggered measurement reporting. And the Rel-18 LTM is also based on MAC CE to perform cell switch. Hence it is reasonable to let MAC entity to determine the cell switch execution.

Proposal 10: MAC layer handles the event evaluation and determines to trigger the cell switch for conditional LTM.

Beam selection for RACH-less conditional LTM

In the NW-based LTM, the UE is indicated with the TCI state in the LTM cell switch command MAC CE. For RACH-less LTM, the UE will use the indicated TCI state to receive the dynamic grant or to select the type-1 configured grant.

For conditional RACH-less LTM, how to determine the selected cell and beam is based on UE. As mentioned above, UE evaluates the execution condition for each candidate cell/beam, and determines to trigger the cell switch upon at least one of the candidate cell/beam fulfills the associated execution condition. Thus, the candidate cell/beam which fulfills the execution condition should be the target cell/beam. If more than one candidate cell/beam fulfills the execution condition, it is up to UE to select which one as the target cell/beam, similar as CHO.

Proposal 11: UE selects the beam which fulfils the associated execution condition as the target beam. If more than one beam fulfills the associated execution condition, it is up to UE implementation to select the target beam.

First UL transmission  Scheduling in RACH-less conditional LTM

Since the conditional LTM is triggered by UE and the target beam is selected by UE, the candidate cell is not aware of when and to which beam the UE will perform conditional LTM execution.

Observation 5: For conditional LTM, the candidate cell is not aware of when conditional LTM is executed and which beam is selected by UE.

Without such information, there will be issues at NW side. In Rel-18 intra-CU LTM, for RACH-less LTM, the source DU informs the target DU over F1 about the target beam information so that the target cell can perform dynamic grant according to the indicated beam or receive the configured grant associated with the indicated beam.

For conditional RACH-less LTM, if the NW does not know which candidate cell is selected as target cell and which beam of this candidate cell is selected by UE, the candidate cell cannot know when and on which beam to perform the dynamic scheduling for the first PUSCH transmission. One may think that each candidate cell can keep sending the dynamic scheduling information on each candidate beam or keep monitor the UL transmission among all CG occasions after the preparation phase.

But this way is very inefficient from resource perspective. We see one feasible way is that UE is configured with legacy L3 measurement reporting or LTM L1 measurement reporting if conditional LTM is configured. Then source cell can determine the potential target cells/beams and then request each potential target cell to start the UL scheduling on the potential target beams. But this option will cause unnecessary signaling overhead. Another way is based on UE assistance information. E.g. UE report the potential target cells/beams to source cell. Source cell then request each potential target cell to start the UL scheduling on the potential target beams.

Proposal 12: If conditional LTM is configured to UE, RAN2 down-selects between the following options to assist candidate cell to determine the target beam:

- Option 1: NW determines the potential candidate cells/beams based on the L3 RRM reporting or L1 LTM measurement reporting from UE

- Option 2: UE determines the potential candidate cells/beams and reports them to the source cell

With this, both CG-based and DG-based can be supported in RACH-less conditional LTM.

Therefore, it is proposed that

Proposal 13: For RACH-less conditional LTM, both CG-based and DG-based first UL transmission on target cell are supported.

- Conditional LTM completion phase

For conditional RACH-less LTM, upon the conditional LTM execution, the UE should send the RRCReconfigurationComplete as the first UL transmission to the target cell/beam to complete the procedure, like the legacy LTM procedure. And the UE considers the conditional LTM is successful when a PDCCH addressing the UE’s C-RNTI in the target cell schedules a new transmission for the same HARQ processes as the first UL transmission, defined in Rel-18 intra-CU LTM.

Proposal 14: For conditional RACH-less LTM, UE transmits RRCReconfigurationComplete as the first UL transmission. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

- Stage-2 conditional LTM procedure

Based on discussion in previous sections, the Stage-2 conditional LTM procedure can be illustrated as follows,

Figure 1: The stage-2 procedure for conditional LTM

Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: The latency of RACH-based conditional LTM is longer than R18 NW-controlled LTM.

Observation 2: In conditional LTM, PDCCH ordered early TA is needed as UE based TA measurement can only be used if the candidate cell has same DL timing as serving cell).

Observation 3: To support PDCCH ordered early TA in conditional LTM, the following issues are to be addressed,

-	How NW can determine candidate cells on which to perform early TA acquisition

-	How to deliver the TA value from NW to UE before the conditional LTM execution

-	How to maintain the TA value at UE side before conditional LTM execution

Observation 4: If R18 candidate TCI State activation/deactivation design is reused in conditional LTM, NW cannot determine candidate cells on which to perform TCI State activation/deactivation if UE does not report candidate measure results.

Observation 5: For conditional LTM, the candidate cell is not aware of when conditional LTM is executed and which beam is selected by UE.

Conditional LTM Preparation phase

Proposal 1: Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

Proposal 2: Event LTM3-like and LTM5-like are used as the conditional LTM execution condition.

Proposal 3a: To support subsequent conditional LTM, the execution conditions provided to UE includes the execution conditions for initial conditional LTM and the execution conditions for subsequent conditional LTM.

Proposal 3b: Source cell provides the initial execution condition for each conditional LTM candidate for initial conditional LTM, and each candidate cell prepares the execution condition for each other conditional LTM candidate for subsequent conditional LTM when it becomes the serving cell.

Proposal 3c:  It is DU to generate the execution condition.

Conditional LTM Early sync phase

Proposal 4: RACH-less Conditional LTM is supported.

Proposal 5:RAN2 discuss whether to support RACH based conditional LTM.

Proposal 6: R18 UE based TA measurement mechanism is reused for conditional LTM.

Proposal 7a: PDCCH ordered early TA acquisition is supported for conditional LTM.

Proposal 7b: On how NW can determine candidate cells on which to perform early TA acquisition, down selection between the following options is needed,

- Option 1: UE needs to be configured with legacy L3 measurement reporting or LTM L1 measurement reporting if conditional LTM is configured

- Option 2: UE requests NW to trigger PDCCH ordered early TA on certain candidate cells

Proposal 7c: For PDCCH ordered early TA in conditional LTM, a new MAC CE is introduced to deliver the TA value and the corresponding candidate configuration ID to UE before the triggering of conditional LTM execution.

Proposal 7d: RAN2 discusses how to maintain the TA value at UE side before conditional LTM execution,

-	Option 1: TAT-like timer is maintained at UE side

-	Option 2: UE determines the TA value as valid if the L1 measurement result change is not above a threshold.

Proposal 8a: Early candidate TCI State activation/deactivation is supported for conditional LTM.

Proposal 8b: To perform early candidate TCI State activation/deactivation in conditional LTM, RAN2 down-selects between the following options,

- Option 1: R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

- Option 2: UE determines the TCI state to activate based on L1 measurement results without NW involvement. The detailed procedure is up to UE implementation.

Conditional LTM Evaluation and execution phase

Proposal 9: UE evaluates the execution condition for each candidate beam and triggers the LTM cell switch to the corresponding candidate cell when at least one beam of the candidate cell fulfills the associated execution condition.

Proposal 10: MAC layer handles the event evaluation and determines to trigger the cell switch for conditional LTM.

Proposal 11: UE selects the beam which fulfils the associated execution condition as the target beam. If more than one beam fulfills the associated execution condition, it is up to UE implementation to select the target beam.

Proposal 12: If conditional LTM is configured to UE, RAN2 down-selects between the following options to assist candidate cell to determine the target beam:

- Option 1: NW determines the potential candidate cells/beams based on the L3 RRM reporting or L1 LTM measurement reporting from UE

- Option 2: UE determines the potential candidate cells/beams and reports them to the source cell

Proposal 13: For RACH-less conditional LTM, both CG-based and DG-based first UL transmission on target cell are supported.

Conditional LTM completion phase

Proposal 14: For conditional RACH-less LTM, UE transmits RRCReconfigurationComplete as the first UL transmission. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Reference

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4