3GPP TSG-RAN WG2 Meeting #127bis                                                               R2-2407991

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Consideration on on-demand SIB1 issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the details on case 2. The contribution is organized as the following. In section 2, we discuss the following aspects,

- WUS configuration transmission

- WUS configuration content

- The OD-SIB1 updating procedure on NES cell

- Msg.1 based RA procedure

- Legacy UE handling

- Cell selection for R19 NES UE

- Indication of the OD-SIB1 transmission status

Our proposals are summarized in Section 3.

Discussion

In this section, regarding the achieved agreements in [1], the further issues associated to RAN2 are listed and discussed for OD-SIB1.

  - WUS configuration transmission

Which SIB includes UL WUS configuration for OD-SIB1 in NES cell

For the updating of WUS configuration on NES cell, RAN2 has agreement in RAN2#127 meeting as follows,

WUS configuration of NES cell from NES cell: 4．Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

For NES cell, the UL WUS configuration for OD-SIB1 could be included in legacy SIB or a new SIB. The way of new SIB will have more impact to legacy SIB design, for example, a new SIBx and its scheduling information should be introduced, while a new SIB is usually used when the content in new SIB is applied to a dedicated feature and hoped to have less impact to other UE in Cell, such as SIB22 to ATG UE, however, this WUS configuration will be applied to all the camping UE in NES Cell. So, the way of new SIB is not suitable. Besides, the legacy on-demand SI request configuration are already included in SIB1, this UL WUS configuration is used to request the NES Cell’s OD-SIB1, so it is better to align the legacy rule and include it in SIB1.

Proposal 1: UE camping on R19 NES cell receives the NES Cell’s WUS configuration from SIB1.

If WUS configuration is included in SIB1, the legacy regular SI modification procedure will be applied if WUS configuration update is needed. UE will read the SIB1 and stored this SIB1, consequently, the updated WUS configuration will be stored. The updated WUS configuration will be used when needed later. This is also the legacy handling for other IEs in the SIB1.

Thus, it can be seen that UE does not need to know whether the WUS configuration in NES cell is changed or not, it will always use the WUS configuration in the stored SIB1, which could be updated based on legacy SI modification procedure.

Observation 1: UE does not need to be indicated explicitly whether the WUS configuration in NES cell has changed or not.

Proposal 2: UE always uses the WUS configuration in the latest received SIB1 in NES cell for the subsequent OD-SIB1 request, and there is no need for extra indication for WUS configuration change.

Which SIB includes neighbor NES cell’s UL WUS configuration in Cell A/NES Cell

For Cell A, based on the RAN2#126 agreement as below, two options as below are proposed:

Transmission of WUS configuration 5. Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

- Option 1: Configure on-demand SIB1 related configuration for neighbour NES cells in new SIB.

- Option 2: Configure on-demand SIB1 related configuration for neighbour NES cells in existing SIB.

Similarly to the analysis to NES Cell, the option 1 with a new SIB for neighbor Cell’s WUS configuration in Cell A is not necessary, which needs to define new SIB and the scheduling information, it will have more spec impact. In option 2, the neighbor cell’s WUS configuration in Cell A could be included in SIB3/4, which contains neighbor cell related information relevant only for intra-frequency/inter-frequency cell re-selection, as the WUS configuration updating is not so frequent, so the impact by WUS configuration to SIB 3/4 is acceptable.

Proposal 3: The WUS configuration for NES Cell is included in SIB3/4 in Cell A.

For NES Cell, some companies proposed that it could also include the neighbor NES cell’s WUS configuration. Although this scenario is not discussed in RAN2 so far, but if this scenario is supported, the neighbor NES cell’s WUS configuration could be included in NES Cell as the way of neighbor NES cell’s WUS configuration in Cell A, no extra design is needed.

Proposal 4: NES Cell can provide the neighbor NES cell’s UL WUS configuration in NES Cell, by reusing the design for transmitting neighbor NES cell’s WUS configuration in Cell A.

  - WUS configuration content

In RAN2#126 meeting, the following agreements are associated to WUS configuration

Agreements on on-demand SIB1: Further clarification - Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells: - More than one Cell A may provide configuration for the same NES cell. - The same Cell A may assist more than one NES Cells. Information of WUS configuration - Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell. - For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Obviously, the WUS configuration needs to include the RA preamble, the RA resource, the parameters used for RA procedure, such as the maximum number of Random Access Preamble transmission preambleTransMax, the parameters on power ramping, the RAR window parameter ra-ResponseWindow and so on. It has been agreed that WUS configuration is agreed to use on-demand SI request configuration in SIB1 as the baseline, referring to the SI-RequestConfig in annex, above information needed by WUS configuration are already included in SI-RequestConfig, hence, the WUS configuration will use the SI-RequestConfig as baseline.

Proposal 5: The WUS configuration reuses the parameters in legacy SI-RequestConfig.

Since Cell A may assist more than one NES Cells, the neighbor NES Cells have better not use the same WUS configuration to avoid the WUS transmission conflict. So, it can be seen that the WUS configuration format in Cell A could be a list including one or more WUS configuration, each WUS configuration is associated to one or more cells.

Proposal 6: The WUS configuration format in Cell A could be a list including one or more WUS configuration, each WUS configuration can be associated to one or more cells.

In legacy procedure, the UE will get UL carrier information by ServingCellConfigCommonSIB in SIB1 which include the DL and UL carrier information. For OD-SIB1 request procedure, the UE needs to transmit WUS signal on the NES Cell’s UL carrier, considering SI-RequestConfig does not include the UL carrier information and UE could not read NES Cell’s SIB1 before transmitting WUS, for Cell A, it is suggested to include the UL carrier information as well in the WUS configuration.

Proposal 7: The neighbor NES cell WUS configuration in cell A includes NES Cell’s UL carrier information.

  - The OD-SIB1 updating procedure on NES cell

In RAN2#126/127 meeting, the agreements on SIB1 updating is as follows:

RAN2#126 meeting agreement: Paging and SIB1 update in NES cell After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc. RAN2#127 meeting agreement: OD-SIB1 Validity: Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

Based on above agreement, RAN2 already agree to apply legacy SI modification procedure to update SIB1. But, it is not clear when the SIB1 will be transmitted in the regular SI modification procedure, based on the below procedure in TS 38.331, it can be seen that the SI will be transmitted in the second MP following the one where SI change indication is transmitted, so it needs to clarify that the SIB1 will be transmitted in the second MP when SIB1 update is perform by the network.

TS 38.331 A modification period is used, i.e. updated SI message (other than SI message for ETWS, CMAS, positioning assistance data, and some NTN-specific information as specified in the field descriptions ) is broadcasted in the modification period following the one where SI change indication is transmitted.

Proposal 8: The updated SIB1 will be broadcasted in the second modification period as legacy when SIB1 update is triggered by network, and there is no extra spec impact.

  - Msg.1 based RA procedure

RAN2#126 meeting agreements: RAR monitoring for MSG1 based OD-SIB1 REQ RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

After UE receives the RAR successfully, it could monitor the SIB1 message. It is under RAN1 discussion on how to determine the window starting time of the PDCCH monitoring occasions for on demand SIB1.

RAN1#118 meeting: Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

RAN2 can wait RAN1 further progress on this issue.

Proposal 9: RAN2 waits RAN1 conclusion on how to determine the window starting time of the PDCCH monitoring occasions for on demand SIB1.

RAN2#126 meeting agreements: UE behaviour on RACH failure of OD-SIB1 REQ As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE.

In RAN2#126 meeting, as baseline, RAN2 already agreed that, UE needs to monitor RAR as network feedback, and upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. In legacy on-demand SIB request procedure, it is UE implementation to request SIB again after the failed RA procedure. However, for on-demand SIB1 request procedure, the SIB1 information is important to UE, UE may be not able to work if SIB1 could not be achieved, so it is suggested to not leave the determination to UE.

Proposal 10: RAN2 confirms that UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred upon random access procedure failure of OD-SIB1 request.

Besides, because of UE mobility, it is also possible that UE may miss the OD-SIB1 even if the RAR for OD-SIB1 is received successfully. UE behavior for this case should also be defined. Generally, we think the UE needs to consider this NES Cell is barred and perform the cell-reselection immediately.

Proposal 11: R19 NES UE considers the NES Cell as barred if it is failed to acquire the OD-SIB1 finally after a successful RAR reception.

  - Legacy UE handling

RAN2#126 meeting agreements: NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism) How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that). RAN2#127 meeting agreements: Access restriction for legacy UEs: Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

According to above agreement, the legacy UE will not camp on the NES cell by barring mechanism. Three options as above are proposed.

First of all, it is necessary to clarify that both legacy non-NES UE and legacy NES UE are not allowed to access the OD-SIB1 NES cell. Similar as legacy non-NES UE, legacy NES UE also needs to face an abnormal situation where SIB1 of NES Cell disappears suddenly.

Proposal 12: Both legacy non-NES UE and legacy NES UE are not allowed to access the OD-SIB1 NES cell.

For option 2, RAN1 also discuss to introduce a new value for ssb-SubcarrierOffset to indicate OD-SIB1 is not applied or Cell is in NES status, this new value cannot be interpreted by legacy UE. Thus, it is FFS whether option 2 can be used to bar legacy UE for NES cell.

Observation 2: As using a new value for ssb-SubcarrierOffset is under RAN1 discussion, it is FFS to apply option 2 to bar legacy UE based on no SIB1 indication via ssb-SubcarrierOffset.

Compared with option 1 and option 2 which will make UE attempt to read MIB for SIB1, option 3 will configure NES Cell in the list of excluded cells, it could make legacy UE not perform measurement and not attempt to read MIB of NES Cell, which is beneficial to UE power saving. So, at least, option 3 is suggested to be considered for barring the legacy UE.

Proposal 13: Neighboring cells supporting OD-SIB1 is included in the list of excluded cells.

  - Cell selection for R19 NES UE

Based on current discussion, R19 NES UE is allowed to perform cell reselection and camp on NES Cell. However, R19 NES UE may power on and perform cell-selection at NES Cell which is transmitting the OD-SIB1 triggered by other UEs, the R19 NES UE behavior should be defined if it could camp on NES Cell. In our view, even it is a corner case and the UE cannot trigger the WUS, but the NES UE could read out the OD-SIB1 for other UEs, so the UE is able to camp on the NES Cell if allowed.

Proposal 14: R19 NES UE can camp on NES Cell upon cell selection if it can acquire OD-SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

  - Indication of the OD-SIB1 transmission status

In last meeting, some companies proposed that multiple UE may request OD-SIB1 in a common and short duration. The issue will be whether the UE needs to check the OD-SIB1 on/off before triggering the WUS to avoid the frequent transmission of OD-SIB at network side and unnecessary WUS transmission at UE side. In our view, the OD-SIB1 on/off indication is in RAN1 scope, and the related issue of occasions of OD-SIB1 is also under the RAN1 discussion, so it is better to wait RAN1 progress before we discuss this issue in RAN2.

Proposal 15: RAN2 waits the RAN1 conclusion on how to indicate the OD-SIB1 transmission status on NES cell.

Conclusion

According to the analysis in section 2, Our observations and proposals are summarized as follows,:

Observation 1: UE does not need to be indicated explicitly whether the WUS configuration in NES cell has changed or not.

Observation 2: As using a new value for ssb-SubcarrierOffset is under RAN1 discussion, it is FFS to apply option 2 to bar legacy UE based on no SIB1 indication via ssb-SubcarrierOffset.

WUS configuration transmission

Proposal 1: UE camping on R19 NES cell receives the NES Cell’s WUS configuration from SIB1.

Proposal 2: UE always uses the WUS configuration in the latest received SIB1 in NES cell for the subsequent OD-SIB1 request, and there is no need for extra indication for WUS configuration change.

Proposal 3: The WUS configuration for NES Cell is included in SIB3/4 in Cell A.

Proposal 4: NES Cell can provide the neighbor NES cell’s UL WUS configuration in NES Cell, by reusing the design for transmitting neighbor NES cell’s WUS configuration in Cell A.

WUS configuration content

Proposal 5: The WUS configuration reuses the parameters in legacy SI-RequestConfig.

Proposal 6: The WUS configuration format in Cell A could be a list including one or more WUS configuration, each WUS configuration can be associated to one or more cells.

Proposal 7: The neighbor NES cell WUS configuration in cell A includes NES Cell’s UL carrier information.

The OD-SIB1 updating procedure on NES cell

Proposal 8: The updated SIB1 will be broadcasted in the second modification period as legacy when SIB1 update is triggered by network, and there is no extra spec impact.

Msg.1 based RA procedure

Proposal 9: RAN2 waits RAN1 conclusion on how to determine the window starting time of the PDCCH monitoring occasions for on demand SIB1.

Proposal 10: RAN2 confirms that UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred upon random access procedure failure of OD-SIB1 request.

Proposal 11: R19 NES UE considers the NES Cell as barred if it is failed to acquire the OD-SIB1 finally after a successful RAR reception.

Legacy UE handling

Proposal 12: Both legacy non-NES UE and legacy NES UE are not allowed to access the OD-SIB1 NES cell.

Proposal 13: Neighboring cells supporting OD-SIB1 is included in the list of excluded cells.

Cell selection for R19 NES UE

Proposal 14: R19 NES UE can camp on NES Cell upon cell selection if it can acquire OD-SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

Indication of the OD-SIB1 transmission status

Proposal 15: RAN2 waits the RAN1 conclusion on how to indicate the OD-SIB1 transmission status on NES cell.

Reference

- R2-2407571, Report from session on V2X/SL, R18/19 MOB, and R19 NES, Samsung.

Annex

6.3.2	Radio resource control information elements

#### –	SI-RequestConfig

The IE SI-RequestConfig contains configuration for Msg1 based SI request without Msg1 repetition. For the random access parameters (e.g. prach-RootSequenceIndex, msg1-SubcarrierSpacing, etc.) not configured in SI-RequestConfig, the UE applies the parameters configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization (as specified in TS 38.321 [3]), of the initial uplink BWP unless otherwise specified.

SI-RequestConfig information element

-- ASN1START

-- TAG-SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SI-REQUESTCONFIG-STOP

-- ASN1STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occasions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization (as specified in TS 38.321 [3]), of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-Request configuration in number of association periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting. Otherwise: - If si-SchedulingInfo-v1700 is not present and the SI-RequestConfig is used for on-demand SI request in SI-SchedulingInfo or PosSI-SchedulingInfo, the 1st entry in the list corresponds to the first SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfig is configured in SI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which si-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfig is configured in PosSI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

SI-RequestResources field descriptions

ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SI request for SI message(s) corresponding to this SI-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.