3GPP TSG-RAN WG2 Meeting #127bis                                                                           R2-2407993

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	The identification of the power control parameters after LTM cell switch

Agenda Item:	7.4.2

Document for:	Discussion and Decision

1 Introduction

After RAN2#125bis meeting, a long email [1] was organized to discuss the issue on the identification of the power control parameters after LTM cell switch, based on the received LS [2] from RAN1. In RAN2#126 meeting, this issue was discussed and addressed, also the spec was modified accordingly. However, with the current modified spec, it is still unclear how to identify the power control parameters after LTM cell switch.

In this contribution, we propose the issue and give the corresponding solutions and TPs from our perspective.

2 Discussion

In RAN2#126 meeting, RAN2 discussed the issue on the identification of the power control parameters after LTM cell switch, based on the received LS [2] from RAN1, and achieved the following agreements.

RAN2 assumes that TCI configuration under LTM TCI info could be different to TCI configurations in candidate configuration.

RAN2 to adopt Approach 2, i.e., introduce new RRC parameters under LTM-TCI-Info.

Based on the agreements above, RAN2 adopted Approach 2, i.e. introduce new RRC parameters under LTM-TCI-Info. In the LS [2], RAN1 provides the following parameters for Approach 2.

If approach 2 is taken, the following parameters need to be newly added: it is noted that the final check/decision is up to RAN2. - Under Uplink-powerControl-r18 - Uplink-powerControlId-r18 ::= INTEGER(1.. maxNrofCandidateUL-TCI-r18) - p0AlphaSetforPUSCH-r18 P0AlphaSet-r17 OPTIONAL, -- Need R - p0AlphaSetforPUCCH-r18 P0AlphaSet-r17 OPTIONAL, -- Need R - p0AlphaSetforSRS-r18 P0AlphaSet-r17 OPTIONAL -- Need R - Under CandidateTCI-State-r18, - ul-powerControl-r18 Uplink-powerControlId-r18 OPTIONAL, -- Need R - Field description: Configures power control parameters for PUCCH, PUSCH and SRS of the LTM candidate that includes this CandidateTCI-State - Under CandidateTCI-UL-State-r18, - ul-powerControl-r18 Uplink-powerControlId-r18 OPTIONAL, -- Need R - Field description: Configures power control parameters for PUCCH, PUSCH and SRS of the LTM candidate that includes this CandidateTCI-UL-State - Under LTM-TCI-Info-r18, - uplink-PowerControlToAddModList-r18 SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControl-r18 OPTIONAL, -- Need N - uplink-PowerControlToReleaseList-r18 SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControlId-r18 OPTIONAL, -- Need N - Field description: Configures UL power control parameters for PUSCH, PUCCH and SRS when field unifiedTCI-StateType is configured for this serving cell.

However, in the current spec, only the parameters highlighted in yellow are included, i.e. only the ul-powerControl-r18 was introduced in CandidateTCI-State and CandidateTCI-UL-State, other parameters that highlighted in blue and green were not introduced. This is not aligning with RAN1’s LS. Due to the Uplink-powerControl has been defined in R17, so it doesn’t need to define new IE with the same parameters. So the parameter highlighted in blue is not needed. However whether the parameters highlighted by green are needed should be discussed. Also, with the current spec, it is still unclear how to identify the power control parameters after LTM cell switch, it doesn’t specify which uplink-PowerControlToAddModList to be referred to, e.g. refer to the uplink-PowerControlToAddModList configured in the serving cell or the uplink-PowerControlToAddModList configured in the candidate cell.

In order to address the issue, following two solutions can be considered.

Solution 1: clarify in the field description of ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State that it refers to the uplink-PowerControlToAddModList configured in the SpCell of the candidate configuration;

Solution 2: introduce uplink-PowerControlToAddModList-r18 and uplink-PowerControlToReleaseList-r18 in LTM-TCI-Info to align RAN1 LS, the ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State refer to the power control parameters in LTM-TCI-Info;

If we go with Solution 1, it seems to be simpler for spec modification compared with Solution 2, but it is not align with RAN1’s LS, RAN2 needs to consider whether the final signalling design needs to be informed to RAN1 as the signalling design is different from the approach 2 provided in the RAN1 LS. We provide a TP for Solution 1 in Annex 1.

If we go with Solution 2, it is align with RAN1’s LS, but it introduces the NBC change as it affects the ASN.1. We provide a TP for Solution 2 in Annex 2.

We suggest RAN2 to discuss the two solutions and give the final decision.

Proposal 1: On how to identify the power control parameters after LTM cell switch, RAN2 down selects between the following two solutions:

- Solution 1: clarify in the field description of ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State that it refers to the uplink-PowerControlToAddModList configured in the SpCell of the candidate configuration;

- Solution 2: introduce uplink-PowerControlToAddModList-r18 and uplink-PowerControlToReleaseList-r18 in LTM-TCI-Info to align RAN1 LS, the ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State refer to the power control parameters in LTM-TCI-Info;

TPs for Solution 1 and Solution 2 are provided in Annex 1 and Annex 2 respectively.

3 Conclusion

Based on the previous analysis in section 2, our proposals are summarized as follows:

Proposal 1: On how to identify the power control parameters after LTM cell switch, RAN2 down selects between the following two solutions:

- Solution 1: clarify in the field description of ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State that it refers to the uplink-PowerControlToAddModList configured in the SpCell of the candidate configuration;

- Solution 2: introduce uplink-PowerControlToAddModList-r18 and uplink-PowerControlToReleaseList-r18 in LTM-TCI-Info to align RAN1 LS, the ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State refer to the power control parameters in LTM-TCI-Info;

TPs for Solution 1 and Solution 2 are provided in Annex 1 and Annex 2 respectively.

4 Reference

[1] R2-2404619	Summary of [Post125bis][519][R18 Mob] Power Control Parameters after LTM cell switch (Fujitsu)	Fujitsu	discussion	Rel-18	NR_Mob_enh2-Core	Late

[2] R2-2404115 LS on the identification of the power control parameters after LTM cell switch

[3] 38331-i30 Radio Resource Control (RRC) protocol specification

# Annex 1		Text Proposal for TS 38.331

The text proposal in this section is written against the latest TS38.331 spec [3].

First of change

6.3.2	Radio resource control information elements

–	CandidateTCI-State

The IE CandidateTCI-State defines a TCI states configuration which associate one or more reference signal with a corresponding quasi-colocation (QCL) type.

CandidateTCI-State information element

-- ASN1START

-- TAG-CANDIDATETCI-STATE-START

CandidateTCI-State-r18 ::=           SEQUENCE {

tci-StateId-r18                      TCI-StateId,

qcl-Type1-r18                        LTM-QCL-Info-r18,

qcl-Type2-r18                        LTM-QCL-Info-r18                                                    OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r18           PathlossReferenceRS-Id-r17                                          OPTIONAL,   -- Cond Joint

tag-Id-ptr-r18                       ENUMERATED {n0,n1}                                                  OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                  Uplink-powerControlId-r17                                           OPTIONAL,   -- Cond Joint2

...

}

LTM-QCL-Info-r18 ::=                 SEQUENCE {

referenceSignal-r18                  CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

qcl-Type-r18                         ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-CANDIDATETCI-STATE-STOP

-- ASN1STOP

CandidateTCI-State field descriptions

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the candidate that includes this CandidateTCI-State and it refers to one of the PathlossReferenceRS configured within LTM-TCI-Info. In this version of the specification only SSB can be included as reference signal when PathlossReferenceRS is included within a CandidateTCI-State IE.

qcl-Type1, qcl-Type2 QCL information for the TCI state.

tci-StateId The ID number of the TCI state.

ul-PowerControl Indicates the UL power control parameters for PUSCH, PUCCH, and SRS of the candidate that includes this CandidateTCI-State. The field is present only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of the SpCellConfig in ltm-CandidateConfig. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the spCellConfig of the ltm-CandidateConfig in the LTM-Candidate where the ltm-TCI-Info is configured.

LTM-QCL-Info field descriptions

referenceSignal The field csi-RS-Index refers to one of the NZP-CSI-RS-Resource configured within LTM-TCI-Info.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfig and unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, Need R, otherwise. |
| Joint | This field is mandatory present, if unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, Need R, otherwise. |
| Joint2 | This field is optionally present, Need R, if unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, otherwise. |

–	CandidateTCI-UL-State

The IE CandidateTCI-UL-State defines an uplink TCI states configuration.

CandidateTCI-UL-State information element

-- ASN1START

-- TAG-CANDIDATETCI-UL-STATE-START

CandidateTCI-UL-State-r18 ::=           SEQUENCE {

tci-UL-StateId-r18                      TCI-UL-StateId-r17,

referenceSignal-r18                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

},

pathlossReferenceRS-Id-r18              PathlossReferenceRS-Id-r17                                   OPTIONAL,   -- Need R

tag-Id-ptr-r18                          ENUMERATED {n0,n1}                                           OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                     Uplink-powerControlId-r17                                    OPTIONAL,   -- Need R

...

}

-- TAG-CANDIDATETCI-UL-STATE-STOP

-- ASN1STOP

CandidateTCI-UL-State field descriptions

csi-RS-Index Indicates an NZP-CSI-RS-Resource of the LTM candidate that includes this CandidateTCI-UL-State.

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the LTM candidate that includes this CandidateTCI-UL-State and it refers to one of the PathlossReferenceRS configured within LTM-TCI-Info. In this version of the specification, only SSB can be included as reference signal when PathlossReferenceRS is included within a CandidateTCI-UL-State IE.

referenceSignal Reference signal with which spatial relation information is provided. The field csi-RS-Index refers to one of the NZP-CSI-RS-Resource configured within LTM-TCI-Info.

ssb-Index The index of a SSB/PBCH block as indicated in ltm-SSB-Config of the LTM candidate that includes this CandidateTCI-UL-State.

tci-UL-StateID The ID number of the uplink TCI state.

ul-PowerControl Indicates the UL power control parameters for PUSCH, PUCCH, and SRS of the candidate that includes this CandidateTCI-UL-State. The field is present only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of the SpCellConfig in ltm-CandidateConfig. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the spCellConfig of the ltm-CandidateConfig in the LTM-Candidate where the ltm-TCI-Info is configured.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfig. It is absent, Need R, otherwise. |

End of change

# Annex 2	Text Proposal for TS 38.331

The text proposal in this section is written against the latest TS38.331 spec [3].

First of change

6.3.2	Radio resource control information elements

–	LTM-TCI-Info

The IE LTM-TCI-Info is used to configure TCI related information for an LTM candidate configuration to be used during activation of TCI state(s) and/or upon the reception of the LTM Cell Switch procedure.

LTM-TCI-Info information element

-- ASN1START

-- TAG-LTM-TCI-INFO-START

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                          OPTIONAL,    -- Need R

...,

[[

uplink-PowerControlToAddModList-r18				SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControl-r17      OPTIONAL,   -- Need N

uplink-PowerControlToReleaseList-r18			SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControlId-r17    OPTIONAL   -- Need N

]]

}

-- TAG-LTM-TCI-INFO-STOP

-- ASN1STOP

LTM-TCI-Info field descriptions

unifiedTCI-StateType Indicates the unified TCI states type the UE is configured for this LTM candidate configuration. The value separate means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StateToAddModList for UL TCI states. The value joint means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation. The network always configures this field.

uplink-PowerControlToAddModList Configures UL power control parameters for PUSCH, PUCCH and SRS for the LTM candidate cell.

–	CandidateTCI-State

The IE CandidateTCI-State defines a TCI states configuration which associate one or more reference signal with a corresponding quasi-colocation (QCL) type.

CandidateTCI-State information element

-- ASN1START

-- TAG-CANDIDATETCI-STATE-START

CandidateTCI-State-r18 ::=           SEQUENCE {

tci-StateId-r18                      TCI-StateId,

qcl-Type1-r18                        LTM-QCL-Info-r18,

qcl-Type2-r18                        LTM-QCL-Info-r18                                                    OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r18           PathlossReferenceRS-Id-r17                                          OPTIONAL,   -- Cond Joint

tag-Id-ptr-r18                       ENUMERATED {n0,n1}                                                  OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                  Uplink-powerControlId-r17                                           OPTIONAL,   -- Cond Joint2

...

}

LTM-QCL-Info-r18 ::=                 SEQUENCE {

referenceSignal-r18                  CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

qcl-Type-r18                         ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-CANDIDATETCI-STATE-STOP

-- ASN1STOP

CandidateTCI-State field descriptions

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the candidate that includes this CandidateTCI-State and it refers to one of the PathlossReferenceRS configured within LTM-TCI-Info. In this version of the specification only SSB can be included as reference signal when PathlossReferenceRS is included within a CandidateTCI-State IE.

qcl-Type1, qcl-Type2 QCL information for the TCI state.

tci-StateId The ID number of the TCI state.

ul-PowerControl Indicates the UL power control parameters for PUSCH, PUCCH, and SRS of the candidate that includes this CandidateTCI-State. The field is present only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of the SpCellConfig in ltm-CandidateConfig. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the ltm-TCI-Info where the ltm-DL-OrJointTCI-StateToAddModList is configured.

LTM-QCL-Info field descriptions

referenceSignal The field csi-RS-Index refers to one of the NZP-CSI-RS-Resource configured within LTM-TCI-Info.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfig and unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, Need R, otherwise. |
| Joint | This field is mandatory present, if unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, Need R, otherwise. |
| Joint2 | This field is optionally present, Need R, if unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, otherwise. |

–	CandidateTCI-UL-State

The IE CandidateTCI-UL-State defines an uplink TCI states configuration.

CandidateTCI-UL-State information element

-- ASN1START

-- TAG-CANDIDATETCI-UL-STATE-START

CandidateTCI-UL-State-r18 ::=           SEQUENCE {

tci-UL-StateId-r18                      TCI-UL-StateId-r17,

referenceSignal-r18                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

},

pathlossReferenceRS-Id-r18              PathlossReferenceRS-Id-r17                                   OPTIONAL,   -- Need R

tag-Id-ptr-r18                          ENUMERATED {n0,n1}                                           OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                     Uplink-powerControlId-r17                                    OPTIONAL,   -- Need R

...

}

-- TAG-CANDIDATETCI-UL-STATE-STOP

-- ASN1STOP

CandidateTCI-UL-State field descriptions

csi-RS-Index Indicates an NZP-CSI-RS-Resource of the LTM candidate that includes this CandidateTCI-UL-State.

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the LTM candidate that includes this CandidateTCI-UL-State and it refers to one of the PathlossReferenceRS configured within LTM-TCI-Info. In this version of the specification, only SSB can be included as reference signal when PathlossReferenceRS is included within a CandidateTCI-UL-State IE.

referenceSignal Reference signal with which spatial relation information is provided. The field csi-RS-Index refers to one of the NZP-CSI-RS-Resource configured within LTM-TCI-Info.

ssb-Index The index of a SSB/PBCH block as indicated in ltm-SSB-Config of the LTM candidate that includes this CandidateTCI-UL-State.

tci-UL-StateID The ID number of the uplink TCI state.

ul-PowerControl Indicates the UL power control parameters for PUSCH, PUCCH, and SRS of the candidate that includes this CandidateTCI-UL-State. The field is present only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of the SpCellConfig in ltm-CandidateConfig. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the ltm-TCI-Info where the ltm-UL-TCI-StateToAddModList is configured.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfig. It is absent, Need R, otherwise. |

End of change