3GPP TSG-RAN WG2 Meeting #127bis                                                   R2-2407998

Hefei, China, 14th – 18th Oct, 2024

Source:	CATT

Title:	Enabling TX/RX for XR during RRM measurements

Agenda Item:	8.7.3

Document for:	Discussion and Decision

Introduction

For Rel-19 XR, one objective is expected to enabling TX/RX during RRM measurement as follows [1]:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In the incoming RAN2#127bis meeting, the discussion on this topic will be kicked off. In the current contribution, we give our considerations on it.

Discussion

Network signaling for enabling TX/RX during RRM measurements

For network signaling to enable TX/RX during RRM measurement, RAN1 discusses several options, including Alt1 dynamic indication and Alt3 Semi-static solution. In RAN1#118 meeting, RAN1 made the following working assumption and sent an LS [2] to RAN4 to inform them of the working assumption and ask them if there is any issue with it.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

According to the RAN1 progress, dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements is a working assumption. With dynamic indication solution (i.e. explicit indication by DCI to skip a particular gap/restriction), RAN2 impacts may include:

- RRC: At least new UE capability should be introduced.

- MAC: The behavior of MAC entity during an activated measurement gap is captured in TS 38.321 [3] as follows. When explicit indication by DCI to skip a particular gap is received, handling of measurement gaps in MAC entity is not valid for the skipped gap. MAC specification needs to be updated with dynamic indication solution, e.g. the MAC entity doesn’t perform the handling of measurement gaps if an explicit indication by DCI to skip a particular gap is received.

- 5.14 Handling of measurement gaps During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Overall, impacts on RAN2 of dynamic indication solution are small. Considering dynamic indication solution is still under discussion, RAN2 can wait for RAN1’s further conclusions.

Proposal 1: With dynamic indication solution for network signaling to enable TX/RX during RRM measurement, RAN2 impacts include:

- RRC: At least new UE capability should be introduced.

- MAC: The handling of measurement gaps needs to be updated considering the explicit indication by DCI to skip a particular gap.

Proposal 1bis: RAN2 waits for RAN1 further conclusions on dynamic indication solution to skip a particular gap/restriction.

UE assistant information

In RAN2#117 meeting, RAN1 discussed UE assistance information for enabling TX/RX during RRM measurements. And the following agreement was made:

Agreement RAN1 agrees to send an LS to RAN4 (CC: RAN2) to convey the following information about UE assistance information (including the conclusion on UAI): 1. Overall Description: RAN1 discussed UE assistance information related to measurements occasions and related to channel conditions, traffic, UE mobility. The following agreement and conclusions were agreed: Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable. 2. Actions: To RAN4: ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.. Final LS is approved in R1-2405736.

Considering UE assistance information related to measurements occasions may impact on RRM performance, RAN1 kindly asked RAN4 to decide whether or not to introduce any UE assistance information related measurement occasions [4] . In RAN4#112 meeting, RAN4 started the discussion on the objective and no agreement was made on UE assistance information related to measurements occasions. RAN2 can wait for RAN4’s conclusions on UE assistance information related to measurements occasions.

Proposal 2: RAN2 waits for RAN4 conclusions on UE assistance information related to measurements occasions before further discussion.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: With dynamic indication solution for network signaling to enable TX/RX during RRM measurement, RAN2 impacts include:

- RRC: At least new UE capability should be introduced.

- MAC: The handling of measurement gaps needs to be updated considering the explicit indication by DCI to skip a particular gap.

Proposal 1bis: RAN2 waits for RAN1 further conclusions on dynamic indication solution to skip a particular gap/restriction.

Proposal 2: RAN2 waits for RAN4 conclusions on UE assistance information related to measurements occasions before further discussion.

Reference

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1#118

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

- R1-2405736, LS on UE assistance information, RAN1#117