3GPP TSG-RAN WG2 Meeting #127bis                                                               R2-2407999

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Consideration on LCP enhancement

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the issues on LCP enhancement including LCP additional priority configuration and when/how to apply the additional priority, our views are also given in this paper.

Discussion

One aspect to prioritize the transmission of delay-critical data is to adjust the priority of the data with low remaining time, so that delay-critical data can be transmitted with higher priority.

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to within the LCH or for the whole LCH LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In RAN2#127 meeting, it is agreed that RAN2 no longer consider the enhancement of the LCP restriction as one of the candidate solutions for LCP enhancements in Rel-19 XR. For LCP priority, based on above agreements in RAN2#126 meeting[1], it has agreed that the priority of LCH will be adjusted based on configuration and applied in the LCP procedure, more details are discussed here:

How to configure the additional priority?

Generally, there are two options to configure the additional priority:

- Option 1: Pre-configuration.

- Option 2: Configured by RRC signalling.

For Option 1, the priority to LCH with delay-critical data could be preconfigured as a fix value, such as the highest priority. This option is not flexible. Furthermore, in some cases, it is not reasonable. For example, if there is important MAC CE or other data which is more important than the delay-critical data, it is not reasonable to always prioritize the delay-critical data. Hence, it is preferable to let it configurable as in Option 2. The network can configure the additional priority for the LCH based on its implementation.

Proposal 1: The additional logical channel priority for specific delay-critical data could be configured by network via dedicated RRC signaling.

When to apply the additional priority for the LCH?

With the introduction of the additional priority, there will be two priority values for one LCH, it should further discuss when to apply the legacy priority and when to apply the additional priority.

Obviously, when there is delay-critical UL data, the additional priority should be applied for the LCH; otherwise, the legacy priority should be applied.  Hence, the key issue becomes how to judge there is delay-critical UL data for an LCH. In Rel-18 XR, UL delay-critical PDCP/RLC data volumes were introduced in TS38.323 and TS38.322. It had better follow it in Rel-19 to define UL delay-critical data. That is if the delay-critical PDCP/RLC data volumes is not empty, the additional priority can be used for the LCH.

Proposal 2：If the amount of the delay-critical PDCP/RLC data volumes is not zero, the LCH should apply the additional priority.

Whether the priority of LCH can be changed during one LCP procedure?

According to legacy procedure as below, the logical channels selected for the UL grant are allocated resources in a decreasing priority order with Bj >0 (step 1), and Bj will be updated (step 2), then logical channels selected are allocated remaining resources in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first (step 3).

5.4.3.1.3 Allocation of resources Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover. The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

If two priorities are configured for one LCH, a new issue rasied. That is, if the resource allocated considering PBR in step 1 can accommodate all the UL delay-critical data, when performs resource allocation in step 3, whether the legacy priority or additional priority can be used since there is no delay-cirtical data. RAN2 needs to discuss whether the LCH priority will fallback to the legacy value to avoid allocating remaining resource to logical channel with an incorrect high priority.

Proposal 3: RAN2 to discuss whether the priority fallback after the resource allocation step considering PBR can happen or not in case that all the UL delay critical data are handled by the resource allocation step considering PBR.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The additional logical channel priority for specific delay-critical data could be configured by network via dedicated RRC signaling.

Proposal 2：If the amount of the delay-critical PDCP/RLC data volumes is not zero, the LCH should apply the additional priority.

Proposal 3: RAN2 to discuss whether the priority fallback after the resource allocation step considering PBR can happen or not in case that all the UL delay critical data are handled by the resource allocation step considering PBR.

Reference

- RAN2#126 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei