3GPP TSG-RAN WG2 Meeting #127bis                                                   R2-2408002

Hefei, China, 14th – 18th Oct, 2024

Source:	CATT

Title:	Discussion on XR rate control

Agenda Item:	8.7.6

Document for:	Discussion and Decision

Introduction

In RAN#105 meeting, the WID on eXtended Reality for NR Phase3 was revised and approved in [1]. Compared to the previous version, one objective for specifying uplink congestion signalling is added:

-	Specify uplink congestion signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

-	Specify Core requirements related to the above objectives as necessary [RAN4].

To facilitate further discussion, we excerpt the illustration figure from our latest co-source contribution in RAN#105 meeting[2] as below:

Figure 1: Procedure for uplink congestion signalling overview

The basic procedure for uplink congestion signalling will be described combining with the above figure: firstly, the gNB aware the UL congestion is happening through the uplink Uu interface. Secondly, the gNB decides to send an indication through MAC layer XR rate control signalling over downlink to UE. Last, after UE receiving the above-mentioned indication, UE will use it to enable faster source rate adaption to alleviate uplink congestion. In the current contribution, we further divide the above procedure into the below issues:

- Is there any enhancement needed for Uu uplink interface to let the gNB aware the UL congestion?

- The type of indication for the MAC layer XR rate control signalling over downlink.

- The congestion granularity of the UL congestion signalling.

Discussion

Is there any enhancement needed for Uu uplink interface to let the gNB aware the UL congestion

This issue had been discussed in Rel-18 XR[3], and concluded as below:

1.It is feasible for RAN to estimate the congestion information (e.g. traffic latency) per-QoS flow and per-DRB in downlink and uplink directions. 2.It is feasible for RAN to estimate the congestion information (e.g. traffic latency) per-QoS flow and per-DRB in uplink without UE impacts.

With the above green marked part, RAN2 can depend on legacy method to let the gNB aware the UL congestion is happening without any UE impacts no matter which granularity (per-QoS flow or per-DRB) is selected for the congestion awareness. Besides in the current WID, it is also limited to specify in MAC layer XR rate control signalling over downlink. Hence, it is obvious that there is no UE specification impact for letting the gNB aware the UL congestion is happening. In other words, the red arrow in Figure 1 will not introduce any UE specification impacts.

Observation 1: There is no UE specification impact for letting the gNB aware the UL congestion is happening.

The type of indication for the MAC layer XR rate control signalling over downlink

For the second step in Figure 1, the gNB decides to send an indication through MAC layer XR rate control signalling over downlink to UE.  What type of indication for the MAC layer XR rate control signalling over downlink is discussed in the current session. During the RAN plenary discussion, there are three major types:

- Qualitative indication shows whether UL congestion is happened or not.

- Quantitative indication shows the degree about the UL congestion.

- Quantitative indication shows the recommendation bit rate due to UL congestion is happening.

For the first option, the UE just only knows that whether the UL congestion is happened or not, there is no more information can be acquired. But this option has the least UL capacity consuming. For the second option, more rich information of the degree about the UL congestion can be further acquired which is better to guide the UE to handle the UL congestion situation. Since XR applications are latency sensitive. RAN congestion information, is one of the key RAN awareness information that would benefit XR application rate control performance and improve its QoS and user experience. More discussion for how to define the degree about the UL congestion is needed which means more specification impact is foreseen. The UL capacity consuming for the second option is more than the first option. For the third option, it can directly feedback the recommendation bit rate due to UL congestion happens, since the gNB has the whole information about the UL congestion, and this direct information has more guidance for UE how to handle the UL congestion situation. This mechanism is the same to legacy recommendation bit rate MAC CE which is an LTE Rel- 14 MAC CE that can be used to increase or decrease the UE's used voice codec rate. All options are technical feasible. Considering the final target is to enable faster source rate adaption to uplink congestion, directly feedback using the recommendation bit rate has the benefit in below two aspects: more direct effect and clear specification impact. Hence, we prefer to choose option 3 to pushforward.

Proposal 1: RAN2 agrees to directly feedback the recommendation bit rate due to UL congestion for XR in MAC layer XR rate control signalling over downlink to enable faster source rate adaption to uplink congestion.

The congestion granularity of the UL congestion signalling

In the current WID, the granularity of the congestion indication has two options: one is per DRB and the other is per QoS flow. Currently the recommendation bit rate MAC CE has already supported to indicate the recommended bit rate per DRB. Though the question turns into that whether to further support recommendation bit rate MAC CE throughout on a per QoS flow basis. As what we had already discussed in the first session, it is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in uplink without UE impacts. There is no technical concern for gNB to feedback both granularities (per-QoS flow and per-DRB).

To better pushforward this issue, the potential enhancement for recommendation bit rate MAC CE is discussed. The Figure 6.1.3.20-1 in 38.321[4] shows the format of recommended bit rate MAC CE. It has a fixed size and consists of two octets.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

The related fields are:

1) “LCID” field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits.

2) “UL/DL” field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink.

3) “Bit Rate” field indicates an index to Table 6.1.3.20-1 in 38.321[4]. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate.

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field

| Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] |
| --- | --- | --- | --- |
| 0 | Note 1 | 32 | 700 |
| 1 | 0 | 33 | 800 |
| 2 | 9 | 34 | 900 |
| 3 | 11 | 35 | 1000 |
| 4 | 13 | 36 | 1100 |
| 5 | 17 | 37 | 1200 |
| 6 | 21 | 38 | 1300 |
| 7 | 25 | 39 | 1400 |
| 8 | 29 | 40 | 1500 |
| 9 | 32 | 41 | 1750 |
| 10 | 36 | 42 | 2000 |
| 11 | 40 | 43 | 2250 |
| 12 | 48 | 44 | 2500 |
| 13 | 56 | 45 | 2750 |
| 14 | 72 | 46 | 3000 |
| 15 | 88 | 47 | 3500 |
| 16 | 104 | 48 | 4000 |
| 17 | 120 | 49 | 4500 |
| 18 | 140 | 50 | 5000 |
| 19 | 160 | 51 | 5500 |
| 20 | 180 | 52 | 6000 |
| 21 | 200 | 53 | 6500 |
| 22 | 220 | 54 | 7000 |
| 23 | 240 | 55 | 7500 |
| 24 | 260 | 56 | 8000 |
| 25 | 280 | 57 | Reserved |
| 26 | 300 | 58 | Reserved |
| 27 | 350 | 59 | Reserved |
| 28 | 400 | 60 | Reserved |
| 29 | 450 | 61 | Reserved |
| 30 | 500 | 62 | Reserved |
| 31 | 600 | 63 | Reserved |

4) “X” is Bit rate multiplier, For UEs supporting recommended bit rate multiplier, when bitRateMultiplier is configured for the logical channel indicated by LCID field, X field set to 1 indicates the actual value of bit rate is the value corresponding to the index indicated by the Bit Rate field multiplied by bitRateMultiplier as specified in TS 38.331.

5) “R” is reserved bit. There are two R bits in the current format which are all set to 0.

To satisfy XR applications, which field above needs to be enhancement is analysis as below:

- To support XR’s higher data rate

Recommended bit rate MAC CE covers only data rates up to 8000 kb/s. Since it mainly targets VoIP traffic. During the discussion in Rel-18 XR, it is common believe that XR bit rates is ranging from a few Mbit/s to 200 Mbit/s (corresponding to frame rates ranging from 30fps to 120fps and number of SDU per frame from a dozen to a few hundreds)[5]. It was raised that the upper bound for the recommended bit rate MAC CE should hence be adapted to cover the higher range for the XR applications. However, the current “X” filed can already supported it.  The bitRateMultiplier in the current 38.331 specification has the below value:

Value x40 indicates bit rate multiplier 40, value x70 indicates bit rate multiplier 70 and so on. Then the upper bound for the recommended bit rate MAC CE can reach 1600Mbps(8Mbps×200) which can satisfy XR’s requirement. Hence the current scope of bit rate can be directly used by XR service.

Observation 2: In the current recommended bit rate MAC CE, the upper bound of the bit rate can reach 1600Mbps, which can satisfy XR’s requirement.

- To support per QoS flow congestion granularity

In case of XR, different QoS flows may be mapped to the same radio bearer, to better alleviate the upper layer congestion it may be better to extend this mechanism to indicate on a per QoS flow basis.

All in all, to cover XR applications in Rel-19, the below potential way-forwards are listed:

- Reuse the current recommended bit rate MAC CE.

- Enhance the recommended bit rate MAC CE to support per QoS flow granularity indication.

Proposal 2: RAN2 down-selects from the below listed way-forwards for covering XR applications in Rel-19:

- Option 1: Reuse the current recommended bit rate MAC CE.

- Option 2: Enhance the recommended bit rate MAC CE to support per QoS flow granularity indication.

Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: There is no UE specification impact for letting the gNB aware the UL congestion is happening.

Proposal 1: RAN2 agrees to directly feedback the recommendation bit rate due to UL congestion for XR in MAC layer XR rate control signalling over downlink to enable faster source rate adaption to uplink congestion.

Observation 2: In the current recommended bit rate MAC CE, the upper bound of the bit rate can reach 1600Mbps, which can satisfy XR’s requirement.

Proposal 2: RAN2 down-selects from the below listed way-forwards for covering XR applications in Rel-19:

- Option 1: Reuse the current recommended bit rate MAC CE.

- Option 2: Enhance the recommended bit rate MAC CE to support per QoS flow granularity indication.

Reference

- RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3 Nokia (Rapporteur)

- RP-242224 On RAN Awareness and UL Rate Control for XR

- R2-2212189, Reply LS to SA2 on XR, vivo, Huawei

- TS 38.321 Medium Access Control (MAC) protocol specification V18.2.0 (2024-06)

- TR 26.928 Extended Reality (XR) in 5G V18.0.0