3GPP TSG-RAN WG2 Meeting #127bis                                                   R2-2408003

Hefei, China, 14th – 18th Oct, 2024

Source:	CATT

Title:	Clarification on the SN Gap Report

Agenda Item:	7.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, the details of SN gap report will be further discussed.

Discussion

In subclause 5.3 of TS38.323 [1], the following was captured:

## 5.3 SDU discard When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs. NOTE 2: Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

Based on the above description, it is obvious that there are two kinds of discarded PDCP SDU:

- When the successfully delivery of PDCP SDU is confirmed by PDCP status report;

- When the discardTimer or discardTimerForLowImportance expiries.

Observation 1: Based on the description in subclause 5.3 of TS38.323, there are two kinds of discarded PDCP SDUs:

- PDCP SDUs discarded based on PDCP status report;

- PDCP SDUs discarded based on discardTimer/discardTimerForLowImportance expiries.

In subcluase 5.16 of TS38.323 [1], when describing the SN gap report, the following was captured:

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

Observation 2: Based on the description in subclause 5.16 of TS38.323, when discussing the SN gap report trigger, the discarded PDCP SDUs which has been submitted by RLC to lower layers were excluded, which includes:

- PDCP SDUs discarded based on PDCP status report;

- PDCP SDUs discarded based on discardTimer/discardTimerForLowImportance expiries but have been submitted by RLC to lower layers.

However, in subcluase 5.16 of TS38.323 [1], when describing how to compile the PDCP SN gap report, the following was captured:

If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s); - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

Based on the above description, when discussing the compiling SN gap report, only “discarded PDCP SDU(s)” were mentioned, it is unclear whether the PDCP SDU discarded based on PDCP status report or based on discard timer expiries should be reflected or not.

For example, as shown in the following Figure-1, the PDU Set A includes four PDUs (PDU 1~4), the transmission state of each PDU is as below:

Figure-1 Illustration of the state of each PDU in one PDU Set

At T1, discard timer of PDU 1 expiry, if pdu-SetDiscard is configured, all PDCP SDUs including PDU1, PDU2, PDU3 and PDU4 should be discarded and PDU3 will trigger PDCP SN gap. In this case, although PDU 1/2/3/4 are discarded simultaneously, but only PDU3 and PDU 4 should be indicated in the SN gap report. But according to the description in subclause 5.16 of TS38.323, ''setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s) '', it is unclear whether PDU 1 or PDU 3 should be indicated in the FDC field. For another example, ''setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded'', based on the above description, PDU 2 should be indicated as '1', but in fact, it should not be included.

Observation 3: Based on the description in subclause 5.16 of TS38.323, when discussing the compiling SN gap report, only ''discarded PDCP SDU(s) '' were mentioned, it is unclear whether the PDCP SDU discarded based on PDCP status report or based on discard timer expiries but has been submitted by RLC to lower layers should be included or not.

Based on Observation3, suggest RAN2 to clarify when setting the control PDCP PDU of PDCP SN gap report, it should clarify the discarded PDCP SDUs only refers to the PDCP SDU(s) which has not been submitted by RLC to lower layers.

Proposal 1: Suggest RAN2 to discuss when setting the PDCP SN gap report whether it should clarify the discarded PDCP SDUs only refer to the PDCP SDU(s) which has not been submitted by RLC to lower layers.

Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: Based on the description in subclause 5.3 of TS38.323, there are two kinds of discarded PDCP SDUs:

- PDCP SDUs discarded based on PDCP status report;

- PDCP SDUs discarded based on discardTimer/discardTimerForLowImportance expiries.

Observation 2: Based on the description in subclause 5.16 of TS38.323, when discussing the SN gap report trigger, the discarded PDCP SDUs which has been submitted by RLC to lower layers were excluded, which includes:

- PDCP SDUs discarded based on PDCP status report;

- PDCP SDUs discarded based on discardTimer/discardTimerForLowImportance expiries but have been submitted by RLC to lower layers.

Observation 3: Based on the description in subclause 5.16 of TS38.323, when discussing the compiling SN gap report, only ''discarded PDCP SDU(s) '' were mentioned, it is unclear whether the PDCP SDU discarded based on PDCP status report or based on discard timer expiries but has been submitted by RLC to lower layers should be included or not.

Based on the above observations, it is proposed that:

Proposal 1: Suggest RAN2 to discuss when setting the PDCP SN gap report whether it should clarify the discarded PDCP SDUs only refer to the PDCP SDU(s) which has not been submitted by RLC to lower layers.

If Proposal 1 is confirmed, the corresponding TP is in the Annex.

Reference

- TS38.323  NR; Medium Access Control (MAC) protocol specification (Release-18)

Annex

START OF THE CHANGE

## 5.16	SN gap report

### 5.16.1	Transmit operation

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	the PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

If a PDCP SN gap report is triggered, the transmitting PDCP entity shall:

-	compile a PDCP SN gap report as indicated below by:

-	setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s) which have not been submitted by RLC to lower layers;

-	if more than one PDCP SDUs which have not been submitted by RLC to lower layers are discarded:

-	allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU which have not been submitted by RLC to lower layers up to and including the last discarded PDCP SDU which have not been submitted by RLC to lower layers, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded;

-	setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded and have not been submitted by RLC to lower layers;

-	submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

END OF THE CHANGE