3GPP TSG-RAN WG2 Meeting #127bis																				R2-2408006

Hefei, China, Oct 14th–18th, 2024

Agenda item:		8.5.4

Title: 	Further consideration on paging occasion adaptation

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

The following agreements have been made in RAN2#125bis [1] on paging adaptation:

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this paper, we share some further consideration on the paging occasion adaptation.

- Discussion

  - Paging occasion adaptation

The following solutions have been agreed to be further studied in RAN2#125bis:

- Solution 1: bundle paging frames

Figure 1: Bundle paging frames

- Solution 2: extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

Figure 2: Less PF in T, more PO in PF

-----------------------------------------------------------------------------------ASN.1 for PCCH-Config----------------------------------------------------------------------------------------

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16        INTEGER (2..4)               OPTIONAL  -- Cond SharedSpectrum2

]],

[[

ranPagingInIdlePO-r17                              ENUMERATED {true}            OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

]]

}

-----------------------------------------------------------------------------------ASN.1 for PCCH-Config----------------------------------------------------------------------------------------

Comparing solution 1 and solution 2, the power saving gain would be similar while solution 2 offers more flexibility, e.g. for the case when there is only one PF in each DRX cycle, solution 2 is almost the same as solution 1, and less spec impact as we only need to introduce new values for the configuration of nAndPagingFrameOffset and ns.

Thus, solution 2 is preferred from our perspective.

Also considering the progress of the email discussion [2], we understand the majority prefer to select option b, i.e. solution 2.

To support solution 2, we need to introduce new values for nAndPagingFrameOffset and ns and the following values can be considered:

| nAndPagingFrameOffset | oneThirtysecondT INTEGER (0..31), oneSixtyfourthT INTEGER (0..63), one128thT INTEGER (0..127), |
| --- | --- |
| ns | Thirty-two, sixteen, eight |

Proposal 1: Introduce new values for nAndPagingFrameOffset and ns to have increased interval between PFs and more POs per PF.

Proposal 2: Introduce value T/32, T/64 and T/128 for N and value 8, 16 and 32 for ns.

  - Compatibility of legacy UE

The following options have been agreed to be studied on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Option 1 is preferred from our perspective as if we go for option 2, network need to send paging to legacy UE following the legacy PF/PO configuration and to the new R19 NES UEs following the new configuration and if paging comes for both legacy UE and new NES UE, the power consumption and network implementation complexity would increase.

Proposal 3: Legacy RRC_IDLE/RRC_INACTIVE UE not supporting paging adaption for NES would be barred if network would like to apply this feature.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: Introduce new values for nAndPagingFrameOffset and ns to have increased interval between PFs and more POs per PF.

Proposal 2: Introduce value T/32, T/64 and T/128 for N and value 8, 16 and 32 for ns.

Proposal 3: Legacy RRC_IDLE/RRC_INACTIVE UE not supporting paging adaption for NES would be barred if network would like to apply this feature.

- Reference

- R2-2404102 RAN2#125bis Meeting Report

- Summary of 109 (Stage-2)_V19-Rapp