3GPP TSG-RAN WG2 Meeting #127bis                                                           R2-2408017

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.9.2

Source:	vivo

Title:	RAN2 Aspect for S&F Operation

Document for:	Discussion and Decision

- Introduction

In RAN2#127, support of the S&F was discussed by RAN2. The following agreements were made [1].

RAN2#127 Agreements: - RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In the contribution, we will continue to discuss the following remaining issues to support S&F,

- S&F indication and access control for S&F;

- Additional S&F information;

- Satellite ID list provided by NAS.

- Discussion

  - S&F indication and access control for S&F

According to Agreement 4 given above, it is FFS whether we should also provide a static indication that the cell supports the S&F. From our perspective, the static indication is not needed considering we already introduced the dynamic S&F operation indication. When the NW is operating in S&F mode, UEs can decide the feeder link is not available based on the S&F dynamic operation indication. When the NW is operating in non-S&F mode, it can provide better service compared with when it is operating in S&F mode, UEs can just consider it as normal cells and access the cell if needed. Thus, there is no need for NW to additionally provide the static S&F indication.

Proposal 1: No need to introduce the static indication of whether the NW supports the S&F.

Further, regarding the way to bar the legacy UEs from accessing the cell operating in S&F mode, we think a new S&F cell barred indication is preferred. This kind of design is more aligned with the current spec to avoid legacy UE accessing cells with a new feature, i.e., UE decides whether the cell can be accessed or not based on the cell barred indication but not the cell operating mode or the cell type. The S&F barred indication should be provided by the cell only when the cell operates in S&F mode, which is similar to the presence of the dynamic S&F operation indication. To avoid repeated information, it is proposed that the dynamic S&F operation indication can be enabled via the S&F cell barred indication.

Proposal 2: Introduce the new S&F cell barred indication in SIB1, i.e., NW includes the S&F cell barred indication in case the cell operates in S&F mode, otherwise it is not included.

Proposal 3: The information on whether the cell is operating in S&F mode is enabled via the S&F cell barred indication, i.e., when the S&F cell barred indication is provided, UE considers the cell operates in S&F mode; otherwise, UE considers the cell operates in normal node.

Additionally, in the case where S&F cell barred indication is present, the S&F capable UEs shall consider the cell operating in S&F mode and ignore the legacy cell barred indication. To enable the S&F capable UEs to decide whether the cell is allowed to access or not, the S&F cell barred indication should further include the cell access information, i.e., barred or not barred for S&F access.

Proposal 4: The new S&F cell barred indication further includes the values “barred, notBarred” to control whether the S&F capable UEs can access the cell or not.

Then based on the newly introduced S&F cell barred indication, the UE behaviours can be like the following:

- For legacy UEs and R19 UEs not supporting the S&F operation, use the legacy cellBarred and cellBarredNTN to decide whether to access the cell ;

- For R19 UEs supporting the S&F cell operation:

- If the new S&F cell barred indication is present, they can decide the cell works at S&F mode, and the UE access is based on the value within the S&F cell barred indication, e.g., the cell is allowed to access if the new S&F cell barred indication is set to “barred”, otherwise the cell is not allowed to access.

- If the new S&F cell barred indication is not present, UE can follow the legacy cellBarred and cellBarredNTN.

Thus, we propose,

Proposal 5: For legacy UEs and R19 UEs not supporting the S&F operation, the UE shall follow the legacy cellBarred and cellBarredNTN indication.

Proposal 6: For R19 UEs supporting the S&F operation, the UE shall:

- follow the legacy cellBarred and cellBarredNTN indication if the new S&F cell barred indication is not present;

- consider the cell is not allowed for S&F operation if the new S&F cell barred indication is set to “barred”;

- consider the cell is allowed for S&F operation if the new S&F cell barred indication is set to “not barred”.

  - Additional S&F information

Depending on the satellite movement, the cell may operate in S&F mode at some times and normal regeneration payload mode at other times. The regeneration payload mode provides better performance than S&F mode, and UEs supporting the S&F mode can work well in the cell with regeneration payload mode, so in case the cell’s operation mode changes from S&F mode to regeneration payload mode, no additional information needs to be provided to UE since no specifical handling is required for UEs.

Proposal 7: In case the cell operates in S&F mode, no additional information except the S&F cell barred indication is provided by the cell.

If the cell is currently working on regeneration payload mode, both the S&F capable UEs and the S&F not capable UEs may work in the cell. Then, when the cell changes its operation mode from regeneration payload to S&F mode, UEs without S&F capability may not work well in the cell. If S&F not capable UE is in RRC_CONNECTED, it can rely on NW implementation to handle these UEs, i.e., NW can trigger the handover of the UEs to cells in regeneration payload mode. However, for S&F not capable UE in RRC-IDLE, this can only be enabled via cell reselection to the cells operating in regeneration payload mode based on the assistance information provided by NW, i.e., the time information when the cell will change its regeneration payload mode to S&F mode.

Currently, T-service which indicates when the NTN cell is going to stop serving the area it is currently covering can be provided by NW for the EFC cell or the feeder link switch case [2]. In general, we think we can reuse the T-service to provide the time information when the cell will turn to S&F mode from regeneration payload mode.

t-Service Time information on when an NTN cell is going to stop serving the area it is currently covering, as specified in TS 36.304 [4]. This field applies for service link switches in NTN quasi-Earth fixed cells and feeder link switches for both NTN quasi-Earth fixed and earth-moving cells.

Proposal 8: In case the cell operates in regeneration payload mode, time information (i.e. t-Service) when the cell will change to S&F mode from regeneration payload mode can be provided by NW to assist S&F not capable UE in performing cell reselection.

  - Satellite ID list provided by NAS

According to the TR23.700-29 [3], MME can provide a list of satellite IDs to UE via the NAS message. The satellite IDs are used for UE to decide which cell/satellite can re-attempt to perform the subsequent attach/TAU procedure after the wait time expires. From this perspective, it is clear that the satellite ID provided by NAS is globally unique.

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

Observation 1: To assist UE perform subsequent attach/TAU, the satellite ID provided by the NAS is globally unique.

Take the satellite assistance information for neighbour cells as an example, the satellite ID is used to identify a satellite of the serving satellite or the neighbour satellites. In general, no coordination procedure on the satellite ID is specified and the index is allocated by the cell itself. Thus, we assume the common understanding is that that index is unique only within a service cell.

#### – SatelliteId The IE SatelliteId is used to identify the satellite assistance information of the serving or neighbour satellites. SatelliteId information element -- ASN1START SatelliteId-r18 ::= INTEGER (0..255) -- ASN1STOP

Observation 2: The satellite ID used in the RAN is not a world global ID, i.e., it is allocated by the eNB and it uniquely identifies a satellite within the cell.

Besides, according to the SA2, the satellite IDs are based on the SIB information broadcasted by eNB. However, this is not possible if the satellite IDs used in NAS by SA2 are globally unique while the satellite IDs used in eNB are unique within the cell. If RAN2 makes enhancements to provide the global satellite IDs and its corresponding satellite assistance information, the signalling overhead is large and may not be enough within one SIB message (at most 8 satellite information can be included). Further, before SA2 made the initial conclusion, SA2 may not have been aware of the issue, i.e., the satellite ID used in RAN is unique within one cell. So it is proposed that RAN2 sends LS to SA2 telling them the issue to trigger SA2 to discuss how to handle it.

Proposal 9: RAN2 confirms that UE cannot recognize a unique satellite marked with monitoring satellite ID provided by NAS, considering that only local satellite ID is used in Uu interface.

Proposal 10: RAN2 to send an LS to indicate the issue to SA2 and ask SA2 to further discuss how to handle it.

- Conclusion

In this contribution, we have further discussed the RAN2 aspects for S&F operation. And we make the following observations and proposals:

S&F indication and access control for S&F:

Proposal 1: No need to introduce the static indication of whether the NW supports the S&F.

Proposal 2: Introduce the new S&F cell barred indication in SIB1, i.e., NW includes the S&F cell barred indication in case the cell operates in S&F mode, otherwise it is not included.

Proposal 3: The information on whether the cell is operating in S&F mode is enabled via the S&F cell barred indication, i.e., when the S&F cell barred indication is provided, UE considers the cell operates in S&F mode; otherwise, UE considers the cell operates in normal node.

Proposal 4: The new S&F cell barred indication further includes the values “barred, notBarred” to control whether the S&F capable UEs can access the cell or not.

Proposal 5: For legacy UEs and R19 UEs not supporting the S&F operation, the UE shall follow the legacy cellBarred and cellBarredNTN indication.

Proposal 6: For R19 UEs supporting the S&F operation, the UE shall:

- follow the legacy cellBarred and cellBarredNTN indication if the new S&F cell barred indication is not present;

- consider the cell is not allowed for S&F operation if the new S&F cell barred indication is set to “barred”;

- consider the cell is allowed for S&F operation if the new S&F cell barred indication is set to “not barred”.

Additional S&F information:

Proposal 7: In case the cell operates in S&F mode, no additional information except the S&F cell barred indication is provided by the cell.

Proposal 8: In case the cell operates in regeneration payload mode, time information (i.e. t-Service) when the cell will change to S&F mode from regeneration payload mode can be provided by NW to assist S&F not capable UE in performing cell reselection.

Satellite ID list provided by NAS:

Observation 1: To assist UE perform subsequent attach/TAU, the satellite ID provided by the NAS is globally unique.

Observation 2: The satellite ID used in the RAN is not a world global ID, i.e., it is allocated by the eNB and it uniquely identifies a satellite within the cell.

Proposal 9: RAN2 confirms that UE cannot recognize a unique satellite marked with monitoring satellite ID provided by NAS, considering that only local satellite ID is used in Uu interface.

Proposal 10: RAN2 to send an LS to indicate the issue to SA2 and ask SA2 to further discuss how to handle it.

- Reference

- Draft_R2_127_Meeting_Report_v1.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- TR 23.700-29, Study on integration of satellite components in the 5G architecture.