3GPP TSG-RAN WG2 Meeting #127bis                                                              R2-2408018

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.9.3

Source:	vivo

Title:	Discussion on CB-Msg3 EDT Enhancement

Document for:	Discussion and Decision

- Introduction

In the RAN2#127 meeting, it was agreed to support the contention-based Msg3 EDT mechanism. The corresponding achieved agreements are listed as follows [1],

RAN2#127 Agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism. - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts. - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In this contribution, we will discuss the contention-based Msg3 EDT enhancement for IoT-NTN, concerning the following two aspects,

- Random access mode with performance comparison amongst SA, DSA, CRDSA, SA-OCC, and SA-backoff;

- UE TA handling.

- Discussion

2.1 Random access mode for CB-Msg3

For CB-Msg3 EDT-like mechanism, we understand that the network will provide CB-Msg3 PUSCH resources via broadcast singalling (e.g. SIB1) for several UEs for bursty data transmission. For the UEs that trigger CB-Msg3 EDT enhanced procedure, they will select a CB-Msg3 PUSCH occasion for CB-Msg3 transmission, as shown in the Figure below. Different UEs may select the same occasion (thus leading to collision) or different occasions based on the random access mode (e.g. randomly selecting an occasion).

Figure 1: CB-Msg3 transmission

For throughput maximization and packet loss minimization, how to control the collision and congestion on the same CB -Msg3 PUSCH resource becomes an important issue (i.e. The collision and congestion should be controlled under a specific level, as once the collision and congestion are higher than that level, generally, the system throughput will degrade dramatically due to high inter-UE interference).

In the previous RAN2 meetings, Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) were proposed as an efficient random access mode for CB-Msg3 transmission. Different companies have different views on the performance gain and implementation complexity. Some believe that DSA and CRDSA can improve packet loss performance while some argue that replica transmission of DSA and CRDSA decreases the peak system throughput, compared with the legacy Slotted ALOHA (SA) mode.

To comprehensively understand the random access modes in terms of UL throughput and packet loss rate (PLR), we have built a  Monte Carlo based system-level simulator, with which each point of the simulation results is averaged over 50000 CB-Msg3 occasions. The CB-Msg3 occasions are modeled based on a time periodicity pattern and each occasion is capable of 2 repetitions (i.e. RepK=2), as shown in the figure below. All the UEs may have a new packet arrival at every 10 CB-Msg3 periodicity (e.g. at occasion #0, occasion #10), thus naming a traffic period (also for SIC buffering length) that equals 10 times of CB-Msg3 occasion periodicity. More simulation details are given in Annex.

Figure 2: CB-Msg3 resources provided by network

All the random access modes for comparison are described as follows,

1. SA:

SA, in our understanding, can be regarded as a technique with which the UE can randomly select one available occasion to transmit the pending Msg3 within the traffic period. It has been used for the PUR occasion selection and PRACH slot selection. In the simulation, retransmission is not considered for SA mode. The network detects all the occasions one by one. If the network cannot successfully decode any data on a CB-Msg3 occasion, it will not send retransmission. The transmitted packet from UE is considered lost directly.

2. SA-BI:

The difference between SA and SA-BI is that once the transmitted packet is not successfully decoded, then the network using SA-BI can send a backoff indicator to UE. Subsequently, the UE retransmits the packet (i.e. the UE prioritizes the retransmission over the new pending packet transmission). If the retransmitted packet cannot be successfully decoded again at the network side, the transmitted packet from UE is considered lost. The UE will trigger the new transmission for the pending packet in the next traffic period.

3. DSA2:

DSA allows UE to transmit multiple copies (i.e. replica number is 2 in our simulation case) of the same packet, where the multiple copies are transmitted on the same sub-carriers but spaced apart by random time intervals (time diversity) within the same traffic period. At the network side, it individually decodes each replica (i.e. no soft combining). If all the replicas are not successfully decoded, the transmitted packet from UE is considered lost.

4. CRDSA2:

CRDSA is enhanced based on the transmission of a chosen number of copies as in DSA. The difference is that the time-domain diversity pattern is included in the MAC PDU (e.g. MAC CE carries the occasion index which the UE selects for all the replicas). For simplicity, the overhead of that pattern is ignored for throughput counting. Once the network has successfully decoded one replica within a traffic period, it can read the time pattern MAC CE and start the Successive Interference Cancellation (SIC) process for the other corresponding occasions within the same traffic period.

5. SA-OCC2:

Orthogonal cover codes (OCC) can be used for UL transmission with repetition to multiplex multiple signals from different UEs in the same time and frequency domain resource. The orthogonality of OCC sequence can be maintained across multiple UEs, if the power consistency and phase continuity of the repeated signals and channel coefficient are not changed dramatically during the time span of an OCC sequence (which is extremely suitable for NTN case). We consider inter-slot time domain OCC2 across 2 occasions in the simulation. The UE randomly selects the occasion in the manner as SA is done and further randomly selects an OCC sequence from a total of 2 sequences.

The following figure shows how the above five random access modes work.

Figure 3: Random access modes for CB-Msg3

The simulation input is the channel load, named G, which is measured in packets/occasion, and it is obtained as follows,

G=Number_of_active_usersNumber of CB-Msg3 occasions per traffic period .

The outputs are throughout (i.e. the total number of successfully decoded packets / the total number of CB-Msg3 occasions, in the unit of packets/occasion) and PLR, respectively. The simulation outputs are given below,

Figure 4: Throughput performance

Figure 5: PLR performance

Combining with Figure 4 and Figure 5, we can observe that the inter-slot OCC solution for the NPUSCH currently discussed in RAN1 achieves higher peak performance than DSA or CRDSA in terms of both throughput and PLR. The reason is obvious since OCC enables multiple UEs MIMO transmission. For OOC2, MIMO transmission with 2 transmission layers at maximum is practical which enables 2 times of UL throughput in theory. If OCC4 is adopted, higher UL throughput can be expected thanks to MIMO transmission with 2 transmission layers enabling. For DSA or CRDSA, only 1 transmission layer can be used for UL transmission from the system perspective. In other words, OOC will be a tough act to follow if the maximum throughput is the final objective.

Nothing is absolute. We can also observe that either DSA or CRDSA achieves a higher performance gain than OCC when the channel load is low (i.e. G=0.1~0.2 for DSA and G is below 0.6 for CRDSA). On one hand, that is because the replica transmission offers more additional time diversity gain. On another hand, when the load is low, there will likely be a CB-Msg3 occasion without collision. Then the SIC processing can based on that clean occasion relieve and even kill all the collisions within the traffic period in such load-light case. Those gains cannot be provided by OCC (i.e. if multiple UEs select the same OCC sequence, the performance is degraded to the same as the SA case). But please note that the performance gap between OCC2 and CRDSA2 with ideal SIC is quite small. With load creases, replica transmission makes collision probability even higher due to more occasions (for replica repetition) being selected by each UE. As a result, the performance of SA and CRDSA degrades significantly when the channel load becomes higher than a specific threshold. What’s worse, the performance of both DSA and CRDSA will be worse than that of SA when G is larger than 1. And the performance of both DSA and CRDSA converges to the same level when the load is extremely high as SIC doesn’t work at all in such a high load.

Furthermore, to our surprise, the performance gain difference between SA, SA-BI, and DSA2 is quite small. For example, at a load-light case (i.e. G=0.4), from throughout point of view, DSA2 only takes a 2.2% (at most 7.8% at G=0.3) advantage over SA while the gap between SA and SA-BI is less than 1.2% (but SA-BI significantly increases PLR performance thanks to one-shot retransmission enabling). Thus the BI-reattempt (like PRACH) solution should be adopted, regardless of which access mode is adopted considering its simplicity and performance gain.

Observation 1: OCC provides the best peak performance gain in both UL throughput and packet loss rate.

Observation 2: DSA and CRDSA provide performance gain in both UL throughput and packet loss rate only when the channel load is light (e.g. G <= 0.3), at the cost of more UE power consumption.

Observation 3: The performance gain in both UL throughput and packet loss rate between OCC and CRDSA with ideal SIC is quite near (i.e. within 1.5%) when the channel load is light or medium (e.g. G <= 0.6).

Observation 4: The performance of DSA and CRDSA is worse than SA in both UL throughput and packet loss rate when the channel load is high (e.g. G >= 1).

Observation 5: Compared with SA, DSA leads to more than 10% of maximum UL throughput degradation while the performance gain in both UL throughput and packet loss rate is less than 10% when the channel load is light.

Observation 6: Enanbling retransmission after backoff time has negligible impacts on UL throughput while significantly improving packet loss rate performance in any channel load case. It is simple but efficient.

Observation 7: Adopting DSA and CRDSA for contention-based Msg3 transmission requires specific RAN1/RAN2 spec modification.

Based on the above analysis, the main characteristics of these 5 random access modes are categorized here:

Table 1: Random access mode summary

| Characteristics | SA | DSA2 | CRDSA2 | SA-BI | SA-OCC2 |
| --- | --- | --- | --- | --- | --- |
| UE resource selection | UE randomly selects an occasion. | UE randomly selects the 1st and 2nd occasions within the traffic period. | UE selects 2 occasions as DSA and additionally indicates the occasion time pattern. | Same as SA | UE selects an occasion as DSA and an OCC sequence. |
| NW decoding | Individual decoding | Same as SA | SIC Decoding within the traffic period. | Same as SA | Individual decoding with OCC scrambling |
| UE response reception | UE starts the response window for contention resolution after the occasion | UE starts multiple response windows after each replica transmission | UE starts the response window for contention resolution after the traffic period. | UE starts the response window for BI or contention resolution | UE starts the response window for contention resolution |
| Throughput | Up to 0.38 | 1. Up to 0.33 2. At most 7.8% gain over SA | 1. Up to 0.48 2. Best if G<=0.6 | 1. Up to 0.38 2. At most 1.2% gain loss than SA | 1. Up to 0.76 2. Best if G>0.6 3. 58% gain over CRDSA2 at peak |
| PLR | 10%@G≈0.2 | 10%@G≈0.30 | 10%@G≈0.42 | 10%@G≈0.26 | 10%@G≈0.32 |
| Potential Spec Impacts | Reuse existing scheme. | 1. RAN1/2 may have to model replica transmission. 2. RAN2 extra modification is needed, e.g. enhancing resource selection method, supporting multiple response windows. | 1. RAN1/2 may have to model replica transmission. 2. RAN1 may have to define occasion pattern period for SIC. 3. RAN2 has to specify the signalling to indicate occasion time pattern. | Reuse existing scheme with BI. | Reuse OOC scheme with UE selection OCC sequence. |

In our understanding, OCC seems the best choice for CB-Msg3 PUSCH, which provides significant performance gain from both UL capacity and PLR points of view with reasonable potential RAN1/2 spec impacts. The potential issues may be whether the UE pre-compensation for frequency offset and UL timing is accurate enough for multiple UE multiplexing with OCC and whether all the OOC multiplexing factors (e.g. OCC2, OOC4) are suitable for the IDLE state case. It is better to send an LS to RAN1, asking for the feasibility of CB-Msg3 with OCC.

Proposal 1: RAN2 firstly asks RAN1 whether the OCC solution for NPUSCH on which they are working can be applied to contention-based Msg3 PUSCH or not.

Proposal 2: Support re-attempt after backoff time (like PRACH) for contention-based Msg3 PUSCH.

2.2 UE TA handling

In the previous meeting, RAN1 and RAN4 had discussed whether an RRC Idle UE with UE-adjusted pre-compensated TA  can satisfy the required timing accuracy for CB-Msg3 PUSCH transmission or not. The corresponding feedback is given as follows [2][3],

From RAN1 perspective: Assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B. From RAN4 perspective: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133 from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP.

According to the feedback from RAN1 and RAN4, we know that the existing UE timing requirement can be reused for CB-Msg3 transmission (i.e. UE shall fulfill the timing requirement for CB-Msg3 transmission). Then when transmitting the CB-Msg3, the time difference (i.e. timing error, as shown in Figure 6) between the starting point of CB-Msg3 at the network received side and the starting point of CB-Msg3 allocation is variable based on the actual implementation error (e.g. the sum of UE timing error and NW timing error). Further, it can be concluded that the timing error may be larger than half CP in the case of both NB-IoT with 15kHz SCS and eMTC CE with mode B. As a result, the decoding performance at the NW side will largely degrade, leading to UE packet loss.

Figure 6: Timing error in CB-Msg3 transmission

Observation 8: The timing error of CB-Msg3 will lead to decoding failure in case of NB-IoT with 15kHz SCS and eMTC CE with mode B.

To avoid potential decoding performance degradation, we think TA validation (e.g. the mechanism used for DPUR with TA validation timer) is needed at least for NB-IoT with 15kHz SCS and eMTC CE with mode B (i.e. the UE can only use the CB-Msg3 resource on the cell where RRC Release is received and the TA is still valid). If companies prefer not to have any TA validation, we think the CB-Msg3 EDT enhancement may only apply to NB-IoT with 3.75kHz SCS and eMTC CE mode A, shrinking the use case of CB-Msg3 EDT enhancement. That is definitely an overkill. Thus, we think RAN2 should consider that TA validation is mandatory for CB-Msg3 with NB-IoT with 15kHz SCS and eMTC CE with mode B. With that, the TA error will be smaller than half CP, thanks to the previous TA adjustment in CONNECTED mode. Thus, we propose,

Proposal 3: RAN2 confirms TA validation mechanism is only needed for CB-Msg3 mechanism in NB-IoT with 15kHz SCS or eMTC CE mode B.

- Conclusion

In this contribution, we have provided our understanding of the contention-based Msg3 EDT enhancement for IoT-NTN, concerning the random access mode and UE TA handling. And we made the following observations and proposals:

Random access mode for CB-Msg3:

Observation 1: OCC provides the best peak performance gain in both UL throughput and packet loss rate.

Observation 2: DSA and CRDSA provide performance gain in both UL throughput and packet loss rate only when the channel load is light (e.g. G <= 0.3), at the cost of more UE power consumption.

Observation 3: The performance gain in both UL throughput and packet loss rate between OCC and CRDSA with ideal SIC is quite near (i.e. within 1.5%) when the channel load is light or medium (e.g. G <= 0.6).

Observation 4: The performance of DSA and CRDSA is worse than SA in both UL throughput and packet loss rate when the channel load is high (e.g. G >= 1).

Observation 5: Compared with SA, DSA leads to more than 10% of maximum UL throughput degradation while the performance gain in both UL throughput and packet loss rate is less than 10% when the channel load is light.

Observation 6: Enanbling retransmission after backoff time has negligible impacts on UL throughput while significantly improving packet loss rate performance in any channel load case.

Observation 7: Adopting DSA and CRDSA for contention-based Msg3 transmission requires specific RAN1/RAN2 spec modification.

Proposal 1: RAN2 firstly asks RAN1 whether the OCC solution for NPUSCH on which they are working can be applied to contention-based Msg3 PUSCH or not.

Proposal 2: Support re-attempt after backoff time (like PRACH) for contention-based Msg3 PUSCH.

UE TA handling:

Observation 8: The timing error of CB-Msg3 will lead to decoding failure in case of NB-IoT with 15kHz SCS and eMTC CE with mode B.

Proposal 3: RAN2 confirms TA validation mechanism is only needed for CB-Msg3 mechanism in NB-IoT with 15kHz SCS or eMTC CE mode B.

- Reference

[1] Draft_R2_127_Meeting_Report_v1.

[2] R1-2405786, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2.

[3] R4-2414114, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2.

- Annex

System-level simulation assumptions:

We consider a multi-access channel populated by a total number of active users M, ranging from [1,25]. All UEs with transmission are perfectly synchronized in both time and DL/UL frequency, randomly scattered around a 50km LEO cell without mobility, as shown in the figure below. No beam selection is considered. The network operates with 15 kHz SCS.

Figure 7: Simulation scenario

Perfect SIC and ideal channel estimation (i.e. no fading at all, only pathloss is considered) are assumed, which means that the receiver is always able to perfectly apply SIC without errors and noise is considered as the extra external input.

For CRDSA, a slot location pointer is included in each packet copy (the pointer indicates all the associated slots in the time domain for restoring collided packets via SIC at the NW receiver). As mentioned above, this overhead is not considered for throughput counting.

For OCC2, all the UEs are considered with perfect TA&FO pre-compensation (i.e. all the transmit packets from different UEs are aligned at the network receiver side). The received power of all the UEs at the NW side is basically aligned since the channel difference of different UEs within the cell is negligible (due to small cell size and no fading channel). 2 Rx at the network is used for OCC2 reception (only 1 Rx for the other cases).

Assuming -6 dB AWGN noise level and the 10^-1 BLER decoding thresholds are 6.6 dB for MCS#16 with 2 repetitions (RV sequence 0-0). For simplicity, we assume the CB-Msg3 PUSCH cannot be successfully decoded once the SINR calculated is below the threshold. Similarly, the PUSCH can be successfully decoded if the SINR is above the threshold.

With the above assumptions, it can be concluded that the only cause of the unsuccessful decoding of a packet comes from the interference among the other UEs using the same transmit resource and the noise. Therefore, each occasion can be in one of three states:

- No packet has been placed in a given slot, thus the slot is empty; Neither throughput nor packet loss is counted.

- Only 1 packet (with a replica) has been transmitted in a given slot, thus the packet can be correctly decoded.

- More than 1 packets (with a replica) has been placed in a given occasion, thus resulting in a collision. It is possible that one or more than one packet can be successfully decoded or none of them can be successfully decoded, depending on the interference level (whether the SINR is high enough for successful decoding).