3GPP TSG-RAN WG2 Meeting #127bis	R2-2408029

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	7.17.2

Source:	Huawei, HiSilicon

Title:	Clarification for the initiation of the MUSIM proactive UAI after HO/CHO

Document for:   Discussion and decision

Introduction

In the existing specification as cited below from section 5.3.5.3 [1], if UE reports UAI to the source gNB during the last 1 second before receiving the HO command or if UE reports UAI to the source gNB before the CHO occurs, UE needs to initiate transmission of a UAI to the target gNB, and provides the concerned UAI again since the source gNB is not able to forward this UAI to the target gNB in the above two cases.

2> if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup: 3> if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or 3> if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2: 4> initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information; 4> start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration; 4> start or restart the leave without response timer (if exists) or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

For the MUSIM UAI introduced in Rel-18, one issue on the proactive signaling was identified for the above two cases and it is discussed in this contribution.

Discussion

The UE may initiate the UAI transmission in two general cases, i.e. when the UE has corresponding problem in the current serving cell (as specified in 5.7.4.3 [1]), or when the UE performs HO/CHO (as specified in 5.3.5.3 [1]). One common condition that UE can initiate the UAI transmission in these two cases is the UE is configured by the gNB to provide the corresponding UE assistance information in OtherConfig. For the former case, such conditions are specified in detail for each UAI, while for the latter case, there is only a general description, i.e. “the UE is still configured to provide the concerned UE assistance information”. Compared with other UAI, we observed that the MUSIM UAI for temporary capability restriction with proactive signaling is a special one with respect to the relationship between the gNB’s configuration and the condition for UE to initiate the UAI transmission. This is explained with the following Table 1. For simplicity, we only list the MUSIM UAI rather than all the UAI here.

Table 1 Mapping between gNB’s configuration and condition for UE to initiate the UAI transmission

| MUSIM UAI | gNB’s configuration | Condition for UE to initiate the UAI transmission |  |
| --- | --- | --- | --- |
| For leaving RRC_CONNECTED | musim-LeaveAssistanceConfig | - if configured to provide MUSIM assistance information for leaving RRC_CONNECTED: 2> if the UE needs to leave RRC_CONNECTED state and the timer T346g is not running |  |
| For gap preference | musim-GapAssistanceConfig | - if configured to provide MUSIM assistance information for gap preference: ….. 3> if the UE has a preference on the MUSIM gap(s)…. |  |
| For gap priority | musim-GapPriorityAssistanceConfig | - if configured to provide MUSIM assistance information for gap priority preference: 3> if the UE has a preference on the MUSIM gap(s)…. |  |
| For temporary capability restriction | musim-CapabilityRestrictionConfig | Reactive | - if configured to provide MUSIM assistance information for temporary capability restriction: 2> if the UE has temporary capability restriction on the current configuration and the timer T348 is not running |
|  |  | Proactive | - if configured to provide MUSIM assistance information for temporary capability restriction: 2> if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList …… |
|  |  | maxCC | - if configured to provide MUSIM assistance information for temporary capability restriction: 2> if the UE has temporary capability restriction on the maximum CC number….. |

From the table, it can be observed that except for the MUSIM temporary capability restriction with proactive signaling, the condition as highlighted in green for the UE to initiate the UAI transmission is only depending on the gNB’s configuration for the concerned UAI in OtherConfig. However, for the MUSIM temporary capability restriction with proactive signaling, there is a hidden condition as highlighted in yellow beside the condition (as highlighted in grey) that depends on the gNB’s configuration for the MUSIM temporary capability restriction. This hidden condition implies that the gNB needs to provide the MUSIM-CandidateBandList in the musim-CapabilityRestrictionConfig.

Then for the HO/CHO case, one question is how to understand the condition “the UE is still configured to provide the concerned UE assistance information”. For the other UAI except for the MUSIM temporary capability restriction with proactive signaling, as long as the target gNB configures the concerned IE in the OtherConfig, the UE can consider this condition is satisfied. For example, if the target gNB configures the musim-GapAssistanceConfig, the UE shall consider it is configured to provide the UAI for reporting the MUSIM gap preference. However, for the MUSIM temporary capability restriction with proactive signaling, if the UE considers it is configured to provide the UAI with the temporary capability restriction on band combination by just checking that the IE musim-CapabilityRestrictionConfig is configured by the target gNB, there will be a problem in the following scenario.

- Scenario:

- Source gNB configured musim-CapabilityRestrictionConfig with MUSIM-CandidateBandList for the UE;

- UE only transmitted UAI with MUSIM temporary capability restriction on band combination to the source gNB, and HO occurs within 1 second after the UAI transmission;

- Target gNB configured musim-CapabilityRestrictionConfig but without MUSIM-CandidateBandList for the UE;

- The UE determines it is configured to provide the UAI with MUSIM temporary capability restriction and hence initiates the UAI transmission in the target gNB according to 5.3.5.3 [1].

- Problem:

- After the initiation of the UAI transmission, the UE shall perform the step to set the contents of the UAI according to 5.7.4.3 [1]. In this step, the UE will find the following condition can’t be satisfied. So, the UE can set nothing in the UAI.

- if UE has a preference to indicate band(s) and/or combination(s) of bands with capabilities restricted which comprise of the band(s) that is/are indicated in musim-CandidateBandList:

- However, according to 5.7.4.3 [1] the UE shall still go to the step to transmit the UAI, which results in that the UE can only transmit an empty UE assistance information message. This is not a desired behavior for both UE and gNB.

One may argue that in the HO/CHO case, with the current text, it is clear that when the UE checks whether it is configured to provide the UAI with MUSIM temporary capability restriction on the band(s) or combination(s) of bands, it also needs to check whether the candidate band list is provided. However, as explained above, this is a special UAI case, and we are concerned that it may be misinterpreted when implementing this and thereafter results in the above issue. So, for such UAI, we think a clarification to the condition that “the UE is still configured to provide the concerned UE assistance information” in the HO/CHO cases is needed.

Proposal 1: RAN2 agree that for the UAI with MUSIM temporary capability restriction on the band(s) or combination(s) of bands, a clarification to the condition that “the UE is still configured to provide the concerned UE assistance information” in the HO/CHO cases is needed.

The root cause of this problem is the special configuration on the condition for the UE to initiate the UAI transmission with MUSIM temporary capability restriction on the band(s) or combination(s) of bands. So, to address the issue above, we think the simplest way is to add a Note for the HO/CHO case in section 5.3.5.3 [1] to clarify that: For UE assistance information with MUSIM temporary capability restriction on the band or combination(s) of bands, the UE shall consider it is still configured to provide such UE assistance information only if the musim-CandidateBandList-r18 is included in MUSIM-CapabilityRestrictionConfig.

Proposal 2: RAN2 agree to add the following NOTE in section 5.3.5.3 in TS 38.331:

NOTE X: For UE assistance information with MUSIM temporary capability restriction on the band(s) or combination(s) of bands, the UE considers it is configured to provide such UE assistance information only if the musim-CandidateBandList is included in MUSIM-CapabilityRestrictionConfig.

For proposal 2, the corresponding draft CR has been provided in the Appendix.

Conclusion

In this contribution, we have the following proposals and provide the draft CR in Appendix:

Proposal 1: RAN2 agree that for the UAI with MUSIM temporary capability restriction on the band(s) or combination(s) of bands, a clarification to the condition that “the UE is still configured to provide the concerned UE assistance information” in the HO/CHO cases is needed.

Proposal 2: RAN2 agree to add the following NOTE in section 5.3.5.3 in TS 38.331:

NOTE X: For UE assistance information with MUSIM temporary capability restriction on the band(s) or combination(s) of bands, the UE considers it is configured to provide such UE assistance information only if the musim-CandidateBandList is included in MUSIM-CapabilityRestrictionConfig.

References

- 3GPP TS 38.331 V18.2.1. NR; Radio Resource Control (RRC) protocol specification (Release 18).

Appendix – draft CR

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the initiation of the MUSIM UAI after HO/CHO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DualTxRx_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to section 5.3.5.3, UE needs to check whether “the UE is still configured to provide the concerned UE assistance information” in the target NB, in order to determine whether to initiate transmission of a UAI after HO/CHO. For the UAI with MUSIM temproary capability restriciton on band(s) or combination(s) of bands, the UE should consider it is configured to provide such UE assistance information only when the target gNB configures MUSIM-CapabilityRestrictionConfig with musim-CandidateBandList included. However, with the curent text it may be misinterpreted that as long as the target gNB configures MUSIM-CapabilityRestrictionConfig, the UE is configured to provide the UE assistance information with MUSIM temporary capability restriciton, irespective of the contents to be included. If so, it may result in the UE to report an empty UAI message, which is not a desired behavior for both UE and gNB. To avoid such misinterpertation and issue, a Note is needed to clarify the understanding of “UE is still configured to provide the concerned UE assistance information” for the case of UAI with MUSIM temproary capability restriciton on band(s) or combination(s) of bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following NOTE in the section 5.3.5.3. NOTE X: For UE assistance information with MUSIM temporary capability restriction on the band(s) or combination(s) of bands, the UE shall consider it is configured to provide such UE assistance information only if the musim-CandidateBandList is included in MUSIM-CapabilityRestrictionConfig. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: MUSIM Inter-operability: No inter-operability is forseen as no functional change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may lead to that UE initiates the transmission of MUSIM UAI to the gNB without any valid MUSIM UAI included, since UE intends to report proactive UAI again but this reporting is not allowed by the gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

- 5.3.5.3	Reception of an RRCReconfiguration by the UE

…………………………………….. …………..Not relevant part are skipped…………………………………………….

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

4>	start or restart the leave without response timer (if exists) or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

3>	if SIB12 is provided by the target PCell, and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication/discovery related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the UE is capable of NR sidelink communication/discovery and SIB12 is provided by the target PCell, and the UE has initiated transmission of a SidelinkUEInformationNR message since it was configured to do so in accordance with 5.8.3.2:

4>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

4>	if any application layer measurement report container has been received from upper layers for which the successful transmission of the MeasurementReportAppLayer message or at least one segment of the message via SRB4 (if reconfigurationWithSync was included in masterCellGroup) or SRB5 (if reconfigurationWithSync was included in secondaryCellGroup) has not been confirmed by lower layers:

5>	if RRC segmentation was used for the MeasurementReportAppLayer message:

6>	if RRC segmentation is enabled based on the field rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 for the reportingSRB (or SRB4 if reportingSRB is not configured):

7>	re-submit all segments of the MeasurementReportAppLayer message to lower layers for transmission via the reportingSRB (or SRB4 if reportingSRB is not configured);

6>	else:

7>	discard all segments of the MeasurementReportAppLayer message;

5>	else:

6>	re-submit the MeasurementReportAppLayer message to lower layers for transmission via the reportingSRB (or SRB4 if reportingSRB is not configured);

2>	if reconfigurationWithSync was included in masterCellGroup and SRB4 is configured in the target cell:

3>	for each application layer measurement configuration in the UE:

4>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, if transmissionOfSessionStartStop is set to true for the application layer measurement configuration and if the session status has changed since the UE was configured with the conditional reconfiguration:

5>	initiate transmission of a MeasurementReportAppLayer message including appLayerSessionStatus, via SRB4 for the application layer measurement in accordance with 5.7.16.2;

2>	if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21 or provides SIB1 including non-ServingCellMII:

3>	if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message:

4>	initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 4: The UE sets the content of UEAssistanceInformation according to latest configuration (i.e. the configuration after applying the RRCReconfiguration message) and latest UE preference. The UE may include more than the concerned UE assistance information within the UEAssistanceInformation according to 5.7.4.2. Therefore, the content of UEAssistanceInformation message might not be the same as the content of the previous UEAssistanceInformation message.

NOTE X: For UE assistance information with MUSIM temporary capability restriction on the band(s) or combination(s) of bands, the UE shall consider it is configured to provide such UE assistance information only if the musim-CandidateBandList is included in MUSIM-CapabilityRestrictionConfig.