3GPP TSG-RAN WG2 Meeting #127bis	R2-2408030

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.4

Source:	Huawei, HiSilicon

Title:	Discussion on LP-WUS for RRC_CONNECTED mode

Document for:	Discussion and Decision

- Introduction

The following agreements are reached on LP-WUS for RRC_CONNECTED mode in RAN2[1] and RAN1.

RAN2#127 Agreement - For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. RAN1#118 Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

In this contribution, we further discuss higher layer design for CONNECTED mode.

- Discussion

  - Basic functionality for LP-WUS

    - Detailed design for Option 1-2

PDCCH monitoring impacted by LP-WUS

Based on latest RAN1/2 agreements, UE can monitor LP-WUS outside legacy C-DRX active time (i.e. UE is not monitoring PDCCH by MR) and UE ignores legacy C-DRX cycle and legacy drx-onDurationTimer, the LP-WUS is used to trigger the start of a timer (during which UE monitors PDCCH) as soon as possible after a time offset. When this timer is running, NW is able to transmit PDCCH scheduling DL data, which triggers the start of drx-InactivityTimer as in legacy C-DRX operation, since RAN1 agreed that “UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected”, so that NW is able to transmit the following DL data until there is no DL data anymore. Thus, during the period while the timer triggered directly by LP-WUS is running, UE behaviors can be similar as the legacy operations while drx-InactivityTimer is running, since the period is for data reception/transmission, e.g. monitoring PDCCH, performing SRS transmission/CSI reporting and RRM/RLM/BFD measurement related operations to assist data reception/transmission. That means, while the timer triggered directly by LP-WUS is running, it can be regarded as the C-DRX Active time, and the existing UE behavior specified can be reused without big impact on specification. The timer can be a new timer, or reuse the legacy C-DRX timer, e.g. drx-InactivityTimer.

Observation 1: While the timer triggered by LP-WUS is running, UE behaviors are similar as the legacy operations performed during C-DRX Active Time since the period is for data reception/transmission, the existing UE behavior specified can be reused without big impact on specification.

Proposal 1: The period while the timer triggered by LP-WUS is running, can be regarded as the C-DRX Active Time.

If there is no data reception/transmission anymore, the UE does not need to keep monitoring PDCCH, and can switch to LR to save power and wait for the next LP-WUS to trigger PDCCH monitoring if new DL traffic arrives. One possible way is, if the timer triggered directly by LP-WUS and all C-DRX legacy timers impacting legacy C-DRX Active Time expire, UE is not required to monitor PDCCH and can switch to LR. Otherwise, if UE still stays in MR but does not monitor PDCCH, the NW cannot reach to the UE or even send a message to switch UE to LR, or some pre-configured occasions for PDCCH monitoring should be defined, like onDuration, but this introduces additional invalid PDCCH monitoring if there is no traffic ongoing, and introduces latency if traffic arrives earlier than next pre-configured occasion.

Observation 2: If the timer triggered directly by LP-WUS and all C-DRX legacy timers impacting legacy C-DRX Active Time expire, UE can start LP-WUS monitoring. Staying in MR introduces unreachability issue, or additional power consumption and latency.

Proposal 2: If the timer triggered directly by LP-WUS and all C-DRX legacy timers impacting legacy C-DRX Active Time expire, i.e. C-DRX Active Time ends, UE starts LP-WUS monitoring.

In some cases, there is no following data to be received before C-DRX Active Time related timers expire, it wastes power if UE keeps monitoring PDCCH. In legacy C-DRX, (Long) DRX Command MAC CE is introduced to overcome this issue, the NW can end the C-DRX Active Time if it finds no more data reception is needed to reduce power consumption. Similar mechanism can be adopted, e.g. MR L1/L2 signaling is supported to end the C-DRX Active Time and triggers LP-WUS monitoring, e.g. introducing new L1/L2 signaling or reusing (Long) DRX Command MAC CE.

Observation 3: If there is no following data to receive, UE waiting for C-DRX Active Time related timers expires consumes more power, it is beneficial to stop PDCCH monitoring as soon as possible and trigger LP-WUS monitoring.

Proposal 3: MR L1/L2 signaling is supported to terminate the C-DRX Active Time and triggers LP-WUS monitoring.

When UE starts LP-WUS monitoring, if UE applies LP-WUS monitoring ahead of the NW, uncompleted services from the NW may be affected, causing UE’s data loss. On another hand, if the LP-WUS function takes effect on the NW before the UE, UE may not enter LP-WUR mode in time, which reduces the power saving gain, and for the NW there may be more signaling overhead as well. Thus, it is beneficial that the NW/UE can align the start point for monitoring LP-WUS.

Proposal 4: Alignment of the start point for monitoring LP-WUS can be further studied.

The example of procedure of Option 1-2 is illustrated in Figure 1 below.

Figure 1. Illustration for procedure of Option 1-2

SRS transmission and CSI reporting

Considering LP-WUS, if LP-WUS triggers PDCCH monitoring for data transmission/reception, i.e. during C-DRX Active Time, normal SRS transmission and CSI reporting can be performed as legacy operation, which is beneficial for data scheduling.

Observation 4: If LP-WUS triggers PDCCH monitoring for data transmission/reception, i.e. during C-DRX Active Time, normal SRS transmission and CSI reporting can be performed as legacy operation without additional specification impact.

During the time that is not C-DRX Active Time, in existing DRX and Rel-16 DCP operation, the UE behavior for SRS (periodic SRS and semi-persistent SRS) transmission and CSI reporting (periodic CSI that is L1-RSRP or not, and semi-persistent CSI) are additionally defined in order to save power, i.e. “not transmit periodic SRS and semi-persistent SRS, not report CSI on PUCCH and semi-persistent CSI configured on PUSCH”. It can also be applied without additional specification impact, if UE is currently monitoring LP-WUS, i.e. not C-DRX Active Time.

Observation 5: If UE is currently monitoring LP-WUS, i.e. not C-DRX Active Time, legacy operations for periodic/semi-persistent SRS, CSI reporting on PUCCH and semi-persistent CSI reporting configured on PUSCH, are applied without additional specification impact.

Moreover, RAN1 made an agreement on periodic CSI/L1-RSRP reporting:

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

  - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

  - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

For periodic CSI/L1-RSRP reporting, same behaviour as Rel-16 DCP is supported: within drx-onDurationTimer duration, there can be no PDCCH monitoring if UE is monitoring LP-WUS (during which the PDCCH monitoring is not triggered by legacy drx-onDurationTimer), in this case, UE reports periodic CSI on PUCCH that is L1-RSRP/not L1-RSRP based on NW configuration, configuration fields like ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI can be introduced.

Proposal 5: Within drx-onDurationTimer duration, if there is no PDCCH monitoring due to LP-WUS monitoring, UE reports periodic CSI on PUCCH that is L1-RSRP/not L1-RSRP based on NW configuration, configuration fields like ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI can be introduced.

RRM/RLM/BFD measurement

It assumes no impact on RRM/RLM/BFD measurement requirements according to RAN1 agreement. With C-DRX configured, RRM/RLM/BFD measurement requirements are clearly defined in RAN4 specification. Even with introduction of LP-WUS, it is combined UE operation in MR, e.g. PDCCH monitoring if during C-DRX Active Time, LP-WUS monitoring if outside C-DRX Active Time, legacy UE behaviours based on RAN4 requirements for RRM/RLM/BFD measurement are applied without RAN2 impact.

Observation 6: Legacy UE behaviors for RRM/RLM/BFD measurement are applied without RAN2 impact.

    - Option 1-1 vs. Option 1-2

Based on TR 38.869 [2], it provided power and latency evaluation on Option 1-1 and Option 1-2. The results in Appendix show that, e.g. for FTP3 traffic, LP-WUS compared with CDRX or CDRX+DCP, the power saving gain provided by Option 1-1 is about average (16.75%~34%) for different CDRX cycle length. However, the power saving gain provided by Option 1-2 can be average 61.3% for the case MR enters deep-sleep state. For latency, showed by UPT gain, the latency of Option 1-1 is much larger than Option 1-2, since for Option 1-1 UE needs to wait until the next onDuration period before it can monitor PDCCH. For DCP solution, the power saving gain is obtained by sacrificing latency due to duty cycle monitoring for DCP. As the power consumption for LP-WUR is less compared to MR, more frequent monitoring of LP-WUS can be supported and the additional power saving gain can be achieved without introducing additional latency.

Observation 7: Option 1-2 can achieve better power saving gain and reduced latency compared with Option 1-1.

Proposal 6: At least Option 1-2 is supported.

Option 1-1 can be considered as one special case as Option 1-2, if the LP-WUS monitoring occasions are configured according to C-DRX cycle, LP-WUS triggers drx-onDurationTimer for Option 1-1 and triggers a timer control PDCCH monitoring for Option 1-2. drx-InactivityTimer will be further started if there is data scheduling. UE stops MR PDCCH monitoring and switches back to LR if there is no more data. Additionally, Option 1-2 can have a more flexible configuration and better performance. Thus, it is unnecessary to configure Option 1-1 and Option 1-2 simultaneously for the same UE.

Proposal 7: If both options are supported, Option 1-1 and Option 1-2 are not configured simultaneously for the same UE.

  - Other higher layer impacts

    - SPS and CG

LP-WUS indication in CONNECTED mode is to trigger MR PDCCH monitoring, the LP-WUS only impacts MR PDCCH monitoring. For DL configured resources, e.g. DL SPS, as the gNB allocates DL SPS resources despite knowing the LP-WUS is enabled, gNB expects the UE to perform legacy operation on DL SPS resources, and there is no mismatch between UE and gNB on LP-WUR/MR monitoring. From UE perspective, UE should monitor DL SPS resources by MR to avoid missing any DL data.

Proposal 8: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

However, for UL configured resources, e.g. configured grant Type 1 or configured grant Type 2 (UL SPS), sometimes there is a UL configured resource but UE does not have the UL data for transmission. If UL skipping is not enabled, the UE should still transmit padding bits in the UL configured resource which leads to additional power consumption. If the UE is operating with LP-WUR, it is beneficial to skip UL configured resource if no UL data available so the UE does not need to turn on the MR and keeps monitoring LP-WUS.

Proposal 9: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

    - LP-WUS with CA

There were the following agreements in RAN1 on CA scenario.

Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC

Based on latest RAN1 agreement, LP-WUS is supported in CA scenario, PDCCH monitoring on different serving cells need to be further studied. For example, UE only monitors LP-WUS on PCell, or multiple LP-WUS monitoring occasions on PCell and SCell(s) can be defined to control different serving cell groups. Once the LP-WUS is received targeted to the UE, UE starts MR PDCCH monitoring in all activated serving cells, or only on specific serving cell (e.g. PCell) and relies on other signaling to activate PDCCH monitoring in other serving cell. Anyway, whether/how LP-WUS works when UE is configured with CA needs to be further studied with taking RAN1 progress into account.

Proposal 10: RAN2 waits for more RAN1 conclusion for further study on LP-WUS with CA.

- Conclusion

In this paper, we discuss higher layer design for CONNECTED mode, and have the following proposals.

Basic functionality for LP-WUS

Proposal 1: The period while the timer triggered by LP-WUS is running, can be regarded as the C-DRX Active Time.

Proposal 2: If the timer triggered directly by LP-WUS and all C-DRX legacy timers impacting legacy C-DRX Active Time expire, i.e. C-DRX Active Time ends, UE starts LP-WUS monitoring.

Proposal 3: MR L1/L2 signaling is supported to terminate the C-DRX Active Time and triggers LP-WUS monitoring.

Proposal 4: Alignment of the start point for monitoring LP-WUS can be further studied.

Proposal 5: Within drx-onDurationTimer duration, if there is no PDCCH monitoring due to LP-WUS monitoring, UE reports periodic CSI on PUCCH that is L1-RSRP/not L1-RSRP based on NW configuration, configuration fields like ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI can be introduced.

Proposal 6: At least Option 1-2 is supported.

Proposal 7: If both options are supported, Option 1-1 and Option 1-2 are not configured simultaneously for the same UE.

Other higher layer impacts

Proposal 8: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

Proposal 9: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

Proposal 10: RAN2 waits for more RAN1 conclusion for further study on LP-WUS with CA.

- References

R2-2407572, Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

TR 38.869, Study on low-power wake up signal and receiver for NR.