3GPP TSG-RAN WG2 Meeting #127bis	R2-2408032

# Hefei, China, 14 – 18 October 2024

Agenda item:		8.3.3

Source:	Xiaomi

Title:	Measurement event prediction

Document for:		Discussion and Decision

Introduction

In RAN2#125bis meeting, following was agreed on measurement event prediction:

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction are is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

In this contribution, we discuss the use cases/scenarios, simulation assumptions, and performance metrics/KPIs for measurement event prediction.

Discussion

Use cases

In RAN2#125bis meeting, the use case for prediction of measurement event evaluation is considered for study.

Time of stay is one HO performance KPI as from the SID [1]:

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

Measurement event prediction (e.g. Event A3) is basically a HO prediction (without considering factors like load, admission control). If the UE can perform prediction of consecutive measurement events, i.e. predicting UE will handover from serving cell to neighbor cell A (at time x) and then to neighbor cell B (at time y), then the time of stay in neighbor cell A can be estimated e.g. by y – x. Given that time of stay is one important HO performance KPI, it is needed to study the prediction of time of stay based on measurement event prediction.

Proposal 1: The use case of prediction of time of stay based on measurement event prediction is considered for study.

Simulation assumptions

Direct vs. indirect approach

As agreed in RAN2#125bis meeting, measurement event prediction can be either based on RRM measurement prediction (indirect) or direct AI/ML model prediction. During RAN2#127 meeting discussion of simulation approaches for RLF prediction, it was agreed that both direction and indirect are allowed. One main reason to allow direct approach for RLF prediction is that SINR is used for indirect RLF prediction, which is different from RSRP measurement of RRM prediction. However, for measurement event prediction, indirect approach can reuse the results of RRM measurement prediction, which can reduce the simulation workload greatly. In addition, additional information can be obtained from indirect approach, e.g. the predicted measurement results of serving and neighbor cells, which can be provided to the network for informed decision about handover. Such information is not available in direct approach, which requires a separate AI model to predict RRM measurement results and therefore adds additional complexity and power consumption. It is therefore proposed to prioritize indirect approach for measurement event prediction.

Proposal 2: For measurement event prediction, RAN2 prioritizes the indirect approach, i.e. measurement event evaluation based on RRM measurement prediction result.

Output of AI model

In RAN2#127 meeting, following was agreed regarding output for RLF prediction:

-	Output for indirect: predicted SINR.  Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

-	Output for direct: probability of RLF within an window

Similar principle can be applicable for measurement event prediction.

Proposal 3: For indirect prediction of measurement event, based on predicted RSRP (as from RRM measurement prediction), the time instance of fulfillment of measurement event entering condition can be determined without further AI/ML models.

Proposal 4: For direct prediction of measurement event (if kept as allowed simulation option), the output for AI model is the probability of fulfillment of measurement event entering condition.

Case A vs. Case B

Following is captured in TR 38.744 for RRM measurement prediction:

The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance.

Intra-frequency intra-cell temporal domain case A prediction is evaluated for the 2nd study goal by predicting measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell for both FR1 and FR2 scenario without reducing any measurement in observation window. Intra-frequency intra-cell temporal domain case B prediction is evaluated for 1st study goal by predicting a sub set of measurement instances in temporal domain of the same cell for both FR1 and FR2 scenario.

Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a sub set of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell. It is only evaluated for FR2 intra-frequency scenario and is applicable for RRM sub-use case 1 and 3.

In phase 1 discussion of “[POST127][030][AI mobility] RRM simulation assumptions (OPPO)” (R2-2407849), most companies agree the case A case as follows:

In case A, continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding.

Figure 3 Example of temporal domain case A

For indirect prediction of measurement event, discussion is needed on which Case (e.g. A or B) should be considered. Given that Case A is for 2nd study goal (improve the handover performance), and the goal of measurement event prediction is to improve the handover performance as well, it is proposed that RAN2 focus on Case A only for measurement event prediction.

Proposal 5: For indirect prediction of measurement event, only Case A is considered.

Evaluation scenario

In TR 38.744, following evaluation scenarios are captured for RRM prediction:

Table 5.2.1-1 prioritization of evaluation scenarios

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Given that event prediction is for 2nd goal, and mobility in FR2 is more challenging than that in FR1, it is proposed to focus on evaluation scenario 4, which is high priority in RRM prediction. Intra-cell temporal RRM prediction is assumed for measurement event evaluation.

Proposal 6: For indirect prediction of measurement event, evaluation scenario 4 (FR2 to FR2 intra-frequency intra-cell temporal domain case A) of RRM prediction is of high priority.

In RAN2#127 meeting, regarding RLF prediction, it was agreed that “For the time being we don’t need HO procedure simulation in RLF simulation.” Discussion is needed on whether HO procedure should be modelled in measurement event prediction. In simulation, due to the mobility, UE eventually moves out of the coverage of its serving cell, and the collected data after UE moving out of coverage is not useful. There are basically two approaches for simulation:

- Option A: HO procedure is not modelled. It is up to implementation of simulator to restart simulation e.g. when entering condition of event A3 is fulfilled based on measurements.

- Option B: “Simple” HO procedure is modelled, e.g. serving cell is changed after entering condition of event A3 is fulfilled based on measurements. This is similar to LTE HetNet mobility simulation, e.g. TR 36.839 Table 5.2.4.1 specifies that handover preparation delay and handover execution time are 50 ms and 40 ms, respectively.

Option B would require more discussion on simulation parameters (e.g. detailed simulation methodology, A3 event parameters used for HO decision, HO delay). It is simpler to go with Option A and let companies to determine how to avoid collecting wrong data (e.g. results when serving cell has very low RSRP).

Proposal 7: For event prediction, at current stage, there is no need to model HO procedure. Companies should ensure that only reasonable simulation data is collected with respect to UE mobility and coverage.

In RAN2#127 meeting, regarding RLF prediction, it was agreed that “FFS full buffer and assumption that all the cells are fully loaded. We will not simulate traffic.” RLF prediction uses SINR, which is related to the interference generated by neighbor cells and is related to traffic model. For measurement event prediction, the measurement is mainly RSRP, therefore is not related to traffic model.

Proposal 8: RAN2 to confirm that for measurement event prediction, traffic is not simulated.

Performance metrics for evaluation

For performance metrics of RRM measurement prediction, following was agreed in RAN2#125bis meeting: “As a first step we will focus on measurement prediction accuracy.  FFS whether and what system level performance evaluation is needed”. It is proposed to adopt same approach for measurement event prediction.

Proposal 9: For measurement event prediction, as a first step, RAN2 focus on measurement prediction accuracy. FFS whether and what system level performance evaluation metric is needed.

Similar to simulation assumptions discussed above, performance metrics for RRM measurement prediction / RLF prediction can be references for discussion on measurement event prediction.

In RAN2#127 meeting, following was agreed regarding performance metrics for RLF simulation:

Agreements on KPI

For indirect

-	SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction.

Direct

-	missed RLF detection, false RLF detection, F1 score.

-	Whether the RLF will happen based on the following methodology.   Above a probability threshold we assume that RLF will happen.   This is then compared with true RLF.

Note that the metric RSRP difference is already captured in TR 38.744, as below, therefore is not further considered in measurement event prediction:

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

Proposal 10: For indirect prediction of measurement event, performance metrics include: missed measurement event detection, false measurement event detection, F1 score, time difference of true time of measurement event fulfillment and predicted measurement event fulfillment, true measurement event prediction.

Proposal 11: For direct prediction of measurement event (if kept as allowed simulation option), performance metrics include: missed measurement event detection, false measurement event detection, F1 score.

Conclusion

In this contribution, we discuss the use cases/scenarios, simulation assumptions, and performance metrics/KPIs for measurement event prediction, and propose the following:

Proposal 1: The use case of prediction of time of stay based on measurement event prediction is considered for study.

Proposal 2: For measurement event prediction, RAN2 prioritizes the indirect approach, i.e. measurement event evaluation based on RRM measurement prediction result.

Proposal 3: For indirect prediction of measurement event, based on predicted RSRP (as from RRM measurement prediction), the time instance of fulfillment of measurement event entering condition can be determined without further AI/ML models.

Proposal 4: For direct prediction of measurement event (if kept as allowed simulation option), the output for AI model is the probability of fulfillment of measurement event entering condition.

Proposal 5: For indirect prediction of measurement event, only Case A is considered.

Proposal 6: For indirect prediction of measurement event, evaluation scenario 4 (FR2 to FR2 intra-frequency intra-cell temporal domain case A) of RRM prediction is of high priority.

Proposal 7: For event prediction, at current stage, there is no need to model HO procedure. Companies should ensure that only reasonable simulation data is collected with respect to UE mobility and coverage.

Proposal 8: RAN2 to confirm that for measurement event prediction, traffic is not simulated.

Proposal 9: For measurement event prediction, as a first step, RAN2 focus on measurement prediction accuracy. FFS whether and what system level performance evaluation metric is needed.

Proposal 10: For indirect prediction of measurement event, performance metrics include: missed measurement event detection, false measurement event detection, F1 score, time difference of true time of measurement event fulfillment and predicted measurement event fulfillment, true measurement event prediction.

Proposal 11: For direct prediction of measurement event (if kept as allowed simulation option), performance metrics include: missed measurement event detection, false measurement event detection, F1 score.

- References

[1] RP-240082, OPPO (rapporteur), "Revised SID on AIML for mobility in NR"

[2] TR 38.744, "Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR"