3GPP TSG-RAN WG2 Meeting #127bis	R2-2408033

# Hefei, China, 14 – 18 October 2024

Agenda item:		8.7.5

Source:	Xiaomi

Title:	RLC AM retransmission enhancements

Document for:		Discussion and Decision

Introduction

In RAN2#127 meeting, following was agreed regarding RLC retransmission enhancements [1]:

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In this contribution, we discus RLC AM retransmission enhancements.

Discussion

Timely RLC retransmission

There are mainly three solution directions for timely RLC retransmission:

- Autonomous RLC retransmission without waiting for status report

- Enhancements to RLC status reporting

- Enhancements to polling

Autonomous RLC retransmission

In RAN2#126 meeting, it was agreed that “Impact on capacity should be considered”. Before discussing any enhancements, it is necessary to investigate whether timely retransmission for UL traffic can be handled by implementation / existing mechanisms. First of all, in Rel-18, UL traffic information (including traffic periodicity, burst arrival time, jitter etc.) is introduced as UAI. With such assistance information, gNB can set up configured grant accordingly. If HARQ retransmission for a CG resource is needed, gNB is aware of the associated traffic characteristics based on CG resource.

Observation 1: gNB is aware of the traffic characteristics (including delay requirements and status) of typical XR traffic.

For UL transmission, one needs to understand what is the reason for RLC ARQ retransmissions. Except for CG-DFI used in NG-U, there is no ACK/NACK transmitted in DL for UL data transmissions. Therefore there is no potential NACK to ACK error. The potential reason for RLC retransmission is that some UL transmissions is severely interfered, resulting in very low SINR. In such case, any additional HARQ retransmission might not boost the SINR to an acceptable level for channel decoding. However, upon giving up on further HARQ retransmissions, gNB can flush HARQ receive soft buffer, and ask UE to perform retransmission. From gNB’s perspective, HARQ buffer is flushed and the reception of the MAC PDU is like starting from an initial state, which is like a RLC ARQ retransmission. This can be done by gNB implementation, e.g. gNB can take into account PDB etc. From UE’s perspective, it is still performing HARQ retransmission, and there is no specification impact and UE behavior is unchanged. This is shown in Figure 1 below.

Figure 1: gNB implementation to initiate a retransmission

Observation 2: gNB implementation can initiate an HARQ retransmission which is similar as RLC retransmission.

Alternatively, gNB can send RLC status report as fast as possible to ask UE to initiate RLC retransmission. Although it might be slower compared with PDCCH based approach described above (e.g. due to MAC / RLC layer processing), the latency would be similar to autonomous RLC retransmission

Observation 3: gNB implementation can send RLC status report to trigger RLC retransmission.

From above discussion, it can be seen that for UL traffic, gNB implementation can ensure timely RLC retransmission without specification impacts. Since gNB knows exactly whether it has received MAC PDUs correctly or not, there is no impact on compacity. As comparison, autonomous RLC retransmission has the potential impact on capacity as follows. RLC AM achieves high reliability as well as high spectrum efficiency based on status report. In existing RLC AM, UE only retransmits the RLC SDUs which are explicitly NACKed in status report. For autonomous RLC retransmission, it is possible that the retransmitted RLC SDUs have already been successfully received by gNB before autonomous retransmission, therefore such autonomous retransmission wastes radio resource. Considering that XR traffic can be of large data rate, such autonomous retransmission might impact overall system capacity.

In summary, for UL traffic, gNB implementation can ensure timely RLC retransmission, and autonomous RLC retransmission has impact on capacity. Therefore it is proposed to deprioritize the solution direction of autonomous RLC retransmission.

Proposal 1: RAN2 to deprioritize the solution direction of autonomous RLC retransmission.

Enhancements to RLC status reporting

According to RAN2#126 meeting agreements, three directions will be considered for timely retransmissions on RLC layer: autonomous retransmission, enhanced status report, enhanced polling. There is additional agreement “RAN2 focuses on the enhancements for UL traffic”, which needs clarification. Our understanding is that the “focus on UL traffic” is not for solution direction enhanced status report, since handling of status report for UL traffic can be left to network implementation.

Proposal 2: For retransmission based on enhanced status report, RAN2 focuses on the enhancements on the DL traffic.

Current framework of RLC status report and RLC retransmission is show in Figure 2 below.

Figure 2: RLC status report and RLC retransmission

Triggers for RLC status report is either from poll or by expiry of t-Reassembly, as from TS 38.322 clause 5.3.4, copied below:

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

Potential enhancements for RLC status report can be the introduction of new triggers, e.g. status report based on bytes / number of PDUs received, as well as periodical status report. Another aspect is that currently after receiving polling from gNB, UE delays triggering of status report until certain condition is satisfied (SN x < RX_Highest_Status or x >= RX_Next + AM_Window_Size). Considering the short PDB of XR traffic, it might be helpful to also consider to allow UE to send status report immediately after receiving polling.

Proposal 3: RAN2 to analyze solution direction of new triggers for RLC status report as well as fast status report after receiving polling.

Enhancements to polling

Regarding polling enhancements, it should be noted that the purpose of polling is to inform the peer to transmit status report. In case of polling sent by gNB, polling enhancement can be handled by gNB implementation. In case of polling sent by the UE, the goal is to get status report from gNB in a fast manner. However, gNB should be aware of the PDB requirements of XR traffic, and can send RLC status report in time by implementation. Therefore, evaluation is needed on the benefits of polling enhancements over gNB implementation on the timely handling of polling and RLC status report.

Proposal 4: Justification is needed for polling enhancement since gNB implementation can handle timely polling and RLC status report taking into account the PDB requirements of XR traffic.

Avoid unnecessary RLC retransmissions

Below we analyze the two approaches.

Tx initiated approach

In Rel-18 XR, PDCP SN gap reporting is introduced. Transmitter uses PDCP control PDU to inform the receiver the PDCP COUNT of the discarded PDCP SDUs so that receiver can update the state variables (e.g. RX_DELIV) accordingly. One restriction in Rel-18 XR PDCP SN gap reporting is that if RLC entity has already submitted the RLC PDUs to MAC layer, the corresponding RLC PDUs cannot be discarded.

For Rel-19, there are proposals to remove above restriction. Basically, the transmitting side of RLC entity stops transmitting outdated packets (which have already been submitted to MAC layer) and informs the receiving side of peer RLC entity, which updates its state variables (e.g. RX_Next) accordingly.

Rx initiated approach

In Rx initiated approach, the receiving side of RLC entity updates its state variables (e.g. RX_Next) and informs the peer RLC entity with RLC status report so that transmitting side of peer RLC entity stops transmitting outdated packets.

Combined Tx and Rx approach

In this approach, Rx side operation is same as Rx initiate approach. Tx side operation is that Tx side stops to retransmit obsolete SDUs based on the discard indication. There is no control PDU to notify about obsolete SDUs sent from Tx side to Rx side.

Comparison

Both Tx initiated and Rx initiated approaches can avoid unnecessary RLC retransmissions. From specification impact point of view, Tx initiated approach requires more changes as from the experience of Rel-18 PDCP SN gap reporting. In Tx initiated approach, both the Tx side operation (triggering, control PDU format) and Rx side operation need to be specified. Rx initiated approach, on the other hand, mainly requires the change on Rx side operation while there is almost no change to Tx side operation, which can be based on existing behavior on handling RLC status report. The combined Tx and Rx approach is slightly more complicated than Rx initiated approach by requiring that Tx side stops to retransmit obsolete SDUs based on the discard indication.

Table 1: Comparison of Tx initiated and Rx initiated approaches

|  | Tx initiated approach | Rx initiated approach | Combined Tx and Rx approach |
| --- | --- | --- | --- |
| Tx operation | - Tx side stops to retransmit obsolete SDUs based on the discard indication - Format of control PDU to notify about obsolete SDUs - Triggering to send the control PDU | - As in legacy, processes status report | - Tx side stops to retransmit obsolete SDUs based on the discard indication - As in legacy, processes status report |
| Rx operation | - Update of state variables / timers based on the reception of control PDU | - Define triggering condition to update state variables / timers and send status report | - Define triggering condition to update state variables / timers and send status report |

It can be seen that Rx initiated approach is simpler compared with Tx initiated approach since only Rx operation needs to be specified in Rx initiate approach. If Rx initiated approach cannot be agreed, combined Tx and Rx approach can be a compromise to consider.

Proposal 5: RAN2 to specify Rx initiated approach to avoid unnecessary RLC retransmissions. If this cannot be agreed, combined Tx and Rx approach can be a compromise to consider.

Conclusion

In this contribution, we discuss RLC AM retransmission enhancements, and have the following observations:

Observation 1: gNB is aware of the traffic characteristics (including delay requirements and status) of typical XR traffic.

Observation 2: gNB implementation can initiate an HARQ retransmission which is similar as RLC retransmission.

Observation 3: gNB implementation can send RLC status report to trigger RLC retransmission.

We propose the following:

Proposal 1: RAN2 to deprioritize the solution direction of autonomous RLC retransmission.

Proposal 2: For retransmission based on enhanced status report, RAN2 focuses on the enhancements on the DL traffic.

Proposal 3: RAN2 to analyze solution direction of new triggers for RLC status report as well as fast status report after receiving polling.

Proposal 4: Justification is needed for polling enhancement since gNB implementation can handle timely polling and RLC status report taking into account the PDB requirements of XR traffic.

Proposal 5: RAN2 to specify Rx initiated approach to avoid unnecessary RLC retransmissions. If this cannot be agreed, combined Tx and Rx approach can be a compromise to consider.

- References

[1] R2-2407575, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”