3GPP TSG-RAN WG2 Meeting #127bis	R2-2408035

# Hefei, China, 14 – 18 October 2024

Agenda item:		8.11.3

Source:	Xiaomi

Title:	Other aspects of SBFD

Document for:		Discussion and Decision

Introduction

In RAN2#127 meeting, following was agreed regarding SBFD resource indication [1]:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In this contribution, we discuss RAN2 aspects of CLI handling and interference issue.

Discussion

CLI handling

UL resource muting for PUSCH

In RAN1#117 meeting, following was agreed regarding UL resource muting for PUSCH:

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - Note: No new DCI field / MAC CE is introduced - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: This feature is subject to UE capability

Based on above agreement, it can be seen that RAN2 needs to define: 1) RRC signalling to indicate UL resource muting for PUSCH; 2) UE capability for UL resource muting for PUSCH.

Proposal 1: RRC signalling is introduced to indicate UL resource muting for PUSCH.

Proposal 2: UE capability is introduced for the support of UL resource muting for PUSCH.

Since this is a RAN1 feature, it is expected that RAN1 will send RAN2 LS for detailed RRC parameter as well as UE feature list. Therefore RAN2 can wait for RAN1 progress before discussing details for RRC signalling and UE capability.

Proposal 3: RAN2 postpones the discussion on details for RRC signalling and UE capability for UL resource muting for PUSCH until receiving related RRC parameter list and UE feature list from RAN1.

L1 based UE-to-UE CLI measurement and reporting

In RAN1#117 meeting, following was agreed regarding L1 based UE-to-UE CLI measurement and reporting:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource - Measurement reporting - At least aperiodic reporting - Note: Periodic and semi-persistent reporting can be considered - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - Note: At least wideband reporting is supported - UCI bits generation - Priority rules for multiple CSI reporting

As the feature is L1 based measurement and reporting, the main RAN2 impacts are related to measurement resource configuration and potentially UE capability reporting. It is expected that RAN1 will send RAN2 LS for detailed RRC parameter as well as UE feature list. Therefore RAN2 can wait for RAN1 progress before discussing details for RRC signalling and UE capability.

Proposal 4: RAN2 postpones the discussion on details for RRC signalling for measurement resource configuration and UE capability for L1 based UE-to-UE CLI measurement and reporting until receiving related RRC parameter list and UE feature list from RAN1.

Interference issue

In Rel-19 SBFD, gNB can receive in UL and transmit in DL simultaneously. An example is shown in Figure 1 below. In Case 1, the DL reception may cause interference to the PUSCH reception in network side. In Case 2, neighbor cell’s DL transmission may also cause interference to the PUSCH reception in network side, i.e., CLI. Although the interference issue above could be mitigate to certain extent by implementation as well as CLI handling techniques, the PUSCH transmitted in SBFD symbols still suffers more interference than PUSCH transmitted in non-SBFD symbols. RAN2 needs to discuss whether such PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Figure 1: Interference issue of PUSCH in SBFD symbols

Proposal 5: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Conclusion

In this contribution, we discuss RAN2 aspects of CLI handling and interference issue, and propose the following:

Proposal 1: RRC signalling is introduced to indicate UL resource muting for PUSCH.

Proposal 2: UE capability is introduced for the support of UL resource muting for PUSCH.

Proposal 3: RAN2 postpones the discussion on details for RRC signalling and UE capability for UL resource muting for PUSCH until receiving related RRC parameter list and UE feature list from RAN1.

Proposal 4: RAN2 postpones the discussion on details for RRC signalling for measurement resource configuration and UE capability for L1 based UE-to-UE CLI measurement and reporting until receiving related RRC parameter list and UE feature list from RAN1.

Proposal 5: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

- References

[1] R2-2407572, RAN2 Vice Chairman (CATT), "Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD"