3GPP TSG-RAN WG2 Meeting #127bis	           R2-2408038

Hefei, China, Oct 14th – Oct 18th, 2024

Agenda item:	8.1.2.3

Source:	China Telecom

Title:	Discussion on LCM for positioning use case

Document for:	Discussion

- Introduction

According to the revised WID [1], the objective of positioning accuracy enhancement use case is:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

As guided by the agenda document, we will focus on UE-sided model for positioning accuracy enhancements cases in this paper, i.e. Case 1 and Case 2a.

- Discussion

The agreements related to LCM for UE-sided model for positioning cases in previous RAN2 meetings are as the following:

- RAN2#125bis Agreements for positioning and beam management 1 Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message. 2 Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality 3 FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed) - RAN2#126 1 The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. 2 wait for RAN1 for associate ID discussion 3 At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

RAN2 sent an LS [5] on applicable functionality reporting for UE-sided model for beam management case to inform RAN1 of RAN2’s agreements and questions for RAN1 to confirm. We try to describe the LCM procedure for positioning Case 1 similarly based on the following understandings on terminologies agreed during RAN2#127 and previous RAN2 meeting agreements related to UE-sided model for positioning cases:

Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference

- Supported functionalities reporting

Step 1: LMF sends RequestCapabilities message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends ProvideCapabilities message to LMF, containing supported functionalities at the UE side.

The existing LPP messages in Step 1 and Step 2 (i.e. Capability Transfer procedure) are reused for reactive reporting of supported functionalities for positioning use cases. The existing LPP message in Step 2 (i.e. Capability Indication procedure) is reused for proactive reporting of supported functionalities for positioning use cases.

- Applicable functionality determination

Step 3: LMF can configure the functionalities expected for applicable functionality reporting, and network side additional conditions to the UE via existing LPP message ProvideAssistanceData. The configured functionalities expected for applicable functionality reporting may be a subset of all the functionalities supported by the UE. The UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions internally known by UE and model availability in device.

The details of the signaling design depend on the understanding/category of functionalities for positioning cases. And the understanding/category of functionalities for positioning cases (such as Case 1, Case 2a, or other dimensionality) need RAN1’s input. Regarding the content of NW-side additional conditions, such as whether the associated ID can be assumed to represent NW-side additional conditions for positioning cases, RAN1 has no agreements for now. We need wait for RAN1’s progress.

Proposal 1: For UE-sided model for positioning cases, RAN2 assumes that UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions internally known by UE and model availability in device.

Proposal 2: For UE-sided model for positioning cases, LMF can configure the functionalities expected for applicable functionality reporting, and network side additional conditions to the UE via existing LPP message ProvideAssistanceData. The details of configuration need RAN1’s input.

- Applicable functionality reporting

Step 4: After the UE determines applicable functionalities, the UE report the applicable functionalities via LPP message. According to RAN2#125bis agreements, both proactive reporting and reactive reporting are supported.

Similar to BM, the UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality.

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3.

Since the existing LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) support dynamic capabilities, the procedure can meet the requirement of dynamic/on-demand reporting applicable functionalities. However, from our point of view, mapping of supported functionality and applicable functionality to static capability and dynamic capabilities respectively, and report them using the same capability procedure is a bit strange, i.e. the logic is not that unclear. On the other hand, the LPP Assistance Data Transfer/Delivery procedure is for LMF send assistant data to the UE. But the direction is inverse. Thus, maybe a new LPP message could be introduced for the UE reporting applicable functionalities to LMF.

Proposal 3: For UE-sided model for positioning cases, RAN2 consider the following options for applicable functionality reporting signaling:

- Option1: existing LPP message ProvideCapabilities

- Option2: a new LPP message

- Inference configuration

Step 5: LMF sends inference configuration to UE after applicable functionality reporting via existing LPP message RequestLocationInformation. The UE can send LPP message ProvideLocationInformation to LMF for providing the results of the UE sided model inference operation.

- Functionality activation/deactivation

In our understanding, upon receiving inference configuration in Step 5 (i.e. via ProvideLocationInformation message), the UE activates the functionality. We think LMF can deactivate functionality if needed, with or without assistant information from the UE.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: For UE-sided model for positioning cases, RAN2 assumes that UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions internally known by UE and model availability in device.

Proposal 2: For UE-sided model for positioning cases, LMF can configure the functionalities expected for applicable functionality reporting, and network side additional conditions to the UE via existing LPP message ProvideAssistanceData. The details of configuration need RAN1’s input.

Proposal 3: For UE-sided model for positioning cases, RAN2 consider the following options for applicable functionality reporting signaling:

- Option1: existing LPP message ProvideCapabilities

- Option2: a new LPP message

4	References

- [1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting#105, Qualcomm.

- [2] RP-240499, Status report for WI on AI/ML for NR air interface, 3GPP TSG RAN Meeting#103, Qualcomm, CAICT, Ericsson.

- [3] RP-241153, Status report for WI on AI/ML for NR air interface, 3GPP TSG RAN Meeting#104, Qualcomm, CAICT, Ericsson.

- [4] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model, TSG RAN WG2.