# 3GPP TSG RAN WG2 Meeting #128      	                        R2-2408040
Hefei, China, 14th – 18rd Oct, 2024

Agenda item:	8.1.5

Source:	China Telecom

Title:	Discussion on model transfer/delivery

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

The Rel-19 WID of AI/ML for air interface (WID RP-234039) was agreed in RAN#102 [1]. In this contribution, we will discuss the following objectives.

Study objectives: - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

# 2 Discussion

## 2.1 Model transfer/delivery cases

Based on the report of RAN1#118 meeting [4], the following conclusion on model transfer/delivery was made.

Conclusion [RAN1#118] From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported).

Observation 1: From RAN1 perspective, model transfer/delivery is needed.

In TR 38.843 Table 4.3-1 [5], the following options for model delivery/transfer are identified.

Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization..

Observation 2: The options for model delivery/transfer could be classified into two class:

- The model is trained at NW-side. This class includes Case y, z2, z4, and z5.

- The model is trained at UE-side. This class includes Case y, z1 and z3.

Based on the report of RAN1#116 meeting [2] and the report of RAN1#116bis meeting [3], the following conclusions on model transfer/delivery were made.

Conclusion [RAN1#116] From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. Conclusion [RAN1#116bis] From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network

Observation 3: From RAN1 perspective, the overall prioritization of model transfer/delivery cases up to RAN1#118 is of the following.

| Cases | UE-sided model | Two-sided model |
| --- | --- | --- |
| Case y |  |  |
| Case z1 |  |  |
| Case z2 | Deprioritized (RAN1#116bis) |  |
| Case z3 | Deprioritized (RAN1#116bis) | Deprioritized (RAN1#116bis) |
| Case z4 |  |  |
| Case z5 | Deprioritized (RAN1#116) | Deprioritized (RAN1#116) |

Therefore, we have following observations:

Observation 4: For model trained at NW side, Case y and Case z4 is left for further study.

Observation 5: For model trained at UE side, Case y and Case z1 is left for further study.

In TR 38.843 [5], the following observation for model transfer/delivery Case y is provided, which indicates that the Case y is not proper for the option with model trained at NW-side.

- For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model.

Additionally, based on RAN1#118 meeting, the following conclusions on model transfer/delivery Case z4 were made.

Conclusion [RAN1#118] From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information

Observation 6: For model trained at NW-side, RAN1 continue the study for model transfer/delivery based on Case z4.

Observation 7: For model trained at UE-side, no more conclusions on the Case y and Case z1 were made by RAN1.

Based on the Observations above, we have following proposals:

Proposal 1: For the model transfer/delivery cases, RAN2 should first focus on the Case z4.

## 2.2 Model transfer/delivery initiation

In TR 38.843 [5], solutions for supporting AI/ML model transfer/delivery are provided. As for how to initiate a model transfer/delivery, a reactive and a proactive approach can be considered. For the reactive approach, an AI/ML model is transferred/delivered to the UE when needed. While for the proactive approach, an AI/ML model is pre-download to the UE.

For the reactive approach, the pros and cons are listed as bellow:

- Pros

  - UEs obtain the AI/ML model on demand, saving the training, transferring and storage resources.

- Cons

  - Long time is needed to switch the AI/ML model when scenarios, configurations, sites, etc change.

For the proactive approach, the pros and cons are listed as bellow:

- Pros

  - UEs pre-download the AI/ML model, switching the AI/ML model quickly when scenarios, configurations, sites, etc change.

- Cons

  - Pre-downloading several AI/ML models may waste the training, transferring and storage resources, because that not all pre-downloaded AI/ML models will be used.

Based on the pros/cons analysis above, in our opinion, the reactive approach is proper for the cases which do not have low latency requirement, and the proactive approach is proper for the cases which have low latency requirement.

Therefore, we have following proposal:

Proposal 2: Support both the reactive and proactive approach for AI/ML model transfer/delivery.

# 3 Conclusion

In this contribution, we further discuss AI/ML model transfer/delivery. We have following observations:

Observation 1: From RAN1 perspective, model transfer/delivery is needed.

Observation 2: The options for model delivery/transfer could be classified into two class:

- The model is trained at NW-side. This class includes Case y, z2, z4, and z5.

- The model is trained at UE-side. This class includes Case y, z1 and z3.

Observation 3: From RAN1 perspective, the overall prioritization of model transfer/delivery cases up to RAN1#118 is of the following.

| Cases | UE-sided model | Two-sided model |
| --- | --- | --- |
| Case y |  |  |
| Case z1 |  |  |
| Case z2 | Deprioritized (RAN1#116bis) |  |
| Case z3 | Deprioritized (RAN1#116bis) | Deprioritized (RAN1#116bis) |
| Case z4 |  |  |
| Case z5 | Deprioritized (RAN1#116) | Deprioritized (RAN1#116) |

Observation 4: For model trained at NW side, Case y and Case z4 is left for further study.

Observation 5: For model trained at UE side, Case y and Case z1 is left for further study.

Observation 6: For model trained at NW-side, RAN1 continue the study for model transfer/delivery based on Case z4.

Observation 7: For model trained at UE-side, no more conclusions on the Case y and Case z1 were made by RAN1.

Based on observations, we have following proposals:

Proposal 1: For the model transfer/delivery cases, RAN2 should first focus on the Case z4.

Proposal 2: Support both the reactive and proactive approach for AI/ML model transfer/delivery.

# 4 References

[1] RP-234039, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN1#116, Final Report of 3GPP TSG RAN WG1 #116 v1.0.0.

[3] RAN1#116b, Final Report of 3GPP TSG RAN WG1 #116b v1.0.0.

[4] RAN1#118, Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface).

[5] TR 38.843 v2.0.0, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, Qualcomm