3GPP TSG-RAN WG2 Meeting #127bis                            R2-2408041

Hefei, China, Oct 14th – 18th, 2024

Source:	China Telecom

Title:	Discussion on A-IoT random access

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

In RAN2#127 meeting, random access procedure for Ambient IoT has been discussed and following agreements were reached [1]:

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

Moreover, we have discussed some FFSs about the random access by post email discussion. In this contribution, we intend to discuss remaining issues including failure/success indication aspects and other related problems.

- Random access Procedures

Considering the solutions for handling random access failure of A-IoT, there may be re-access or re-transmit methods. Thus, we need to make the relevant concept explicit first. RAN2 has agreed that slotted-ALOHA is the baseline for A-IoT random access. Therefore, we should define access occasion similar to the concept of ‘slot’. Access occasion means the opportunity for one device to finish access. When there are multiple devices to complete random access, an access round can be used for different devices assigned different access occasions. It means that one access round can include multiple access occasions, and the number of access occasions in one round depends on the reader’s indication. The device can randomly select one access occasion in an access round. Moreover, we can also define the paging round as the post email discussion. One paging round consists of one or multiple access rounds. The number of access rounds after an A-IoT paging message can be indicated by this paging message.

Proposal 1: RAN2 to introduce the defination of "access round". One access round consists of multiple access occasions.

Proposal 2: The device can randomly select one access occasion in one access round for A-IoT random access.

Proposal 3: RAN2 to introduce the defination of "paging round". One paging round consists of one or multiple access rounds.

RAN2 has discussed the procedure of 3-step CBRA, 2-step CBRA or CFRA. In legacy NR systems, UE can know which random access methods are supported by the network via SIB, and decide which random access method to use based on conditions such as measured RSRP. RAN2 is currently investigating both 3-step CBRA and 2-step CBRA. Since there are no legacy SIB messages in A-IoT scenarios and the A-IoT device does not support measurements, etc., RAN2 needs to discuss whether both methods will be supported in the future and how to support them if both are supported.

One option is to explicitly indicate the RA type in the paging message so that A-IoT devices do not need to decide which RA to use which can reduce the processing capability requirement for A-IoT devices.

Another option is to implicitly indicate the RA type in the paging message. For example, if the reader indicates resource information in a paging message, the resource information can be associated with a specific RA type, just like the preamble in group A and group B in legacy NR. And if the A-IoT device selects a set of resources, it means the A-IoT device would use the corresponding RA type.

Proposal 4: RAN2 to discuss how to determine 3-step CBRA/2-step CBRA/CFRA, the following options can be considered:

Option 1: Explicitly indicate RA type in Msg0.

Option 2: Implicitly indicate RA type in Msg0, e.g. transmission resources are associated with different access methods.

For the transmission resource configuration, Msg1’s transmission resource information can be indicated by a paging message or a separate R2D trigger message after paging. If an A-IoT device needs to respond with some data (as responding to a read command), it is meaningful for the device to report some assistance information including the size of the data volume to enable the reader to accurately allocate transmission resources. As for the transmission resource of Msg3, the reader can indicate accurate resource information in Msg2 based on the assistance information in Msg1. In addition, the resource information of Msg3 can also indicated by paging messages. Resource details wait for RAN1’s progress.

Proposal 5: A Paging message or a separate R2D trigger message can indicate transmission resource information for Msg1 and Msg3.

Proposal 6: Msg1 may include assistance information (e.g. the size of data) for the reader to allocate resources to Msg3.

For 3-step CBRA, if the device receives the Msg2 with the same random ID, it indicates Msg1 transmission success. Then, when the reader does not receive Msg1 or the Msg2 transmission/reception fails, the device may receive an R2D message indicating the next access occasion for other devices after transmitting the Msg1, which indicates RA failure on the previous occasion.

Proposal 7: For 3-step CBRA, from the device side, if it receives the next access occasion message instead of the access response Msg2, it indicates RA failure.

Based on the discussion from the last meeting, whether implicit or explicit failure/success indication of D2R (Msg3) or not is still FFS. In our opinion, the reader can detect the failure when D2R data transmission fails, but the device may be not able to detect its D2R data transmission failure. Therefore, the reader needs to send a failure indication message to the device if it does not receive A-IoT Msg3. Regarding the success indication of D2R, the subsequent data transmission can be regarded as an implicit success indication. Thus, the failure indication is the most urgent need. The specific indication is FFS.

Proposal 8: An explicit failure indication of D2R is needed for 3-step CBRA.

As for the device behaviour after the consequence of failure detection, the rapporteur has proposed several options for us to discuss, including re-transmitting the D2R data, re-access in another opportunity, no particular action and following the reader’s paging/triggering message. We think that options 1/2/4 are efficient in handling the D2R data transmission failure, and option 2 has a wider range of applications and can be discussed as a baseline. How to achieve re-access (i.e. select another occasion in this round to re-access or re-access in the next round) should be further discussed. In our understanding, if collision or response loss happens resulting in access failure, these failed devices can re-access in the next access round according to the indication by the reader. In addition, the triggering method for re-access can be initiated by the reader or CN. From RAN2 perspective, at least it should support reader-initiated method for triggering re-access. Whether to perform CN-initiated re-access is up to SA2.

Proposal 9: RAN2 to support A-IoT devices perform re-access in the next access round triggered by the reader when access failure occurs.

- Conclusion

In this contribution, we discuss the random access procedures for Ambient IoT, and have the following proposals:

Proposal 1: RAN2 to introduce the defination of "access round". One access round consists of multiple access occasions.

Proposal 2: The device can randomly select one access occasion in one access round for A-IoT random access.

Proposal 3: RAN2 to introduce the defination of "paging round". One paging round consists of one or multiple access rounds.

Proposal 4: RAN2 to discuss how to determine 3-step CBRA/2-step CBRA/CFRA, the following options can be considered:

Option 1: Explicitly indicate RA type in Msg0.

Option 2: Implicitly indicate RA type in Msg0, e.g. transmission resources are associated with different access methods.

Proposal 5: A Paging message or a separate R2D trigger message can indicate transmission resource information for Msg1 and Msg3.

Proposal 6: Msg1 may include assistance information (e.g. the size of data) for the reader to allocate resources to Msg3.

Proposal 7: For 3-step CBRA, from the device side, if it receives the next access occasion message instead of the access response Msg2, it indicates RA failure.

Proposal 8: An explicit failure indication of D2R is needed for 3-step CBRA.

Proposal 9: RAN2 to support A-IoT devices perform re-access in the next access round triggered by the reader when access failure occurs.

# References

- 3GPP TSG-RAN WG2 Meeting #127 chairman notes.