3GPP TSG-RAN WG2 Meeting #127bis	R2-2408042

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.5

Source: 	China Telecom

Title:	Discussion on Topology 2 related aspects for Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, Topology 2 related aspects for Ambient IoT were discussed and the following agreements were reached.

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In this contribution, we would like to share our opinions on the remaining issues on Topology 2 related aspects for Ambient IoT.

- Discussion

Radio resource management

In last RAN2 meeting, it was agreed that NW is in control of resources to be used by AIoT air interface for Topology 2. The details remain FFS. There might be the following potential options for radio resource management in Topology 2.

- Option 1: the gNB provides dedicated radio resources for A-IoT air interface to the UE reader by dedicated RRC signalling.

- Option 2: the UE reader randomly selects radio resources for A-IoT air interface in a pre-configured shared resource pool.

As per the WID, the UE reader shall be controlled by the NW. For the air interface, the related radio configuration for A-IoT service between A-IoT device and the UE reader shall be controlled and configured by the gNB. The UE reader will follow the configuration to trigger A-IoT service. Thus, Option 1 is the most straightforward solution for Topology 2. For Option 2, leaving UE reader to randomly select radio resources might lead to interference in not only the Uu interface, but also the A-IoT air interface and also increase the implementation complexity of the UE reader, which is not suitable to A-IoT system.

Proposal 1: For A-IoT Topology 2, the gNB provides dedicated radio resources for A-IoT air interface to the UE reader by dedicated RRC signalling.

UE reader selection

For the selection of UE reader, it could be the CN and/or gNB to determine a suitable UE reader upon an inventory and/or command service. Since this issue is under discussion in RAN3 and SA2, RAN2 can wait for more progress and focus on the potential Uu impacts if identified by RAN3/SA2.

Proposal 2: For the selection of UE reader, RAN2 wait for more progress in RAN3/SA2 and further discuss the potential impacts if identified by RAN3/SA2.

Moreover, from RAN2’s perspective, before selecting a UE reader and sending the A-IoT radio configuration to a UE reader, the gNB shall get the UE radio capability of A-IoT and decide the configuration accordingly. Regarding the A-IoT UE radio capability as a UE reader, RAN2 can further discuss the details in the normative work phase.

Proposal 3: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the normative work phase.

RRC state of UE reader

In order to support the A-IoT service by the UE reader for Topology 2, the UE reader shall be in RRC connected state to receive the service request and the radio configuration from the NW. After gathering the A-IoT data from A-IoT devices, the UE reader need to send the related data to the NW, which may require the UE in RRC connected stated as well. If an idle mode or inactive mode UE is selected as a UE reader by the NW, it could be paged in the legacy way and enter into RRC connected mode accordingly.

Proposal 4: For A-IoT Topology 2, the UE reader shall be in RRC connected state when receiving the A-IoT related configuration from the gNB or when reporting the A-IoT data to the NW.

Proposal 5: For A-IoT Topology 2, if an idle mode or inactive mode UE is selected as a UE reader by the NW, it could be paged in the legacy way and enter into RRC connected mode accordingly.

Regarding whether to support the UE reader to fallback to idle mode or inactive mode during the A-IoT procedure between the UE reader and A-IoT devices, it might be a further enhancement comparing to keep the UE reader in RRC connected state. In the Rel-19 SI phase, we suggest RAN2 focus on the basic function for supporting A-IoT Topology 2. Whether to support idle mode or inactive mode during the A-IoT procedure could be further discussed in future release.

Proposal 6: For A-IoT Topology 2, the UE reader keeps in RRC connected mode during the A-IoT procedure between the UE reader and A-IoT device.

UE reader “temporarily” out of connection

In last RAN2 meeting, the issue on UE reader “temporarily” out of connection was discussed. RAN2 will study the potential cases such as RLF and HO, etc.. Considering the commercial use cases and the network deployment for A-IoT, the intermediate UE might be relatively stationary or moving slowly in a small area. The possibility of HO is relatively low. To have a seamless solution for the ongoing A-IoT service during HO is too complex and will increase the cost of the UE reader. Moreover, it is quite likely that the potential HO of the UE reader is under the control of the gNB, since there is no clear need to support the coexistence of A-IoT and UE triggered HO such as CHO. Thus, a simple way might be that the UE reader suspends the A-IoT service during the HO. After HO to the target cell, the UE reader can just follow the configuration of the network to resume/re-initiate/stop the A-IoT service.

Proposal 7: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure.  After HO to the target cell, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

Regarding the Uu RLF handling of UE reader, the UE reader can follow the legacy behaviour to initiate RRC re-establishment procedure and suspend the transmission of all RBs in the Uu interface. For the A-IoT air interface, the ongoing A-IoT service shall be suspended as well. After the re-establishment procedure, the UE reader can follow the configuration of the network to resume/re-initiate/stop the A-IoT service, which is similar to the HO case.

Proposal 8: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 9: For the A-IoT air interface, the ongoing A-IoT service shall be suspended upon Uu RLF of the UE reader. After the re-establishment procedure, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

- Conclusion

In this contribution, we discuss Topology 2 specific issues for Ambient IoT. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For A-IoT Topology 2, the gNB provides dedicated radio resources for A-IoT air interface to the UE reader by dedicated RRC signalling.

Proposal 2: For the selection of UE reader, RAN2 wait for more progress in RAN3/SA2 and further discuss the potential impacts if identified by RAN3/SA2.

Proposal 3: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the normative work phase.

Proposal 4: For A-IoT Topology 2, the UE reader shall be in RRC connected state when receiving the A-IoT related configuration from the gNB or when reporting the A-IoT data to the NW.

Proposal 5: For A-IoT Topology 2, if an idle mode or inactive mode UE is selected as a UE reader by the NW, it could be paged in the legacy way and enter into RRC connected mode accordingly.

Proposal 6: For A-IoT Topology 2, the UE reader keeps in RRC connected mode during the A-IoT procedure between the UE reader and A-IoT device.

Proposal 7: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure.  After HO to the target cell, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

Proposal 8: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 9: For the A-IoT air interface, the ongoing A-IoT service shall be suspended upon Uu RLF of the UE reader. After the re-establishment procedure, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

- Reference

RP-240826	Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR 		CMCC, Huawei, T-Mobile USA

3GPP TR 38.848 v18.0.0: " Study on Ambient IoT (Internet of Things) in RAN".

R2-xxxxxx	draft Report of 3GPP TSG RAN WG2 meeting #125bis, 126,127