3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408043

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.2

Source:	China Telecom

Title:	LP-WUS in RRC_IDLE and INACTIVE

Document for:	Discussion and Decision

- Introduction

In the RAN2#127 meeting, the following agreements for LP-WUS operation in RRC_IDLE/INACTIVE were made [1]:

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE. Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

In this contribution, we will further discuss the issues for RRC_IDLE/INACTIVE with LP-WUR including at least sub-grouping, configuration and dynamic PO aspects.

- Discussion

Enable/disable LP-WUS monitoring

The LP-WUS configuration by broadcasting is cell-specific and common for all the UEs in the cell. However, it is not applicable to enable LP-WUS monitoring for all users. Considering different device types or traffic types, it may be necessary to enable/disable a certain user or some kinds of users to monitor LP-WUS for the UEs supporting LP-WUS. Moreover, it’s better to re-enable LP-WUS monitoring for these users that generate movement or location updates. Thus, to enhance the power saving gain and reduce false paging probability, we think the network (e.g., CN or RAN) should indicate per UE to enable/disable the LP-WUS functionality. The detailed signalling can be further discussed later.

Proposal 1. The network should indicate per UE to enable/disable the LP-WUS functionality.

LP-WUS Subgrouping

As for the UE_ID based subgrouping, we discussed it in the last RAN2 meeting, but no consensus has been reached due to time constraints. We have agreed that the PEI subgrouping method should be used as a baseline for LP-WUS subgrouping. A similar formula as PEI can be used as a baseline for the UE_ID based subgrouping. However, in case LP-WUS is used together with PEI, an independent subgrouping ID should be designed for LP-WUS and PEI separately to reduce false alarm rates. In this way, UE from the same LP-WUS subgroup can be relevant to different PEI subgroups. In order to minimise the impact on the modification of Spec, we can modify the parameters (i.e. UE_ID) in the formula as PEI. FFS on the details.

Proposal 2. In UE_ID based subgrouping, an independent subgrouping ID different from PEI should be applied for LP-WUS .

Proposal 3. For UE_ID based subgrouping for LP-WUS, we can modify the parameters based on the design of the formula for PEI. FFS on the details.

If both LP-WUS and PEI are configured and used for the same user, the types of subgrouping can be the same or different. For example, LP-WUS and PEI are both use either CN assigned or UE_ID based subgrouping methods. Alternatively, LP-WUS applies CN assigned subgrouping method and PEI applies UE_ID based subgrouping method and vice versa. In all, the detailed method can be indicated by NW.

Proposal 4. NW can decide which subgrouping method should be applied for LP-WUS and PEI together.

Configuration

RAN1 has divided the UE into three categories including OFDM-based WUR, OOK-based WUR and the UE supports the above two types of WURs. Different kinds of devices can bring different performance gains, i.e. latency, and accuracy. Therefore, the categories of WUR can be configured in SIB to indicate users in the cell.

Proposal 5. Which kind of WUR(i.e. OFDM-based, OOK-based, both types) can be configured in SIB.

Whether to support dynamic PO

As for the dynamic PO, RAN1 has discussed and reached the following agreement:

Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO

The dynamic PO indicates that UE starts to monitor PO after receiving network indication, which is different from waiting for legacy paging occasion. From the perspective of latency, the dynamic PO can indeed reduce latency. However, it also leads to complex signalling processes and protocol impacts. Thus, we think it is not necessary to support dynamic PO at this stage.

Proposal 6. From the RAN2 perspective, it is not necessary to support dynamic PO at this stage.

The minimum Time gap

According to RAN1 agreements, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring for RRC_CONNECTED is introduced for RRC CONNECTED mode. Similarly, this kind of parameter is also necessary to be defined for RRC IDLE/INACTIVE modes. When LP-WUS and PEI are simultaneously activated, the time gap refers to the offset between LP-WUS reception and MR monitoring for PEI. If UE does not support PEI or PEI is not activated, the time gap refers to the offset between LP-WUS reception and MR monitoring for paging. It is efficient to configure the minimum time gap through SIB. The value of the minimum time gap is up to RAN1 discussion.

Proposal 7. RAN2 to define the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring for RRC IDLE/INACTIVE modes.

SI reception procedure

Regarding SI reception/updates, we have agreed that no new procedure is introduced for them. We can reuse the legacy SI procedure to monitor messages in paging DCI. To avoid redundant signalling transmission, we can introduce a common LP-WUS to wake up UEs in all the subgroups to receive the updated SI. In our view, it’s very efficient for this design.

Proposal 8. For SI reception/updates, we should design a common LP-WUS to wake up all the UEs to monitor paging messages.

- Conclusion

According to the above discussion, our proposals are provided as follows:

Proposal 1. The network should indicate per UE to enable/disable the LP-WUS functionality.

Proposal 2. In UE_ID based subgrouping, an independent subgrouping ID different from PEI should be applied for LP-WUS .

Proposal 3. For UE_ID based subgrouping for LP-WUS, we can modify the parameters based on the design of the formula for PEI. FFS on the details.

Proposal 4. NW can decide which subgrouping method should be applied for LP-WUS and PEI together.

Proposal 5. Which kind of WUR(i.e. OFDM-based, OOK-based, both types) can be configured in SIB.

Proposal 6. From the RAN2 perspective, it is not necessary to support dynamic PO at this stage.

Proposal 7. RAN2 to define the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring for RRC IDLE/INACTIVE modes.

Proposal 8. For SI reception/updates, we should design a common LP-WUS to wake up all the UEs to monitor paging messages.

- References

3GPP TSG RAN WG2 #127 RAN2 Chair’s Notes.