3GPP TSG-RAN WG2 Meeting #127bis	           R2-2408045

Hefei, China, Oct 14th – Oct 18th, 2024

Agenda item:	8.5.2

Source:	China Telecom

Title:	Further consideration for on-demand SSB

Document for:	Discussion

- Introduction

According to the revised WID [1], one of the objectives is on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

RAN2 discussed on R19 on-demand SSB for the first time in RAN2#127 meeting and had the following agreements on on-demand SSB:

Scenarios on OD-SSB: 1. RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: 2. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 3. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Measurement on OD-SSB: 4. Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). 5. For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. 6. RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. 7. RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this paper, we would like to discuss on the remaining issues.

- Discussion

2.1	RRC based signaling to indicate on-demand SSB transmission

RAN1 has discussed signaling mechanisms to indicate on-demand SSB transmission and has reached agreements to support indication by both RRC and MAC CE signalling.

Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

According to RAN1’s agreements, RRC signaling can be used to provide on-demand SSB configuration and indicate on-demand SSB transmission. In current RRC spec, IE SCellConfig is used to add SCell(s) or modify the SCell configuration information, and the parameter sCellState contained in IE SCellConfig is used to indicate whether the SCell shall be considered to be in activated state upon SCell configuration. A similar indication can be introduced to indicate initial activation/deactivation state of on-demand SSB configuration, which has been agreed by RAN2#127. Reconfiguration is left FFS. In our understanding, reconfiguration of on-demand SSB configuration should be supported to modify or release the on-demand SSB configuration, e.g. via RRCReconfiguration message. The initial activation/deactivation state indication is part of new IE introduced for on-demand SSB configuration. In this way, the initial activation/deactivation state is reconfigured.

On the other hand, on-demand SSB should be configured per SCell. The IE introduced for on-demand SSB configuration can be included in current IE SCellConfig. The SCell index in current IE SCellConfig is reused for on-demand SSB.

Proposal 1: Reconfiguration should be supported to modify or release the on-demand SSB configuration including the initial activation/deactivation state of OD-SSB configuration per SCell.

2.2	MAC CE based signaling to indicate on-demand SSB transmission

RAN1 send an LS [2] to request RAN2 to design MAC CE based signaling to indicate on-demand SSB transmission for Scenarios #2 and #2A. RAN2#127 also agreed that new MAC-CE for OD-SSB transmission indication is introduced. However, according the RAN1#118 agreements, besides the indicator of on-demand SSB transmission, the MAC CE is also used to convey other information including at least periodicity of the on-demand SSB. Any other relevant parameters is FFS. From our point of view, it is better to wait for RAN1’s further input about other relevant parameters for on-demand SSB transmission indication for the cell, before designing the MAC CE format.

Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement (RAN1#116bis) For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

Proposal 2: RAN2 wait for RAN1’s further input about the relevant parameters for on-demand SSB transmission indication, before designing the MAC CE format.

Regarding scenario 2A, we think it’s better to follow the agreement that we will not change legacy SCell activation/deactivation MAC CE and have common solution for scenario 2 and 2A, i.e. no further optimization for scenario 2A.

Proposal 3: Regarding new MAC-CE based OD-SSB transmission indication, no further optimization special for scenario 2A is needed.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: Reconfiguration should be supported to modify or release the on-demand SSB configuration including the initial activation/deactivation state of OD-SSB configuration per SCell.

Proposal 2: RAN2 wait for RAN1’s further input about the relevant parameters for on-demand SSB transmission indication, before designing the MAC CE format.

Proposal 3: Regarding new MAC-CE based OD-SSB transmission indication, no further optimization special for scenario 2A is needed.

4	References

- [1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson, Apple.

- [2] R1-2407438, LS to RAN2 for on-demand SSB SCell operation, TSG RAN WG1.