3GPP TSG-RAN WG2 Meeting #127bis	R2-2408054

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.6.3

Source: 	China Telecom

Title:	Discussion on L1 event triggered measurement reporting

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made on measurement enhancements for LTM. Regarding the event-triggered L1 measurement reporting for LTM, the following agreements were reached.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we would like to share our opinions on further details of L1 event triggered measurement reporting for LTM.

- Discussion

Event evaluation and triggering

In RAN2#126 meeting, the following agreement was reached on LTM event evaluation. Whether to support measurement report once leaving conditions is met remains FFS.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In legacy L3 measurement reporting, whether to report measurement results when meeting the leaving condition is indicated by an RRC parameter configured per event. For L1 event-trigger measurement reporting, supporting reportOnLeave can help the NW to know the latest beam quality of serving cell and/or candidate cell(s) timely, which is also useful for LTM decision. Similar to L3 measurement reporting, whether to support measurement report upon meeting the leaving condition could be configured per-event level to have more flexibility.

Proposal 1: For event-triggered L1 measurement reporting, support L1 measurement reporting once leaving conditions is met, which could be configured by RRC parameter in per-event level.

In last RAN2 meeting, the issue on beam(s) of the serving cell and candidate cell used for event evaluation was discussed. For the event evaluation in L1 measurement reporting for serving cell, current beam (i.e. a beam corresponding to the indicated TCI state) is used, while for the candidate cell(s), any beam in candidate RS configuration can be used for LTM event evaluation. Based on the agreement, it may need further discussion on how to determine the fulfilment of an event with the configuration of TTT. For example, beam X of candidate cell A satisfies the entering condition of EventLTM4 in time of t1. During TTT, beam Y of cell A satisfies the entering condition of EventLTM4 as well. Then, how to handle TTT remains unclear, which may be related to the granularity of TTT. If TTT is a beam-level configuration, the UE has to associate a TTT to one beam of a candidate cell. The handling of TTT from different beams is independent. In this way, it may trigger multiple L1 measurement reports of the same candidate cell, since different beams may fulfil the condition in different time. If TTT is a cell-level configuration, the UE will associate a TTT to a candidate cell. Then, during TTT, if the beam that satisfy the entering condition is changed, whether to reset the TTT may need further discussion. Since different granularity may lead to different handling of TTT and measurement report triggering, it is better to clarify this issue.

Proposal 2: RAN2 clarifies the granularity of TTT for event-triggered L1 measurement reporting, e.g. beam-level or cell-level.

In legacy Layer 3 mobility, S-MeasureConfig is used to avoid unnecessary measurements on non-serving cells when the quality of the current serving cell is good enough. For event-triggered L1 measurement reporting for LTM, similar mechanism could be considered as well to avoid unnecessary measurements on candidate beams if the current beam(s) of the serving cell is good enough.

Proposal 3: To avoid unnecessary measurements on candidate beams in LTM, S-Measure like mechanism could be considered for event-triggered L1 LTM measurement report.

Configuration of event-triggered L1 measurement reporting for LTM

In last few RAN2 meetings, the following agreements on configuration of L1 LTM measurement event were reached.

RAN2#125bis

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

RAN2#127

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

Based on the above agreement, L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signalling. The event triggered report configuration is provided in serving cell config. The detailed association between the L1 LTM measurement event configuration and L1 measurement resource configuration may need further discussion.

For L3 measurement configurations, there are multiple reporting configurations with different Reportconfig IDs associated with different measurement objectives of different MesureObject IDs. For configuration of LTM event, the similar framework could be followed. A configuration ID for L1 LTM measurement event configuration could be introduced to identify an L1 LTM measurement event configuration.

Proposal 4: For L1 LTM measurement event configuration, a configuration ID could be introduced to identify an L1 LTM measurement event configuration.

To associate a configuration ID of L1 LTM measurement event configuration with L1 measurement resource configuration, there may be different options with different granularities.

- Option 1: a configuration ID of L1 LTM measurement event configuration is associated with one beam;

- Option 2: a configuration ID of L1 LTM measurement event configuration is associated with one cell.

For Option 1, different beams of a candidate cell may have different L1 measurement event configurations. Then the candidate beam in the event definition refers to the associated beam. For Option 2, different beams of a candidate cell have the same L1 event configuration. The configuration ID of L1 event configuration is associated with a LTM-CSI-ResourceConfigId.

Since no strong motivation is foreseen to support per-beam level event configuration, per-cell level event configuration is preferred. In this way, for a number of candidate cells that have the same LTM event configuration, the network can associate these candidate cells (i.e. LTM-CSI-ResourceConfigId) to the same configuration ID instead of configuring the same content of the event configuration multiple times.

Proposal 5: Regarding how to associate a configuration ID of L1 LTM measurement event configuration with L1 measurement resource configuration, a configuration ID of L1 LTM measurement event configuration is associated with one candidate cell and/or one serving cell.

What information in the measurement report?

In Rel-18, LTM is introduced to reduce the handover latency and interruption time compared to Layer 3 based mobility. The network may trigger LTM cell switch based on the L1 measurement report(s) from the UE. In legacy Layer 3 mobility, the Layer 3 measurement report is event triggered, which can not only reduce signalling overhead but also provide the available measurement in time. However, UE evaluated events for triggering of L1 measurement reports is not supported for Rel-18 LTM. In Rel-19, event-triggered L1 measurement reporting for LTM will be supported to reduce signalling overhead for CSI reporting.

In Rel-18 LTM, CSI report procedure is used for reporting L1 measurements for LTM, which could be reported in periodic/semi-Persistent/aperiodic way according to the network configuration. The UE can report a fixed number of L1 measurement results within a single L1 measurement report instance, which is configured by the parameters of nrOfReportedRS-PerCell and nrOfReportedCells. And the UE can only report the L1 measurement result of the current SpCell if the parameter of spCellInclusion is configured when the current SpCell is configured as an SpCell of an LTM candidate configuration.

In previous RAN2 meeting, the following events for LTM are introduced.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

As per the above events for LTM, not only the L1 measurement results of candidate cells but also serving cell are needed to be reported when meeting the trigger conditions. It is obvious that the Rel-18 L1 measurement report cannot be reused directly. Take Event LTM3 and Event LTM5 as example, the L1 measurement reports shall include the L1 measurement results of both the candidate cells and the current serving cells within a single L1 measurement report. For Event LTM2, only the L1 measurement results of serving cells are needed, while for Event LTM4, only the L1 measurement results of candidate cells are needed.

Proposal 6: Regarding the event-triggered L1 measurement reporting for LTM, a single measurement report could include the following L1 measurement results.

- Only the results of serving cell;

- Only the results of candidate cell(s);

- Both candidate cell(s) and serving cell results.

Besides, on top of reducing signalling overhead for reporting, we shall also consider how to provide enough valid L1 measurement results to the gNB in time to guarantee the mobility performance and user experience. Considering different Events may have different number of L1 measurement results to be reported, it is better to have the event-triggered L1 measurement report with a flexible length.

Proposal 7: For LTM event-triggered L1 measurement reporting, the L1 measurement report could have a flexible length.

- Conclusion

In this contribution, we discuss L1 event triggered measurement reporting for Rel-19 LTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For event-triggered L1 measurement reporting, support L1 measurement reporting once leaving conditions is met, which could be configured by RRC parameter in per-event level.

Proposal 2: RAN2 clarifies the granularity of TTT for event-triggered L1 measurement reporting, e.g. beam-level or cell-level.

Proposal 3: To avoid unnecessary measurements on candidate beams in LTM, S-Measure like mechanism could be considered for event-triggered L1 LTM measurement report.

Proposal 4: For L1 LTM measurement event configuration, a configuration ID could be introduced to identify an L1 LTM measurement event configuration.

Proposal 5: Regarding how to associate a configuration ID of L1 LTM measurement event configuration with L1 measurement resource configuration, a configuration ID of L1 LTM measurement event configuration is associated with one candidate cell and/or one serving cell.

Proposal 6: Regarding the event-triggered L1 measurement reporting for LTM, a single measurement report could include the following L1 measurement results.

- Only the results of serving cell;

- Only the results of candidate cell(s);

- Both candidate cell(s) and serving cell results.

Proposal 7: For LTM event-triggered L1 measurement reporting, the L1 measurement report could have a flexible length.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB

R2-24xxxx	Draft report of 3GPP TSG RAN WG2 meeting #127