3GPP TSG-RAN WG2 Meeting #127bis	R2-2408056

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	MRO for Rel-18 mobility features

Agenda Item:	8.10.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#127 meeting, RAN2 reached the following agreements regarding the MRO for Rel-18 mobility features:

LTM Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC. introduce a new field in RLF report to indicate the LTM recovery cell id. RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled. We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command. RA-based access will not be a new triggering condition for the SHR report. HO interruption time will not be a new triggering condition for the SHR report Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios. We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases. We will not log interruption time in SON reports (e.g. SHR) for LTM. We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec. CHO with candidate SCG UE includes following information in RLF report: - b. Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled. - c. Measurement results of PCells and PSCells.

In this contribution, we will provide further analysis of the remaining issues.

- Discussion

  - LTM

At the RAN2#127 meeting, it was agreed to log LTM access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

In the current RLF report content determination procedure, when the RLF type is HOF, the RLF report will include random access information, i.e., ra-InformationCommon. It has been agreed that extending lastHO-Type in RLF-Report to indicate the LTM cell switch as the last executed mobility procedure. Combining this new type and random access information, whether it is an RA-based or RA-less LTM cell switch can be implicitly signaled.

1> if connectionFailureType is hof and if the failed handover is an intra-RAT handover: 2> set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5;

Therefore, we propose:

Proposal 1: The access type (i.e., RACH-based or RACH-less) of LTM switch can be implicitly indicated by the presence of ra-InformationCommon in RLF report.

Furthermore, another aspect related to RACH-less LTM should be considered, i.e., the approach of RACH-less LTM. Specifically, two approaches for UE to obtain the TA during RACH-less HO are supported:

- Early sync: the UE sends a preamble to the target cell and the target cell forwards the TA value to the source cell. Upon LTM HO is triggered, the source cell indicates the TA value to UE in the LTM cell switch command.

- UE-based TA management: NW indicates UE whether UE-based TA measurement can be supported, and UE performs UE-based TA measurement (e.g., get the TA value of the target cell based on the RSTD between the source cell and target cell).

For the first approach, the TA may be invalid due to the UE moves after performing the early sync. In this case, the NW may optimize the early sync configuration, e.g., configure more frequent early sync or use RACH-based HO.

For the second approach, if the source cell and the candidate target cell are out of sync, the UE-based TA management may lead to HO failure. In this case, the NW may optimize the UE-based TA management configuration, e.g., adjust the ltm-ServingCellUE-MeasuredTA-ID associated with the cells.

To differentiate the above two approaches, how UE obtains the TA during RACH-less HO should be indicated in the RLF report.

Proposal 2: Record how UE obtains the TA during RACH-less HO in the RLF report, e.g., Early-sync, UE-based TA management.

Another open issue is how to indicate whether a neighbor cell is an LTM candidate cell or not, i.e., explicit or implicit.

Currently, the procedure of recording candidate CHO cells in SHR is as follows.

3> for each of the neighbour cells included in measResultNeighCells: 4> if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied: 5> set the choCandidate to true in measResultNR;

Our understanding is that a similar procedure should be introduced to record LTM candidate cells.

Proposal 3: Introduce a separate field in measResultNeighCells to indicate whether the neighbor cell is an LTM candidate cell.

  - CHO with candidate SCG

At the RAN2#125bis meeting, it was agreed to study failure and near-failure scenarios for CHO with candidate SCGs.

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

2.2.1	Failure CHO with candidate SCGs

For CHO with candidate SCG, UE cannot execute CHO with candidate SCG(s) until the execution conditions for both the candidate PCell and the associated candidate PSCell are met. Non-optimal configuration of CHO with candidate SCGs may result in MCG/SCG failure due to UE not executing CHO with candidate SCG(s) if only PCell’s execution condition is fulfilled or only associate PSCell’s execution condition is fulfilled.

For MCG failure, the SON for CHO in the RLF report can be taken as the baseline for enhancement.

[[ choCandidate-r17 ENUMERATED {true} OPTIONAL, choConfig-r17 SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16 OPTIONAL, triggeredEvent-r17 SEQUENCE { timeBetweenEvents-r17 TimeBetweenEvent-r17 OPTIONAL, firstTriggeredEvent-r17 ENUMERATED {condFirstEvent, condSecondEvent} OPTIONAL } OPTIONAL ]],

choCandidate This field indicates whether the associated cell is a candidate target cell for conditional handover or conditional PSCell change or addition. This field may be included only in the SuccessHO-Report or SuccessPSCell-Report within UEInformationResponse message.

choConfig If the associated cell is a candidate target cell for conditional handover, this field indicates the conditional handover execution condition for each measId within condTriggerConfig associated to the cell. This field may be included only in the rlf-report within UEInformationResponse message.

firstTriggeredEvent This field is set to condFirstEvent if the execution condition associated to the first entry of choConfig was fulfilled first in time. This field is set to condSecondEvent if the execution condition associated to the second entry of choConfig was fulfilled first in time. This field may be included in rlf-report within UEInformationResponse message or in SCGFailureInformation message.

timeBetweenEvents Indicates the time elapsed between fulfilling the conditional execution conditions included in choConfig. Value in milliseconds. The maximum value 1023 means 1023ms or longer. This field may be included in the reports associated to UEInformationResponse message, e.g., rlf-Report or in the SCGFailureInformation message.

From our understanding, the following information should be included in the MeasResultNR in RLF report:

- The CPAC configuration. Similar to CHO configuration, the CPAC configuration should also be included.

- The association between the candidate PCell for CHO and candidate PSCell for CPAC. With the association, the network may figure out whether to adjust the configuration of CHO or CPAC.

Proposal 4: For the MCG failure scenario of CHO with candidate SCGs, introduce the following information in the MeasResultNR within RLF report:

- CPAC configuration.

- Association between the candidate PCell for CHO and candidate PSCell for CPAC.

For SCG failure, the UE will send the SCGFailureInformation message to the serving gNB. As the serving gNB has the configuration of CHO with candidate SCGs, with the measurement in the SCGFailureInformation, the serving gNB can optimize the configuration of CHO with candidate SCGs. However, we are open to the essential enhancement, if any. Thus no proposal for SCG failure scenario from our perspective.

2.2.2	Near-failure CHO with candidate SCGs

From the NW perspective, the NW expects T1 and T2 to be as close as possible, where T1 is the time when PCell's execution condition is satisfied and T2 is the time when the associate PSCell’s execution condition is satisfied. So if PCell changes with PSCell addition/change is successfully executed but if the time difference between T1 and T2 is large, the NW needs to know that situation, UE should report some information to assist the NW in optimizing the configuration.

For the above near-failure scenario, the SHR can be taken as the baseline for enhancement. The most straightforward way is introducing another triggering event for the SHR report, i.e., the time difference between T1 and T2 exceeds a configured threshold. Correspondingly, the time difference between T1 and T2 should be introduced in the SHR. In addition, which one is fulfilled at first should also be indicated.

Proposal 5: Introduce another triggering event for the SHR report, i.e., the time difference between T1 and T2 exceeds a configured threshold, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

Proposal 6: For the near-failure scenario of CHO with candidate SCGs, introduce the following information in the SHR report:

- Time difference between T1 and T2, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

- The first fulfilled event, CHO or CPAC.

  - Subsequent CPAC

From our understanding, for failure and near-failure scenarios for subsequent execution of SCPAC, the existing SCGFailureInformation message and SPR can be reused, and no other enhancements are needed. However, we are open to the essential enhancement, if any. Thus no proposal for subsequent CPAC from our perspective.

- Conclusion

Based on the above analysis, we make the following proposals:

LTM

Proposal 1: The access type (i.e., RACH-based or RACH-less) of LTM switch can be implicitly indicated by the presence of ra-InformationCommon in RLF report.

Proposal 2: Record how UE obtains the TA during RACH-less HO in the RLF report, e.g., Early-sync, UE-based TA management.

Proposal 3: Introduce a separate field in measResultNeighCells to indicate whether the neighbor cell is an LTM candidate cell.

CHO with candidate SCG

Proposal 4: For the MCG failure scenario of CHO with candidate SCGs, introduce the following information in the MeasResultNR within RLF report:

- CPAC configuration.

- Association between the candidate PCell for CHO and candidate PSCell for CPAC.

Proposal 5: Introduce another triggering event for the SHR report, i.e., the time difference between T1 and T2 exceeds a configured threshold, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

Proposal 6: For the near-failure scenario of CHO with candidate SCGs, introduce the following information in the SHR report:

- Time difference between T1 and T2, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

- The first fulfilled event, CHO or CPAC.

- Reference