3GPP TSG-RAN WG2 Meeting #127bis	R2-2408057

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	Discussion on RACH optimization for SDT

Agenda Item:	8.10.5

Document for:	Discussion and Decision

- Introduction

At the RAN2#126 meeting, the following agreements were made regarding RACH optimization for SDT:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT. When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

At the RAN2#127 meeting, further agreements were reached:

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it. UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

In this contribution, we will provide our views on the open issues.

- Discussion

For SDT, the UE may initiate the SDT if the following criteria are met:

- sdt-Config is configured and SIB1 includes sdt-ConfigCommon;

- all the pending data in UL is mapped to the radio bearers configured for SDT;

- data volume of the pending UL data across all RBs configured for SDT is less than or equal to a threshold;

- RSRP of the downlink pathloss reference is higher than a threshold (if configured).

In Rel-18, RAN2 mainly focuses on RACH report enhancement for RA-SDT:

- UE includes RA and SDT information in RA report when an SDT operation fails.

- Include a single flag indicating whether the SDT failed or not.

#### 5.7.10.4 Actions for the Random Access report determination Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5 <omitted> 4> if the random-access procedure is initiated for SDT and the SDT transmission was failed as defined in TS 38.300 [2]: 5> include the sdt-Failed;

To assist the network in optimizing the RSRP threshold, the UE should record the RSRP measurement upon RA-SDT failure in the RA Information. Specifically, if the RSRP threshold is too low, the UE may fail to perform RA-SDT towards the camping cell due to poor signal quality. In this case, the gNB should increase the RSRP threshold for SDT.

Proposal 1: To assist the network in optimizing the RSRP threshold for SDT, upon RA-SDT failure, the UE may record the RSRP measurement in the RA-InformationCommon.

As to the failure cause, the SDT procedure will be unsuccessfully completed for the following cases:

Once initiated, the SDT procedure is either: - successfully completed after the UE is directed to RRC_IDLE (via RRCRelease) or to continue in RRC_INACTIVE (via RRCRelease or RRCReject) or to RRC_CONNECTED (via RRCResume or RRCSetup); or - unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or integrity check failure while SDT procedure is ongoing, or expiry of SDT-specific timing alignment timer or configuredGrantTimer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

From our understanding, within the causes, cell re-selection is not an abnormal case, and the rest causes are quite similar to RLF causes as follows:

rlf-Cause-r16 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, beamFailureRecoveryFailure, lbtFailure-r16, bh-rlfRecoveryFailure, t312-expiry-r17, spare1},

Therefore, cell re-selection does not need to be indicated in the failure cause of SDT and the other cause values can be considered: T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, integrityCheckFailure, TATimer-Expiry.

Proposal 2: For the SDT failure cause in RA-Report, the candidate values include: T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, integrityCheckFailure, TATimer-Expiry.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Proposal 1: To assist the network in optimizing the RSRP threshold for SDT, upon RA-SDT failure, the UE may record the RSRP measurement in the RA-InformationCommon.

Proposal 2: For the SDT failure cause in RA-Report, the candidate values include: T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, integrityCheckFailure, TATimer-Expiry.

- Reference