3GPP TSG-RAN WG2 Meeting #127bis							       R2-2408058

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.2

Source: 	NEC

Title:	Ambient-IoT Functionality Aspects

Document for:	Discussion/Decision

- Introduction

The Rel-19 SID for Ambient IoT (also referred to as A-IoT) includes following RAN2-led objectives [1].

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

This contribution discusses the functionality aspects based on the RAN2-led objectives and the latest RAN1 and RAN2 agreements.

- Discussion

  - User Plane Functions

    - Padding

In A-IoT, the D2R transmission is triggered by the reader while there isn’t any buffer status at that time. Thus, it may happened that the grant is larger than the higher layer data available to transmit. In NR, padding is used to solve this problem by reaching the exact grant size in UL. Since there is no need to keep byte alignment for A-IoT, the device can terminate the transmission early according to the exact data size. How to solve it is also related to the following agreement of RAN1#116bis [2] meeting on acquiring the end of PDRCH transmission:

RAN1#116 Agreements: For the reader to acquire the end of PDRCH transmission, study at least following options: • Option 1: D2R postamble immediately follows the PDRCH • Option 2: Based on control information

Either one of the above options can be used for the indication of the early PDRCH termination. So we suggest not to adopt padding function for MAC layer and the exact method for the indication of the early PDRCH termination is up to RAN1.

RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

    - Multiplexing

The transmission of signaling and the transmission of data are usually separate. Thus, there is no need to consider the multiplexing of multiple layer2 (e.g., logical) channels. Moreover, there is no need to consider the LCP of multiple layer2 channels.

In previous RAN2 meeting, only one logical channel is agreed for A-IoT transmission. However, during RAN2#126 discussion, some companies mentioned that indications on segmentation and harvesting statues from devices to the reader might be useful for the control of devices. These indication may be reported to the reader via MAC layer. Therefore, these indications may be sent together with the higher layer data. But they are not discussed and decided yet. So we propose:

RAN2 postpones the discussion on multiplexing.

  - Control Plane Functions

    - Visibility

RAN2#127 [3] meeting agreed the following:

At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

Knowing the specific command type (e.g. read/write/disable) allows for more precise resource scheduling for D2R transmission. For example, the response for “read” is a piece of data which may need more resources than that for “write”. So we propose:

The command type (e.g. read/write/disable) is useful to be visible to the reader from CN

    - Repetition

Transmission via the A-IoT air interface between the reader and the A-IoT device may encounter signal fading or blockage, just like traditional Uu interface. In addition, some A-IoT devices need to be charged to perform data reception or feedback. Then, it may fail to receive inventory/command request message due to energy status.

To solve the above mentioned problems, the reader may send inventory/command several times. To control the number of repetitions, the following methods can be considered:

- The specific number of repetition is visible to the reader from CN to control the repetition.

- The survival time of a single inventory/command request is visible to the reader. And the specific repetition times depend on the reader implementation.

RAN2 to consider the repetition of inventory/command request.

The control information for repetition (e.g. specific number of repetitions or survival time of a single inventory/command request) is useful to be visible to the reader from CN.

    - Energy Status Report

RAN2#127 [3] meeting agreed the following:

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

The transmission is triggered by the reader, so the energy status report can be transmitted along with the response message. And the following cases can be considered:

- The D2R transmission includes both the data for response and the energy status report, which can be used in situations where only partial data can be transmitted;

- The D2R transmission includes only the energy status report, which can be used in cases where no data can be transmitted.

RAN2 to study the following cases for the energy status report:

The D2R transmission includes both the data for response and the energy status report;

The D2R transmission includes only the energy status report.

In addition, the energy status report may include the following information:

- The energy status achieved via the (latest) energy harvesting procedure: this information may be used by the reader to track the harvesting procedure.

- The energy available in the battery: this information may be used to adjust the harvesting procedure or the scheduling of AIoT service. For example, if the energy status can support the next D2R transmission, the reader may trigger the D2R transmission directly. Otherwise, the reader may initiate another harvesting procedure for the next transmission.

- The energy needed for the leftover data or for the next transmission: this information may be used to adjust the harvesting procedure. For example, if this information is received by the reader, the reader may initiate a harvesting procedure for the next transmission.

- Power stability indication: this information may indicate whether the power/energy supply of this A-IoT device is stable or not, in other words, whether power stability should be considered in the operation of this A-IoT device. For example, if this information is received by the reader or if it is indicated as true, the second device/entity may consider this A-IoT device has discontinuous power/energy supply, i.e., it may be out of power in some conditions. This information may be used to adjust the scheduling of AIoT service.

- Validation duration: this information gives the validation duration of the above one or more information, e.g., it may be multiple seconds/minutes/hours/days or multiple frames/hyper frames, the above one or more information will be valid within this validation duration. If this information is absent, the above one or more information elements will be valid until a new PSI which include a new valid duration information is received by the second device/entity.

RAN2 to study the following information as the energy status report:

The energy status achieved via the (latest) energy harvesting procedure;

The energy available in the battery;

The energy needed for the leftover data or for the next transmission;

Power stability indication;

Validation duration.

    - Message size indication

In RAN2#127 [3] meeting, there is also an agreement on message size reporting shown as below:

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

As we already known from RAN2#126 meeting that legacy NR BSR/SR is agreed not necessary for A-IoT communication, then simple message size reporting mechanism is needed to support efficient data/message scheduling in AIoT. Targeting this, a ratio based message size reporting mechanism is proposed in this section, where the ratio is defined as remaining message size over previous scheduled size at device side. By introducing this metric, efficient data scheduling for AIoT communication can be supported as shown in Fig. 1.

Fig. 1 Ratio based message size reporting mechanism for AIoT communication

To be specific,

Step 1. A first PRDCH with TBS1 indication of a subsequent PDRCH is scheduled by reader.

Step 2. Device receives the PRDCH and obtains TBS1, then a ratio r = remaining message size / TBS1 = (total D2R message size - TBS1) / TBS1 will be calculated by device. After that, device reports the ratio together with TBS1 size of D2R message in the corresponding PDRCH.

Step 3. After reader receives PDRCH successfully, reader will calculate the remaining D2R message size by the reported r, then reader may indicate TBS2 in another PRDCH for another PDRCH scheduling. This procedure will repeat until the completion of all D2R message transmission.

Ratio based message size report is a simple and efficient mechanism for data scheduling in AIoT communication.

RAN2 to study ratio based message size reporting for AIoT communication.

    - Device variable control

Considering the limited size and complexity required by practical applications for battery-less devices with no energy storage capability or devices with limited energy storage that do not need to be replaced or recharged manually, the persistence of device variables may consider both the energy status of the A-IoT device and the type of the device variables.

From the persistence behavior perspective, three types of device variable control can be considered:

Type 1 Infinite persistent: The variables required to be kept even the A-IoT device is de-energized. For example, the information related to registration with the NW, which is stored in the device memory.

Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

Considering three types of Device variable control behaviors:

Type 1 Infinite persistent: The variables need to be stored even the A-IoT device is de-energized, such as information related to registration with the NW, etc.

Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	Ratio based message size report is a simple and efficient mechanism for data scheduling in AIoT communication.

Observation 2:	Considering three types of Device variable control behaviors:

−	Type 1 Infinite persistent: The variables need to be stored even the A-IoT device is de-energized, such as information related to registration with the NW, etc.

−	Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

−	Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

Proposal 1:	RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

Proposal 2:	RAN2 postpones the discussion on multiplexing.

Proposal 3:	The command type (e.g. read/write/disable) is useful to be visible to the reader from CN

Proposal 4:	RAN2 to consider the repetition of inventory/command request.

Proposal 5:	The control information for repetition (e.g. specific number of repetitions or survival time of a single inventory/command request) is useful to be visible to the reader from CN.

Proposal 6:	RAN2 to study the following cases for the energy status report:

−	The D2R transmission includes both the data for response and the energy status report;

−	The D2R transmission includes only the energy status report.

Proposal 7:	RAN2 to study the following information as the energy status report:

−	The energy status achieved via the (latest) energy harvesting procedure;

−	The energy available in the battery;

−	The energy needed for the leftover data or for the next transmission;

−	Power stability indication;

−	Validation duration.

Proposal 8:	RAN2 to study ratio based message size reporting for AIoT communication.

- Reference

- RP-240826, Rel-19 SID of Ambient IoT.

- Chairman Notes, RAN1 #116bis meeting.

- Chairman Notes, RAN2 #127 meeting.