3GPP TSG-RAN WG2 Meeting #127bis                      R2-2408059

Hefei, China, Oct.14th – Oct.18th, 2024

Agenda Item:	8.2.3

Source: 	Transsion Holdings

Title:  	Discussion on Paging for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN2#127 meeting, the A-IoT Paging was discussed and the following aggrements were made[1]:

Agreements: 1Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we continue to discuss the Paging for A-IoT.

- Discussion

2.1 A-IoT Paging cases

It has made the agreement that RAN2 will study the following cases for AIoT paging message:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

- a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion.

Moreover RAN2 has also confirm that slotted-ALOHA is the baseline for A-IoT random access and the A-IoT device will randomly choose a slot based on a Q-like value to access. For all device or group paging, if the Q-like value is too small it will cause access congestion, if the Q-like value is to large it will waste resources and cause latency. For group paging or all device paging, as the reader may not know the number of the devices been paged, it may can’t set the Q-like value appropriately. As the CN or APP layer may have the rough number of the devices covered by a reader, the quantity information of the devices can be provided to the reader by upper layers.

Proposal 1: The quantity information of the devices can be provided to the reader.

With the quantity information of the devices, if the number of devices is huge, to improve the access efficiency, the group paging can be considered. The devices can be further grouped and the devices only respond to the paging belong to its own group. For example, the reader can include the number of groups N and the current paging group i in the paging message, the device can choose the paging group index based on the number of group N and when the current paging group i in the paging equals its paging group index, it responds to the paging message.

Proposal 2: For all device paging, the devices can be further grouped and device only respond to the paging belong to its own group.

It has been agreed to support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN and study how to avoid duplicated response from devices. To support this, devices receving the paging has to know whether the paging is the same as the previous one. To achieve this, a session id or failed device respond indication can be included in the paging message and the device has to record the session id or the paging response status. If the session id in the paging message is the same with the previous one or the failed device respond indication is in the paging message and the device has already respond the previous paging successfully, the device will ignore the current paging.

Proposal 3: A session id or failed device respond indication can be included in the paging message to avoid duplicate response.

2.2 A-IoT Paging message content

For all device or group paging, the devices may have to initiated contention-based access. As the periodical system information is not supported by AIoT devices, the contention-based access configuration can be provided in the Paging message.

Proposal 4: contention-based access configuration can be provided in the Paging message.

For a single A-IoT device or multiple devices paging, as the number of devices is already known by the reader, the corresponding access configuration can be assigned to each device to increase efficiency and avoid contention, the access configuration can include time and frequency configuration.

Proposal 5: The device id list and corresponding access configuration list can be included in the paging message if the number of devices being paged is known by the reader to avoid contention.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT paging and made the following proposals:

Proposal 1: The quantity information of the devices can be provided to the reader.

Proposal 2: For all device paging, the devices can be further grouped and devices only respond to the paging belong to its own group.

Proposal 3: A session id or failed device respond indication can be included in the paging message to avoid duplicate response.

Proposal 4: contention-based access configuration can be provided in the Paging message.

Proposal 5: The device id list and corresponding access configuration list can be included in the paging message if the number of devices being paged is known by the reader to avoid contention.

- 4 References

- “Draft_RAN2_127_Meeting_Report_v1”