3GPP TSG-RAN WG2 Meeting #127bis                      R2-2408060

Hefei, China, Oct.14th – Oct.18th, 2024

Agenda Item:	8.2.4

Source: 	Transsion Holdings

Title:  	Discussion on Random Access for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN2#127 meeting, the A-IoT Random Access was discussed and the following agreements were made[1]:

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

In this contribution, we continue to discuss the Random Access for A-IoT.

- Discussion

2.1 A-IoT contention-free access procedure

In last meeting the procedure of CFRA for AIoT device was discussed with which the device sends Device ID and/or any other upper layer data in MSG1, the reader may echo some information from MSG1 in MSG2. For the contention-free access procedure RAN2 will study single and multi-device case. For single and multi-device case, as the number of devices is already known by the reader, when trigger the device to access, the corresponding access configuration can also be assigned to each device to increase efficiency, the access configuration can include time and frequency configuration.

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

2.2 A-IoT Random access failure handling

If the triggered device access failed, the reader may meed to triggered the device access again which may not efficient. In legacy RACH procedure, the UE can re-try access using the RACH resources provided in SIB if RACH failed. For AIoT system the re-try access mechanism can be reused. However as the periodical system information is not supported by AIoT devices, the retry access configuration can be provided in the trigger message. If the devices being triggered access failed, it can re-try access using the retry access configuration without the reader re-initiated trigger. And the reader can limited the re-try times and provides the corresponding amount of retry access configuration.To save resources, the retry access configuration can be shared by devices.

Proposal 2: The retry access configuration can be provided in the trigger message for the failed access device to retry access without the reader re-initiated trigger.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT Random Access and made the following proposals:

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

Proposal 2: The retry access configuration can be provided in the trigger message for the failed access device to retry access without the reader re-initiated trigger.

- 4 References

- “Draft_RAN2_127_Meeting_Report_v1”