3GPP TSG-RAN WG2 Meeting #127bis                     R2-2408062

Hefei, China, Oct.14th – Oct.18th, 2024

Agenda Item:	8.6.3

Source: 	Transsion Holdings

Title:  	Discussion on L1 event triggered measurement reporting

Document for:	Discussion and Decision

- Introduction

In the last RAN2#127 meeting, the L1 event triggered measurement reporting was discussed and the following agreements were made[1]:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we continue to discuss the measurement enhancement for LTM.

- Discussion

- 2.1 Measurement event configuration for LTM

In last meeting it has been agreed that any beam in candidate RS configuration can be used for LTM event evaluation to reduce latency, which means that if any beam among the beams configured by LTM RS configuraiton meets the event, it triggerd MR. However due to the lots of configured LTM RS and the limited measurement capability, the beam triggers the L1 MR maybe not the best beam which will result the reduced performance. To avoid this, some meaurement priority for candidace cell or beam can be defined. For example, for some cells the UE is connected to, the UE is more likely to switch to the intra-DU candidate cells and for some other cells the UE is more likely to switch to the inter-DU/inter-CU candidate cells. To make this, the measurement priority of the candidate cell or beam can be defined or signaled with the measurement configuraiton.

Proposal 1:  The measurement priority for candidace cell or beam can be defined, UE priority meaures the candidate cell or beam with high priority.

2.2 Event triggered L1 measurement purpose for LTM

As agreed in last meeting, the event triggered L1 measurement shoud be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

The R18 LTM supports UE-based TA measurement for early synchronization and the exact time the UE performs TA measurement is up to UE implementation, thus the network doesn’t  know whether the UE has performs TA measurement or not. If the event triggered L1 MR is used for trigger early synchronization, it may cause repeated triggering early synchronization if the UE has already get the TA of candidates cell by UE-based TA measurement. To avoid this there can be two approaches: One is the purpose of the event triggered L1 measurement can be included in the measurement configuration. If the purpose is to select candidate beam/cell to trigger early synchronization and the UE has already get the TA of candidate cells, the UE can consider these candidate cells unavailable or doesn’t perform L1 measurement on these candidate cells or doesn’t trigger L1 measurement report if event condition is met to avoid repeated triggering early synchronization. The other one is the TA available information can be included in the L1 measurement report. If the network receives L1 measurement report with TA available information for a candidate cell, then the network will not send PDCCH order for this candidate cell to trigger early synchronization as the UE has already get the TA. The TA available information can be just one bit to indiate the network that the UE has already get the TA of the candidate cell.

Proposal 2: The purpose of event triggered L1 measurement can be included in the measurement configuration to avoid repeated triggering early synchronization.

Proposal 3: The TA available information can be included in the measurement report to avoid repeated triggering early synchronization.

- 3 Conclusions

In this contribution we discussed issues related to measurement enhancement for LTM and made the following proposals:

Proposal 1:  The measurement priority for candidace cell or beam can be defined, UE priority meaures the candidate cell or beam with high priority.

Proposal 2: The purpose of event triggered L1 measurement can be included in the measurement configuration to avoid repeated triggering early synchronization.

Proposal 3: The TA available information can be included in the measurement report to avoid repeated triggering early synchronization.

- 4 References

- “Draft_RAN2_127_Meeting_Report_v1”