3GPP TSG-RAN WG2 Meeting #127bis                            R2-2408064

Hefei, China, Oct. 14th – Oct.18th, 2024

Agenda Item:	8.9.2

Source: 	Transsion Holdings

Title:  	Discussion on support of Store&Forward

Document for:	Discussion and Decision

Introduction

In the last RAN2#127 meeting, the support of Store&Forward for IoT-NTN was discussed, and the following agreements were made[1]:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution, we provide our views on these issues to proceed the work item.

Discussion

Awareness of satellite Store&Forward mode

- In last meeting it has been agreed that the UE is informed of whether its serving satellite is currently operating in S&F via System information broadcast. Whether the UE is informed of the change time of the satellite’s current operation mode was discussed and no agreement is made. We think the change time of the satellite’s current operation is beneficial for UE’s operation. For example if the serving satellite works in normal mode and the work mode will change to S&F mode soon, then the UE may not choose to initiate access to the satellite. One may argue that this change can be done by system information change notification procedure. However this system information change notification procedure will result in paging for all UEs which is not good for power saving. Morever the IoT UEs in RRC_CONNECTED can’t acquire system information which will result not knowing the work mode change of the satellite. We think this work mode change is essential for IoT UEs in RRC_CONNECTED, the network has to release the UE according to current spec. So we suggest the change time of the satellite’s current operation mode can be broadcast together with the current satellite operation mode.

- Proposal 1: The network broadcasts the current operation mode of the satellite and the next change time of operation mode.

Paging enhancement in satellite Store&Forward mode

- As the satellite has already stored the downlink data for UE in advance, it means the network knows which UE needed to be paged and the paging demand is not random anymore and as the large coverage of the satellite, the number of UEs needed to be paged and the amount of downlink data needed to be send by the satellite maybe huge, which may cause paging congestion.

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 2: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- The satellite in S&F mode can distribute the paging message in time to avoid congestion from network’s perspective, however the UE has to monitor the PF/POs in every DRX cycle no matter whether there is a paging for the UE or not and there are no more paging message from the core network as there is no feeder link between the satellite and the ground, so the paging send by satellite and paging monitoring by UE can be optimized for power saving. For example there can be two DRX parameters which is different for S&F mode and normal mode, the DRX cycle or nB for S&F mode can be set larger than those in normal mode. Moreover if the UE keeps monitoring paging for some cycle or some times and no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

- Proposal 3: The paging send or monitoring can be optimized for power saving when network operates in S&F mode.

UP CIoT EPS optimisation support in satellite Store&Forward mode

For UP mode, when UE resume the RRC connection with a eNB, if the eNB is not the old eNB which suspend the connection, the eNB has to fetch the context from the old eNB. However for satellite in S&F mode, as the fast moving satellite and the no ISL between the satellite, the eNB the UE trys to resume may not the old eNB and can’t get the context from the old eNB, the UE has to fallback to the RRC connection setup procedure. To avoid this, it has to make sure that the eNB the UE resume has the UE context. To achieve this, the context can be transfer to specific satellites and UE only resume the connection to this satellites. As to how to decide these satellites, there can be two methods: one is the UE itself determines the satellites based on the satellite assistance information and the UE traffic pattern and send the satellite list to the network, then the netwrok confirms some or all of them and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered. The other method is the UE provide some assistant information such as UE location and traffic pattern to the network and the network determines the satellite list and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered.

- Proposal 4: When UP mode is used, the UE context can be transfered/distributed to specific satellites via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

Conclusion

In this contribution we discussed issues related to S&F mode in IoT-NTN and made the following proposals:

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 1:  The network signals the current operation mode of the satellite and the change time of operation mode change.

- Proposal 2: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- Proposal 3: The paging monitoring can be optimized for power saving when network operates in S&F mode.

- Proposal 4: When UP mode is used, the UE context can be transfered/distributed to specific satellies via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

Reference

- “Draft_RAN2_127_Meeting_Report_v1”