# 3GPP TSG-WG2 Meeting #127bis              	   R2-2408066

# Hefei, China, Oct 14 - 18, 2024

# Source: 			CMCC

# Title:  Discussion on IoT NTN Store and Forward

# Agenda item:	8.9.2

# Document for: 	Discussion

Introduction

In the last RAN2#127 meeting, the following agreements [1] were approved as below:

RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

RAN2 will consider both single satellite pass and multiple satellite pass scenarios

RAN2 will consider both MO and MT data within scope

UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

Come back to this in the next meeting：UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

Based on the discussions and conclusions in the previous meetings, we continue discussing some RAN2 related aspects in S&F operation.

Discussion

Access control

If the satellite realizes there are continually operation switching between normal operation and S&F operation before t-service timeout (e.g. the feeder link status is not stable), it’s better for network to indicate operation switching using half-static indication via SIB message (which means sending all indication information of every operation switching in SIB message at once). To be specific, if there are lots of operation switching between normal and S&F operation (e.g. more than 4 times), dynamic indication requires the network to send four indications when switching that will cause big signaling overheads. If UE failed to receive SIB message and there will be some latency and inconsistency. Thus, by using half-static introduction, the network can send all the detail indication information of every operation switching in SIB message at once. When the half-static indication is not changed, network does not need to send it again. If changed, only sending changed part is enough. In this way, UE is clear to know the total numbers of operation transition and the duration time of each operation which will bring great convenience for UE to set related parameters and configurations in advance and to provide a better connection. If the S&F operation information does not change, the network also does not need to change the broadcast message and if some information is changed (e.g. duration time or S&F operation start time, etc), then the network only needs to broadcast the changed information in SIB message.

Proposal 1: For multi operation transition scenario, satellite can send all indication information of every S&F operation switching in SIB message at once.

Proposal 2: If Proposal 1 is agreed, the satellite only needs to broadcast the changed S&F operation information (e.g. starting time, duration time).

As for the legacy UE, RAN2#127 has made such agreement that not discussing about barring legacy UE in RAN2 anymore except that there is progress in SA2. Though no need to talk about barring legacy UE, it means there still has possibility that legacy UE can work in S&F operation. To be specific, during the operation transition procedure, when legacy UE camp on the cell in normal operation first, then the feeder link of the satellite is not available so that the satellite changes to S&F operation. Since the satellite has already stored UE context, NAS information, legacy UE can skip the initial attach procedure in S&F operation and directly work in the S&F operation. If the current operation mode of satellite is S&F operation, then the legacy UE can still not complete the initial access procedure because of the huge NAS latency both in MME-split and full CN onboard architecture.

Observation 1: In both MME-split and full CN onboard architecture, legacy UE can not complete the initial access procedure in the cell within S&F operation.

Proposal 3: If the satellite is currently operating in normal operation and change to S&F operation, then legacy UE which already finished initial access procedure with the satellite may access the cell in S&F operation.

Based on Proposal 4, since legacy UE has chance to access in the cell in S&F operation, we need to analyze legacy UE behaviour in S&F operation and potential impact to data transmission. Table 1 show the impact of S&F architecture to legacy UE DL/UL data transmission.

Table 1 S&F architecture impact to legacy UE DL/UL data transmission

| Data direction /Architecture | MME split architecture | Full CN onboard architecture |
| --- | --- | --- |
| UL | No impact from RAN aspect | No impact from RAN aspect |
| DL | Can not receive new NAS DL data | No impact from RAN aspect |

From the table, we can see that legacy UE has no problem in DL/UL data transmission in full CN onboard architecture. However, within MME split architecture, legacy UE can not receive new NAS DL data even legacy UE can receive the RLC/MAC status report from eNB about the UL data transmission status which is from AS layer. Thus, whether legacy UE can transmit UL data in this situation needs further discussion.

Proposal 4: Legacy UE can work in the cell in S&F operation at least in full CN onboard structure. FFS further enhancement is needed for DL transmission in MME-split structure when there is no new NAS data received.

- During the initial attach procedure, if UE is rejected with new reject message including wait timer, SA2 give the conclusion about the UE behaviour during the wait time as below:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

3)	How the UE process this information is up to UE implementation and during the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service.

- From SA2 conclusion, it is clear to see that UE can still search for terrestrial or satellite in normal operation during the waiting time. Thus, in our opinion, if UE is rejected by the satellite during initial access procedure due to S&F operation, it can still try to camp on other normal cell or satellite if available before the wait timer expired. This operation can keep UE flexibility and do not need to only wait for S&F timer expired.

- Proposal 5: If UE is rejected by the satellite during initial access procedure due to S&F reason, it can still try to camp on other normal cell or satellite if available before the wait timer expired.

Data transmission:

- In the RAN2#125bis meeting, RAN2 gave a conclusion on CP and UP solution as “The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)” [3]. About the UP solution, SA2 has already made the following conclusion:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

3)	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

- From the conclusion, we can see that SA2 has agreed that legacy CP and UP solutions can be used for data traffic in S&F operation

- Observation 2: SA2 has confirmed that legacy CP and UP procedure can be used for data traffic in S&F operation.

- Thus, from RAN2 point of view, it is nature to follow SA2 conclusion that legacy CP and UP solution can be reused.

- Proposal 6: It is proposed that RAN2 confirm legacy CP and UP solution can be reused for data traffic in S&F operation.

- In the last RAN2#127 meeting, RAN2 has made an agreement that RAN2 will consider both MO and MT data within scope, it’s natural to support CIoT EPS optimisations and from Proposal 7, both CP and UP solution are supported. Thus when the satellite is moving away and makes service link unavailable, RRC connection suspend procedure in TS 36.300 as below [4] can be reused.

- Figure 1: RRC Connection Suspend procedure when the satellite is moving away

- During the initial access procedure, the wait timer is provided by the MME using rejection message to tell the UE how long should it wait for the next/current satellite coming back. Thus, in our opinion, the similar method can be also used for RRC connection state. That means when the satellite is moving away (the service link becomes available), eNB releases RRC connection using the legacy procedure with adding a new resumeTime IE in the RRCConnectionRelease message. In this way, UE can know how long it should wait until next/current satellite comes back and do not need to receive unnecessary paging messages to save power.

- Figure 2: RRC Connection Suspend procedure in S&F operation when the satellite is moving away

Proposal 7：RRCConnection suspend/resume procedure can be reused in S&F operation

Proposal 8：It is proposed to add a new IE to notify UE about the wait timer in RRCConnectionRelease message (e.g. resumeTime).

Conclusions

Observation 1: In both MME-split and full CN onboard architecture, legacy UE can not complete the initial access procedure in the cell within S&F operation.

- Observation 2: SA2 has confirmed that legacy CP and UP procedure can be used for data traffic in S&F operation.

- Access control

Proposal 1: For multi operation transition scenario, satellite can send all indication information of every S&F operation switching in SIB message at once.

Proposal 2: If Proposal 1 is agreed, the satellite only needs to broadcast the changed S&F operation information (e.g. starting time, duration time).

Proposal 3: If the satellite is currently operating in normal operation and change to S&F operation, then legacy UE which already finished initial access procedure with the satellite may access the cell in S&F operation.

Proposal 4: Legacy UE can work in the cell in S&F operation at least in full CN onboard structure. FFS further enhancement is needed for DL transmission in MME-split structure when there is no new NAS data received.

- Proposal 5: If UE is rejected by the satellite during initial access procedure due to S&F reason, it can still try to camp on other normal cell or satellite if available before the wait timer expired.

- Data transmission

- Proposal 6: It is proposed that RAN2 confirm legacy CP and UP solution can be reused for data traffic in S&F operation.

Proposal 7：RRCConnection suspend/resume procedure can be reused in S&F operation

Proposal 8：It is proposed to add a new IE to notify UE about the wait timer in RRCConnectionRelease message (e.g. resumeTime).

References

- 3GPP Chair_Notes, R2-127 NR-NTN-IoT-NTN (Sergio) EOM

- 3GPP TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

- 3GPP Chair_Notes, R2-125bis NR-NTN-IoT-NTN (Sergio) EOM

- 3GPP TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage2 (Release 18)