3GPP TSG-RAN WG2 Meeting #127bis	R2-2408070

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.3

Source:	CMCC

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion

Introduction

At the RAN2#127 meeting we reached the following agreements [1]:

RAN2 #127: R2-2407571 Agreements on L1 MR event evaluation - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. Agreements on L1 measurement reporting - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

At the RAN #105 meeting, it was agreed to support conditional Intra-CU LTM [2]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Discussion

Event for conditional LTM

Based on the agreements of RAN2#126 and RAN2#127, beam level events Event LTM 2/3/4/5 are supported, but Event LTM1 is not supported.

RAN2#126:

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events:

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

RAN2#127:

- Event LTM1 is not defined.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

In traditional LTM handover, the UE reports the L1 measurement results and the network makes the handover decision. In this case, when a candidate beam triggers a measurement report, the network does not immediately make a handover decision based on this measurement report alone. In other words, we can assume that the network has a whole picture of the UE's serving cell and candidate cell beams, and the network will make reliable handover decisions based on this.

This is different for conditional LTM, where the handover decision is made by the UE itself, and it is difficult for the UE to evaluate the overall quality of the candidate cell based on beam level measurements. Specifically, if only beam level events are considered, it may cause a ping-pong problem between two Cus. Therefore, cell level Event LTM3a/5a should be introduced:

- Event LTM3a: Candidate cell becomes amount of offset better than serving cell;

- Event LTM5a: Serving cell becomes worse than absolute threshold1 AND candidate cell becomes better than another absolute threshold2.

And there are two options for defining cell level measurement results:

- Option 1: Similar to L3 measurements, beam specific measurements are consolidated to derive cell quality;

- Option 2: N beams all meet the corresponding thresholds, and then Event LTM3a/4a/5a is considered to be satisfied.

Proposal 1: To avoid potential ping-pong handover for conditional LTM, following cell level Event LTM3a/5a are introduced:

- Event LTM3a: Candidate cell becomes amount of offset better than serving cell;

- Event LTM5a: Serving cell becomes worse than absolute threshold1 AND candidate cell becomes better than another absolute threshold2.

Proposal 2: There are two options for defining cell level measurement results::

- Option 1: Similar to L3 measurements, beam specific measurements are consolidated to derive cell quality;

- Option 2: A configurable number of beams meet the corresponding thresholds, and then Event LTM3a/4a/5a is considered to be satisfied.

Triggering for sending the measurement report

Cell level TTT or beam level TTT for an event evaluation?

If it is beam level TTT, for Event LTM2/4, the UE should maintain an independent timer for each beam in the serving cell and the candidate cell; for Event LTM3/5, the UE should also maintain an independent timer for the compared beam pairs.

If it is cell level TTT, for Event LTM2/4, the UE maintains only one timer for the serving cell or candidate cell; for Event LTM3/5, the UE maintains only one timer for all beams in the same candidate cell. In this case, there are two problems: first, the current beam of the serving cell may change during the TTT duration; second, assuming a beam meets event and TTT starts, during TTT if another beam that belongs to the same candidate cell meets the event, what will happen?

Proposal 3: RAN2 discusses whether to use cell level TTT or beam level TTT.

If it is beam level TTT, beam-level measurement results may change rapidly, and multiple RSs may meet event triggering conditions within the TTT time. When this happens, triggering a measurement report for each event will result in a very large resource overhead, so only one measurement report should be triggered at this time. This measurement report can carry all measurement results and triggering event information.

Proposal 4: For beam level TTT, if multiple RSs meet the event conditions at the same time, only one L1 measurement report will be triggered.

It is necessary to consider the UE behavior for cell level TTT, that is, whether the UE should reset the TTT timer and which beam will be carried in the measurement report.

For cell level TTT, when the current beam of serving cell changes, there are two options:

Option 1: If the TTT timer is reset when the current beam of the serving cell changes, it will increase the difficulty of triggering the event, and the NW may not be able to obtain the L1 measurement results in time to trigger the LTM cell handover.

Option 2: Another option is the TTT timer is not reset. Actually, the purpose of event triggering is to send the measurement results to the NW when the beam quality of the serving cell deteriorates and/or the beam quality of the candidate cell is better than the beam quality of the serving cell. Therefore, it does not matter which beam is the current beam of the serving cell, so there is no need to reset the TTT timer when the current beam of the serving cell changes.

Proposal 5: For cell level TTT, TTT timer is not reset when the current beam of serving cell changes.

Proposal 6: When the current beam of serving cell changes, the UE carries the latest current beam measurement information in the measurement report.

Similarly, for cell level TTT, if another beam that belongs to the same candidate cell meets the event during TTT and the original beam no longer meets the event, TTT timer is not reset.

Proposal 7: For cell level TTT, TTT timer is not reset when the beam of candidate cell changes.

RS related issue

RAN2#127 meeting agreed that current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. However, in the conclusion of RAN1#118 meeting, best beam is still under discussion:

Agreement    RAN1#118

- For the identification of the serving cell RS for event evaluation,

  - At least the following options are further studied in RAN1, where different options could apply to different LTM event

    - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell

    - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell

      - QCL RS or SSB is configured by the network

    - Option. 3: Measurement RS(s) is/are explicitly configured

    - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality.

    - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states

In our opinion, RAN2's conclusion corresponds to Option 1 or Option 2. Besides, RAN2#126 made a working assumption that “same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.” Since different RS type are transmitted with different beamwidth and/or different transmission power, then, it is reasonable to have same RS type for RS measurement for serving and candidate cell, and the comparison between serving and candidate cell is fair.

So for SSB-based events, if the indicated serving beam is CSI-RS/TRS, the UE need to uses the SSB of the serving cell QCLed with the indicated CSI-RS/TRS for event evaluation. Based on the above discussion, for the identification of the serving cell RS for event evaluation, RAN1's ​​option 2 that “derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell” meets the discussion assumptions of RAN2.

Proposal 8: For the identification of the serving cell RS for event evaluation, RAN1's ​​option 2 that “derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell” could be supported, and send a LS to RAN1.

LTM measurement configuration

At the RAN2#127 meeting agreed the following conclusions:

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

Considering the L3 measurement configuration can be carried in LTM-config (e.g. in reference config) and shared across all candidate cells, it can be discussed whether LTM event configuration can be shared among all candidate cells. If L1 event triggered report reference config is carried in LTM-Config, the network only needs to send some specific parameters after LTM cell switch, and does not need to send the complete configuration in RRC-Reconfiguration message.

Proposal 9: RAN2 discuss whether introduce L1 event triggered report reference config carried in LTM-Config.

Conclusions

In this contribution, we analyse for measurement event evaluation, following are the proposals.

Proposals:

Proposal 1: To avoid potential ping-pong handover for conditional LTM, following cell level Event LTM3a/5a are introduced:

- Event LTM3a: Candidate cell becomes amount of offset better than serving cell;

- Event LTM5a: Serving cell becomes worse than absolute threshold1 AND candidate cell becomes better than another absolute threshold2.

Proposal 2: There are two options for defining cell level measurement results::

- Option 1: Similar to L3 measurements, beam specific measurements are consolidated to derive cell quality;

- Option 2: A configurable number of beams meet the corresponding thresholds, and then Event LTM3a/4a/5a is considered to be satisfied.

Proposal 3: RAN2 discusses whether to use cell level TTT or beam level TTT.

Proposal 4: For beam level TTT, if multiple RSs meet the event conditions at the same time, only one L1 measurement report will be triggered.

Proposal 5: For cell level TTT, TTT timer is not reset when the current beam of serving cell changes.

Proposal 6: When the current beam of serving cell changes, the UE carries the latest current beam measurement information in the measurement report.

Proposal 7: For cell level TTT, TTT timer is not reset when the beam of candidate cell changes.

Proposal 8: For the identification of the serving cell RS for event evaluation, RAN1's ​​option 2 that “derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell” could be supported, and send a LS to RAN1.

Proposal 9: RAN2 discuss whether introduce L1 event triggered report reference config carried in LTM-Config.

References

- R2-2407571 Report from session on V2X/SL, R18/19 MOB, and R19 NES.

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4.