3GPP TSG-RAN WG2 Meeting #127bis	R2-2408073

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.3.2

Source:	CMCC

Title:	Simulation results for RRM measurement prediction

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In the previous RAN2 meeting, there were discussion on Simulation assumptions and evaluation methodology for RRM measurement prediction. The agreements about simulation assumptions for RRM measurement prediction were achieved [1][2][3] with the following agreed evaluation scenarios to start the relevant simulations.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we would provide our simulation results of two high priority evaluation cases for RRM measurement prediction with agreed simulation assumptions, including Case 2-FR1 to FR1 intra-frequency temporal domain case B and Case 4-FR2 to FR2 intra-frequency temporal domain case A. And we also provide our consideration on Evaluation metrics/KPIs for frequency domain prediction and handover performance.

# 2 Discussion

2.1 Simulation assumptions of the evaluation scenario

2.1.1 Evaluation scenarios

Regarding to the study on AI/ML for mobility in NR, there are two main study goals of evaluation on AI/ML for mobility.

- 1st study goal of evaluation is to reduce measurement overhead

- 2nd study goal of evaluation is to enhance handover performance

As show above, Case2 that FR1 to FR1 intra-frequency temporal domain case B focus on Intra-cell scenario and the study goal is to reduce measurement overhead. As per the definition of measurement reduction in agreements on simulation assumptions in RAN2#126 meeting [2], Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances:

MRRT= skipped measurement time instances / total measurement time instances

For Case4 that FR2 to FR2 intra-frequency temporal domain case A, it focus on Intra-cell scenario and the study goal is to enhance handover performance. As per the agreements for the evaluation exercise for 2nd study goal, there is no reduction in measurement overhead in this case.

For temporal domain prediction, there are Case A, Case B and Case B+ defined for the AI-PHY beam management in TR 38.843[4]. Here are the examples illustrated for Case A and Case B.

Figure 1: Example for Case A in [4]

Figure 2: Example for Case B in [4]

2.1.2 RRM sub case

Based on the agreements in RAN2#125bis meeting[1], there are at least the following RRM sub cases for cell level measurement prediction model:

- RRM sub case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

- RRM sub case 2: To directly predict cell level results based on cell level results;

- RRM sub case 3: To directly predict cell level results based on beam level results.

In our simulation, we use RRM sub case 2 as starting point. The measurement results of L1 beam level RSRP are uses to derive the measurement results of L3 cell level RSRP and then the measurements of L3 cell level RSRP are used as AI/ML model input to predict L3 cell level RSRP results.

Figure 3: RRM sub case 2

2.1.3 L1/L3 filtering

During RAN2#127 meeting, RAN2 concluded that there are two L1/L3 filtering options as shown below that could be used in the simulations of RRM measurement prediction and up to company to report. In sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every sample period. In non-sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every measurement period.

Figure 4: Sliding L1/L3 filtering [5]

Figure 5: Non-sliding L1/L3 filtering [5]

In our simulation, we use non-sliding L1/L3 filtering option in Case 2-FR1 to FR1 intra-frequency temporal domain case B, and use sliding L1/L3 filtering option in Case 4-FR2 to FR2 intra-frequency temporal domain case A.

2.2 Simulation results and analysis

2.2.1 Case 2-FR1 to FR1 intra-frequency temporal domain case B

In our evaluation of case B, we use temporal domain case example 2 as shown below and simulate the following two sub-use cases with the simulation assumptions in Table 1 and Table 2:

- Sub-case 1 in case 2: to compare the RSRP difference at different speeds with the same MRRT

- Sub-case 2 in case 2: to compare the RSRP difference at different MRRT with the same UE speed

Figure 6: Temporal domain case example 1 [6]

Figure 7: Temporal domain case example 2 [6]

Table 1: The simulation parameters of Sub-cases in Case 2

| Case 2 | OW vs PW | MRRT | UE Speed |
| --- | --- | --- | --- |
| Sub-case 1 | N:N | 1/2 | - 30km/h - 60km/h - 90km/h |
| Sub-case 2 | N:N N:2N N:3N N:4N | - 1/2 - 2/3 - 3/4 - 4/5 | 90km/h |

Table 2: The key simulation assumptions in Case 2

| Data Size (Number of Samples) | Training/validity | 13000,1625 |
| --- | --- | --- |
|  | Testing | 1625 |
| AI/ML model input/output | Model input | Cell level RSRP of all cells |
|  | Model output | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |
|  | Model complexity in a number of parameters(M) | 24181 |
|  | Model complexity in model size (e.g. Mbyte) | 0.093MB |
|  | Computational complexity [FLOPs] | 292.66K |

Sub-case 1 in case 2: to compare the RSRP difference at different speeds with the same MRRT

The simulation results of Sub-case 1 in Case 2-FR1 to FR1 intra-frequency temporal domain case B is shown in Table 3. The more details of the simulation outcomes are captured in the Spreadsheet1 for Scenario2 RRM Measurement Prediction Evaluation results for case B.

Table 3: The simulation results of Sub-case 1 in Case B

| Set No. | UE Speed | OW vs PW | MRRT | RSRP-Differ (dB) |
| --- | --- | --- | --- | --- |
| 1 | 30 km/h | N:N | 1/2 | 0.330 |
| 2 | 60 km/h | N:N | 1/2 | 0.602 |
| 3 | 90 km/h | N:N | 1/2 | 0.877 |

Based on the above simulation results, it could be observed that in the case that MRRT is 1/2 (OW vs PW=N:N), the average L3 cell level RSRP difference increases from 0.330 dB to 0.877 dB with the increase of UE speed from 30 km/h to 90 km/h. And the prediction accuracy for FR1-to-FR1 intra-frequency temporal domain case B reduces as the increase of UE speed with the same MRRT. But although UE speed is high (90km/h), the RSRP difference is kept less than 1dB.

Observation 1: In our simulation, in the case that MRRT is 1/2 (OW vs PW=N:N), the average L3 cell level RSRP difference increases from 0.330 dB to 0.877 dB with the increase of UE speed from 30 km/h to 90 km/h.

Observation 2: With the same MRRT, the prediction accuracy for FR1-to-FR1 intra-frequency temporal domain case B reduces as the increase of UE speed.

Observation 3: In the case that MRRT is 1/2, although UE speed is high (90km/h) the RSRP difference is kept less than 1dB.

Sub-case 2 in case 2: to compare the RSRP difference at different MRRT with the same UE speed

The simulation results of Sub-case 2 in Case 2-FR1 to FR1 intra-frequency temporal domain case B is shown in Table 4. The more details of the simulation outcomes are captured in the Spreadsheet1 for Scenario2 RRM Measurement Prediction Evaluation results for case B.

Table 4: The simulation results of Sub-case 2 in Case B

| Set No. | UE Speed | OW vs PW | MRRT | RSRP-Differ (dB) |
| --- | --- | --- | --- | --- |
| 1 | 90 km/h | N:N | 1/2 | 0.877 |
| 2 | 90 km/h | N:2N | 2/3 | 1.344 |
| 3 | 90 km/h | N:3N | 3/4 | 1.754 |
| 4 | 90 km/h | N:4N | 4/5 | 2.133 |

Based on the above simulation results, it could be observed that in the case that UE speed is 90 km/h, the average L3 cell level RSRP difference increases from 0.877dB to 2.133dB with the increase of MRRT from 1/2 to 4/5. And the prediction accuracy for FR1-to-FR1 intra-frequency temporal domain case B reduces as the increase of MRRT with the same UE speed. Moreover, when the MRRT is up to 2/3 and above, the RSRP difference is more than 1dB, and even up to 2.133dB with MRRT is 4/5. That means the MRRT (OW vs PW) should be set in an appropriate range to ensure the RRM measurement prediction accuracy for temporal domain case B.

Observation 4: In our simulation, in the case that UE speed is 90 km/h, the average L3 cell level RSRP difference increases from 0.877dB to 2.133dB with the increase of MRRT from 1/2 to 4/5.

Observation 5: With the same UE speed, the prediction accuracy for FR1-to-FR1 intra-frequency temporal domain case B reduces as the increase of MRRT.

Observation 6: When the MRRT is up to 2/3 and above, the RSRP difference is more than 1dB, and even up to 2.133dB with MRRT is 4/5.

Proposal 1: For temporal domain case B, the MRRT (OW vs PW) should be set in an appropriate range to ensure the RRM measurement prediction accuracy.

2.2.2 Case 4-FR2 to FR2 intra-frequency temporal domain case A

In our evaluation, we simulate the following two sub-use cases with the simulation assumptions in Table 5 and Table 6:

- Sub-case 1 in case 4: to compare the RSRP difference at different speeds with the same Observation window vs Prediction window

- Sub-case 2 in case 4: to compare the RSRP difference at different Observation window vs Prediction window with the same UE speed

Table 5: The simulation parameters of Sub-cases in Case 4

| Case 4 | OW vs PW | UE Speed |
| --- | --- | --- |
| Sub-case 1 | 3N:N | - 60km/h - 90km/h - 120km/h |
| Sub-case 2 | - 3N:N - 2N:N - N:N - 2N:2N - 3N:3N | 90km/h |

Table 6: The key simulation assumptions in Case 4

| Data Size (Number of Samples) | Training/validity | 208656,23184 |
| --- | --- | --- |
|  | Testing | 33810 |
| AI/ML model input/output | Model input | Cell level RSRP of all cells |
|  | Model output | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 868,560 |
|  | Model complexity in model size (e.g. Mbyte) | 10.5MB |
|  | Computational complexity [FLOPs] | 1300.08K |

Sub-case 1 in case 4: to compare the RSRP difference at different speeds with the same Observation window vs Prediction window

The simulation results of Sub-case 1 in Case 4-FR2 to FR2 intra-frequency temporal domain case A is shown in Table 7. The more details of the simulation outcomes are captured in the Spreadsheet2 for Scenario4 RRM Measurement Prediction Evaluation results for case A.

Table 7: The simulation results of Sub-case 1 in Case A

| Set No. | UE Speed | OW vs PW | RSRP-Differ (dB) |
| --- | --- | --- | --- |
| 1 | 60 km/h | 3N:N | 0.405 |
| 2 | 90 km/h | 3N:N | 0.487 |
| 3 | 120 km/h | 3N:N | 0.634 |

Based on the above simulation results, it could be observed that in the case of the same observation window and prediction window, the average L3 cell level RSRP difference increases from 0.405dB to 0.634dB with the increase of UE speed from 60 km/h to 120 km/h. And the prediction accuracy for FR2-to-FR2 intra-frequency temporal domain case A reduces as the increase of UE speed with the same observation window and prediction window. But although UE speed is high (120km/h), the RSRP difference is less than 1dB.

Observation 7: In our simulation, in the case of the same observation window and prediction window for temporal domain case A, the average L3 cell level RSRP difference increases from 0.405dB to 0.634dB with the increase of UE speed from 60 km/h to 120 km/h.

Observation 8: With the same observation window and prediction window, the prediction accuracy for FR2-to-FR2 intra-frequency temporal domain case A reduces as the increase of UE speed.

Observation 9: In the case where OW vs PW is 3N:N, although UE speed is high (120km/h), the RSRP difference is less than 1dB.

Sub-case 2 in case 4: to compare the RSRP difference at different Observation window vs Prediction window with the same UE speed

The simulation results of Sub-case 2 in Case 4-FR2 to FR2 intra-frequency temporal domain case A is shown in Table 8. The more details of the simulation outcomes are captured in the Spreadsheet2 for Scenario4 RRM Measurement Prediction Evaluation results for case A.

Table 8: The simulation results of Sub-case 2 in Case A

| Set No. | UE Speed | OW vs PW | RSRP-Differ (dB) |
| --- | --- | --- | --- |
| 1 | 90 km/h | 3N:N | 0.487 |
| 2 | 90 km/h | 2N:N | 0.601 |
| 3 | 90 km/h | N:N | 0.949 |
| 4 | 90 km/h | 2N:2N | 0.855 |
| 5 | 90 km/h | 3N:3N | 0.947 |

Based on the above simulation results, it could be observed that with the same prediction window, the average L3 cell level RSRP difference increases from 0.487dB to 0.949dB with the decrease of the observation window. And the prediction accuracy for FR2-to-FR2 intra-frequency temporal domain case A reduces as the decrease of the observation window. That means it is helpful to ensure the prediction accuracy with the more measurement results in observation window are used as the inputs of AI/ML model in the case with the same prediction window. Moreover, it could also be observed that with the increase of prediction window from N to 3N and the observation window increases accordingly, the average L3 cell level RSRP difference is close.

Observation 10: In our simulation, with the same prediction window, the average L3 cell level RSRP difference increases from 0.487dB to 0.949dB with the decrease of the observation window from 3N to N.

Observation 11: With the same prediction window, the prediction accuracy for FR2-to-FR2 intra-frequency temporal domain case A reduces as the decrease of the observation window.

Observation 12: With the increase of prediction window from N to 3N and the observation window increases accordingly, the average L3 cell level RSRP difference is close.

Proposal 2: It is helpful to ensure the prediction accuracy with the more measurement results in observation window are used as the inputs of AI/ML model in the case with the same prediction window.

Proposal 3: For temporal domain case A, the OW vs PW should be set in an appropriate range to ensure the RRM measurement prediction accuracy.

2.3 Evaluation metrics/KPIs

Similar to Top-1 beam prediction accuracy for beam management use case, we think the best cell prediction accuracy can be also considered as a metric for the prediction accuracy of cell level measurement prediction, which refers to the percentage of "the actual best cell based on the measurement results is the best cell based on the predicted results", because the network would select one target cell for handover procedure.

Proposal 4: The best cell prediction accuracy can be considered as a metric for the prediction accuracy of cell level measurement prediction.

- Best Cell prediction accuracy (%): the percentage of "the actual best cell based on the measurement results is the best cell based on the predicted results"

In RAN2#126 meeting, RAN2 agreed that we will start the evaluation with measurement prediction accuracy and model complexity, and we can discuss system performance after we see which scenarios have good measurements prediction accuracy. However, we think that system performance is also important for evaluation. The target study goal of the agreed evaluation scenarios 2, 3, 5, 6 is to reduce the measurement overhead. But we think the measurement reduction should keep the mobility performance from degrading, for example, to keep the HO successful rate above a threshold (e.g. 95%) or keep the degrading of HO successful rate below a threshold (e.g. 3%).

Observation 13: The measurement reduction should keep the mobility performance from degrading, for example, to keep the HO successful rate above a threshold (e.g. 95%) or keep the degrading of HO successful rate below a threshold (e.g. 3%).

In addition, we understand  that even if the measurement prediction accuracy is less than optimal (e.g. RSRP difference to the actual measurement is larger than 2dBm, where the threshold may be defined by RAN4), the handover performance can still be satisfactory, for example when the rank of cells based on cell quality is correct.

On the other hand, the target study goal for FR2-FR2 prediction is to enhance handover performance. Thus, it is straightforward to evaluate the handover performance in this scenario, which cannot be reflected by the measurements prediction accuracy directly.

Therefore, we think that the handover performance should be evaluated. As listed in SID [7], Ping-pong HO, HOF/RLF, Time of stay, Handover interruption etc., can be considered. In addition, the HO successful rate can be also considered. And RAN2 can discuss the baseline threshold for evaluation.

Proposal 5: RAN2 consider the performance metrics/KPIs (e.g. Ping-pong HO rate, HO failure rate, RLF rate, short stay rate, HO successful rate), and discuss the baseline threshold for evaluation, such as the HO successful rate above a threshold (e.g. 95%) or the degrading of HO successful rate below a threshold (e.g. 3%).

Furthermore, the existing Handover Failure Modelling, RLF modelling and Ping-pong Modelling in TR 36.839 [8] can be reused as the baseline model for statistic of Handover failure rate, RLF rate, Ping-Pong HO rate and short stay rate, etc.

Proposal 6: The existing Handover Failure Modelling, RLF modelling and Ping-pong Modelling in TR 36.839 can be reused as the baseline model for statistic of the system level performance.

# 3 Conclusion

In this paper, we provide our simulation results of Case 2-FR1 to FR1 intra-frequency temporal domain case B and Case 4-FR2 to FR2 intra-frequency temporal domain case A, and the consideration on Evaluation metrics/KPIs for frequency domain prediction and handover performance. We have the following observations and proposals:

Case 2-FR1 to FR1 intra-frequency temporal domain case B

Sub-case 1 in case 2: to compare the RSRP difference at different speeds with the same MRRT

Observation 1: In our simulation, in the case that MRRT is 1/2 (OW vs PW=N:N), the average L3 cell level RSRP difference increases from 0.330 dB to 0.877 dB with the increase of UE speed from 30 km/h to 90 km/h.

Observation 2: With the same MRRT, the prediction accuracy for FR1-to-FR1 intra-frequency temporal domain case B reduces as the increase of UE speed.

Observation 3: In the case that MRRT is 1/2, although UE speed is high (90km/h) the RSRP difference is kept less than 1dB.

Sub-case 2 in case 2: to compare the RSRP difference at different MRRT with the same UE speed

Observation 4: In our simulation, in the case that UE speed is 90 km/h, the average L3 cell level RSRP difference increases from 0.877dB to 2.133dB with the increase of MRRT from 1/2 to 4/5.

Observation 5: With the same UE speed, the prediction accuracy for FR1-to-FR1 intra-frequency temporal domain case B reduces as the increase of MRRT.

Observation 6: When the MRRT is up to 2/3 and above, the RSRP difference is more than 1dB, and even up to 2.133dB with MRRT is 4/5.

Proposal 1: For temporal domain case B, the MRRT (OW vs PW) should be set in an appropriate range to ensure the RRM measurement prediction accuracy.

Case 4-FR2 to FR2 intra-frequency temporal domain case A

Sub-case 1 in case 4: to compare the RSRP difference at different speeds with the same Observation window vs Prediction window

Observation 7: In our simulation, in the case of the same observation window and prediction window for temporal domain case A, the average L3 cell level RSRP difference increases from 0.405dB to 0.634dB with the increase of UE speed from 60 km/h to 120 km/h.

Observation 8: With the same observation window and prediction window, the prediction accuracy for FR2-to-FR2 intra-frequency temporal domain case A reduces as the increase of UE speed.

Observation 9: In the case where OW vs PW is 3N:N, although UE speed is high (120km/h), the RSRP difference is less than 1dB.

Sub-case 2 in case 4: to compare the RSRP difference at different Observation window vs Prediction window with the same UE speed

Observation 10: In our simulation, with the same prediction window, the average L3 cell level RSRP difference increases from 0.487dB to 0.949dB with the decrease of the observation window from 3N to N.

Observation 11: With the same prediction window, the prediction accuracy for FR2-to-FR2 intra-frequency temporal domain case A reduces as the decrease of the observation window.

Observation 12: With the increase of prediction window from N to 3N and the observation window increases accordingly, the average L3 cell level RSRP difference is close.

Proposal 2: It is helpful to ensure the prediction accuracy with the more measurement results in observation window are used as the inputs of AI/ML model in the case with the same prediction window.

Proposal 3: For temporal domain case A, the OW vs PW should be set in an appropriate range to ensure the RRM measurement prediction accuracy.

Evaluation metrics/KPIs

Proposal 4: The best cell prediction accuracy can be considered as a metric for the prediction accuracy of cell level measurement prediction.

- Best Cell prediction accuracy (%): the percentage of "the actual best cell based on the measurement results is the best cell based on the predicted results"

Observation 13: The measurement reduction should keep the mobility performance from degrading, for example, to keep the HO successful rate above a threshold (e.g. 95%) or keep the degrading of HO successful rate below a threshold (e.g. 3%).

Proposal 5: RAN2 consider the performance metrics/KPIs (e.g. Ping-pong HO rate, HO failure rate, RLF rate, short stay rate, HO successful rate), and discuss the baseline threshold for evaluation, such as the HO successful rate above a threshold (e.g. 95%) or the degrading of HO successful rate below a threshold (e.g. 3%).

Proposal 6: The existing Handover Failure Modelling, RLF modelling and Ping-pong Modelling in TR 36.839 can be reused as the baseline model for statistic of the system level performance.

# Reference

- R2_125bis_ChairNotes_24-04-19_final, Apr 2024

- R2_126_ChairNotes_24-05-24_final, May 2024

- R2_127_ChairNotes_24-08-23_17-00_eom, Aug 2024

- TR 38.843: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2407781, Summary of offline [AT127][026][AI Mob] Simulation assumptions, OPPO

- R2-2407885, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions-phase 2, OPPO

- RP-234055, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR 36.839 Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks