3GPP TSG-RAN WG2 Meeting #127bis	R2-2408074

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.7.3

Source:	CMCC

Title:	Discussion on RRM measurement gaps enhancements

Document for:	Discussion and Decision

# 1 Introduction

In RAN #102 meeting, the new WID on XR (eXtended Reality) for NR Phase 3 was approved. According to the WID, RRM measurement gaps/restrictions related enhancements for XR was approved [1] with the following objectives:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This is the first meeting for the discussion on RRM measurement gaps/restrictions enhancements for XR in RAN2. In this contribution, we would provide our initial consideration on RAN2 impacts from the solutions considered by RAN1/RAN4.

# 2 Discussion

2.1 RAN1/RAN4 progress

According to our observations, RAN1 discussions on RRM measurement gaps/restrictions enhancements mainly focus on enabling TX/RX for XR during RRM measurements and UE assistance information related to measurements occasions.

For enabling TX/RX for XR during RRM measurements, the latest agreements achieved in RAN1#118 meeting [2] are duplicated as following.

For solutions based on triggering/enabling by network signalling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

It can be seen that, for solutions based on triggering/enabling by network signaling, explicit indication by DCI (1 bit) is used to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

For UE assistance information related to measurements occasions, there is no consensus in RAN1 and the conclusion in RAN1#117 meeting [3] as shown below. RAN1 leaves the discussion on UE assistance information to other working groups and will trigger further work when needed.

Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

RAN4 starts the discussion on RRM requirements for XR_Ph3 in RAN4#112 meeting, the consideration on need/feasibility of UE assistance information has been included in the way forward of RAN4 [4]. In our opinion, RAN2 could simultaneously start the discussion on whether or not to introduce UE assistance information related to measurement occasions.

2.2 Potential RAN2 impacts

Based on the above observations about the progress on the solutions considered by RAN1 and RAN4, we think currently RAN2 discussion could mainly focus on the UE assistance information related to measurement occasions. So we have the following proposal:

Proposal 1: As starting point for RRM measurement gaps/restrictions enhancements for XR, RAN2 to discuss whether UE assistance information related to measurement occasions is needed and the corresponding solutions.

RRM measurements include intra-frequency measurement and inter-frequency measurement. For inter-frequency measurement, it is usually required to configure measurement gaps for a UE because the UE may need to perform RF tuning to measure the SSB of the target cell. For intra-frequency measurement, UE also needs measurement gap (MG) when SSBs to be measured is not within the active BWP of UE. The configuration of MG consists of measurement gap length (MGL), measurement gap repetition period (MGRP), and gap offset. During the measurement gaps, UE is not required to conduct reception/transmission from/to the serving cells except the reception of signals used for RRM measurement(s) and the signals used for random access procedure.

For inter/intra-frequency RRM measurement without measurement gap, there are restrictions on the scheduling availability. When any of the conditions about performing measurements in TDD bands on FR1, performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1, performing measurements on FR2 is met, there are restrictions on the scheduling availability. In detail, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on certain symbols, e.g. SSB symbols to be measured or  all symbols within SMTC window duration.

The periodicity of MG/SMTC is integer, but the periodicity of XR traffic is non-integer, e.g. 16.67ms for 60fps, and the XR packet arrival is subject to a random jitter. Due to the misalignment of periodicities, the arrival time of XR packets may collide with the MG/SMTC window with scheduling restrictions, resulting in transmission/reception delay of the XR packet and decrease in XR capacity. To minimize the impact of RRM measurements on XR capacity, enhancement mechanisms should be specified to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. That is the motivation for enhancements on RRM measurement gaps/restrictions for XR.

From the perspective of gNB, although the MG/SMTC pattern is configured to a UE, the gNB can skip some MGs/SMTC windows through network signalling, such that the TX/RX for XR is enabled in these time durations. For example, when the DL traffic load is high and the network has a high demand for data, the network could require the UE to receive the scheduled data rather than perform measurement during some specific MGs/SMTC windows. During the other MGs/SMTC windows, the UE is still required to perform measurements. Based on RAN1 agreements, it can be seen that RAN1 has agreed using explicit indication by DCI (1 bit) to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

The measurement behavior is up to the UE implementation/capability and gNB could not know, e.g., how many gaps are required or which SSBs will be used to satisfy the RRM measurement requirement. For a UE with advanced processing capability or good channel quality, the number of gaps to be used in the time period for RRM measurement may be less than the number configured/specified. For instance, the existing measurement delay requirements in RAN4 assume 5 samples for intra-frequency measurement and 8 samples for inter-frequency measurement. However, the delay requirements are minimum requirements, which means that with the better UE implementation it is feasible to use less samples to satisfy the measurement accuracy.

Observation 1: For a UE with advanced processing capability or good channel quality, the number of gaps to be used in the time period for RRM measurement may be less than the number configured/specified.

Observation 2: The measurement behavior is up to the UE implementation/capability and gNB could not get to know how many gaps are required or which SSBs will be used by UE to satisfy the RRM measurement requirement.

As we discussed above, some measurement occasions, e.g. measurement gap occasion or SMTC occasion, could not be used for measurement. But network has no information about how many gaps are required by UE for measurement accuracy or which gaps could be skipped for data scheduling. So, before the gNB decides to skip some MGs/SMTC windows, the assistance information about measurement behavior from UE is needed to help gNB make reasonable decisions.

Observation 3: Before the gNB decides to skip some MGs/SMTC windows, the assistance information about measurement behavior from UE is needed to help gNB make reasonable decisions.

From this point of view, UE assistance information (UAI) related to measurement occasions, e.g. the number of required measurement gaps/SMTC with restrictions within a certain time period for the unchanged measurement accuracy, could help network to configure indication to skip a measurement gap(s) /restriction(s). Taking intra-frequency measurement as an example, existing measurement period assumes 5 samples. If UE inform network by UAI that only two sample are needed to satisfy the measurement accuracy, then network could decide that 3 measurement occasions within existing measurement period can be skipped.

Observation 4: UE assistance information (UAI) related to measurement occasions, e.g. the number of required measurement gaps/SMTC with restrictions within a certain time period for the unchanged measurement accuracy, could help network to configure indication to skip a measurement gap(s) /restriction(s) to enable Tx/Rx.

Proposal 2: It is proposed to introduce UE assistance information (UAI) related to measurement occasions, e.g. the number of required measurement gaps/SMTC with restrictions within a certain time period for the unchanged measurement accuracy, which is beneficial in network indication to skip a measurement gap(s) /restriction(s) to enable Tx/Rx.

# 3 Conclusion

In this paper, we provide our initial consideration on RAN2 impacts from the solutions considered by RAN1/RAN4 for RRM measurement gaps/restrictions enhancements, we have the following observations and proposals:

Proposal 1: As starting point for RRM measurement gaps/restrictions enhancements for XR, RAN2 to discuss whether UE assistance information related to measurement occasions is needed and the corresponding solutions.

For UE assistance information

Observation 1: For a UE with advanced processing capability or good channel quality, the number of gaps to be used in the time period for RRM measurement may be less than the number configured/specified.

Observation 2: The measurement behavior is up to the UE implementation/capability and gNB could not get to know how many gaps are required or which SSBs will be used by UE to satisfy the RRM measurement requirement.

Observation 3: Before the gNB decides to skip some MGs/SMTC windows, the assistance information about measurement behavior from UE is needed to help gNB make reasonable decisions.

Observation 4: UE assistance information (UAI) related to measurement occasions, e.g. the number of required measurement gaps/SMTC with restrictions within a certain time period for the unchanged measurement accuracy, could help network to configure indication to skip a measurement gap(s) /restriction(s) to enable Tx/Rx.

Proposal 2: It is proposed to introduce UE assistance information (UAI) related to measurement occasions, e.g. the number of required measurement gaps/SMTC with restrictions within a certain time period for the unchanged measurement accuracy, which is beneficial in network indication to skip a measurement gap(s) /restriction(s) to enable Tx/Rx.

# Reference

- RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm (Rapporteurs).

- Draft_Minutes_report_RAN1#118_v030, Aug 2024.

- R1-2407511, Moderator summary #3 - Enabling TX/RX for XR during RRM measurements, Moderator (Nokia).

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia.