3GPP TSG-RAN WG2 Meeting #127bis                     	                                 R2-2408075

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.7.5

Source: 	CMCC

Title:  	Discussion on the RLC re-transmission related enhancements for XR

WID/SID:	         NR_XR_Ph3-Core

Document for:	Discussion and Decision

Introduction

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In the last RAN2 #127 meeting, the RLC enhancements has been discussed and the following related agreements have been approved.

In this contribution, we provide our view on the RLC re-transmission related enhancements.

Discussion

Avoidance of unnecessary re-transmission

In the last RAN2 meeting,  three approaches have been selected as the alternative options to enhance the RLC AM mechanism for avoidance of the unnecessary retransmissions:

Option 1: Tx initiated approach

Option 2: Rx initiated approach

Option 3: Combined Tx and Rx initiated approach

Approaches for avoidance of the unnecessary retransmissions

The comparison of these three options could be considered from three aspects:trigger mechanism, transmitting operations and receiving operation, including window handling in both side.

Option 1: Tx initiated approach

Trigger: Abandoning the retransmissions of an RLC SDU or a segment of RLC SDU in the transmission side of RLC AM could be triggered by the discard indications received from PDCP layer or a new timer expired in RLC layer.

Transmitting operations: The transmitting side of AM RLC entity stops to retransmit the obsolete RLC SDU or RLC SDU segments and sends report to the receiving side to indicate the RLC SN gap. To ensure that windows at Tx side and Rx side are aligned, the transmitting side moves its windows when the ACK of the obsolete RLC SDU or he RLC SDU segment is received from the receiving side in STATUS  report.

Receiving operations: When the receiving side of AM RLC entity receives the RLC SN gap report, it discards the buffered AMD PDUs or segments of AMD PDUs associated with SNs contained in RLC SN gap report. To simplify the reception procedure, the receiving side of AM RLC entity may tread the discarded AMD PDU or segment of AMD PDU as complete received AMD PDU in the reception buffer. The State variables and timers in receiving window should be updated according to the action as when an AMD PDU is placed in the reception buffer in legacy RLC specification [1].

Option 2: Rx initiated approach

Trigger: the discarding of AMD PDU in the Rx initiated approach could be triggered by a new timer.  The RLC timer could be aligned with the t-Reordering timer in PDCP. When the expiry of t-Reordering timer, the PDCP entity moves the receiving window forward and discards the PDUs having SN lower than RX_DELIV. Then the related PDU in RLC could be deleted accordingly by the new timer.

Receiving operations: The receiving side of RLC AM entity discards the obsolete SDUs and advance the receiving window. The receiving side of RLC AM entity informs the transmitting side with the STATUS report, where the SN of abandoned SDUs is set to ACK_SN. The transmitting side treats the discarded SDUs as complete received as legacy.

Transmitting operations: when the transmitting side is received with the STATUS report, the transmitting side moves the transmitting windows forward as legacy.

Option 3: Combined Tx and Rx initiated approach

Trigger: Abandoning the obsolete RLC SDUs in the transmission side of RLC AM could be triggered by the discard indications received from PDCP layer while the discarding of AMD PDU in the receiving side could be triggered by a new timer, like the t-Reordering in PDCP.

Receiving operations: The receiving side of RLC AM entity discards the obsolete AMD PDUs and moves its window.

Transmitting operations: the transmitting side moves its windows forward when discarding the the obsolete RLC SDUs.

The comparison of the approaches

As listed above, all the mechanisms for RLC AM enhancement could work to avoid the unnecessary re-transmission.

In Tx initiated approach, the UE as transmitting side of RLC AM entity could directly acquire the remaining time of an SDU from the upper layer in XR service. It is more natural and accurate that transmitting side of RLC AM entity considers the remaining time of an SDU and triggers the discard procedure timely.

Observation 1: In Tx initiated approach, it is more natural and accurate that transmitting side of RLC AM entity considers the remaining time of an SDU from the upper layer in XR service and triggers the discard procedure timely.

In Rx initiated approach, the UE as transmitting side has to wait the report sent from the gNB then stop the retransmissions of obsolete SDUs based on the received report, which could bring the delay. The gNB as receiver could perceive the remaining time below a threshold from DSR but it is not accurate. If the UE receive the discard report quite early from gNB e.g. based on the inaccurate reported remaining time, and then terminate the retransmission of obsolete SDUs which may cause more results in failure.

Observation 2: In Rx initiated approach, the gNB as receiver could perceive the remaining time below a threshold from DSR but it is not accurate. If the UE receives the discard report early from gNB e.g. based on the inaccurate reported remaining time, and then terminate the retransmission of obsolete SDUs which may cause more results in failure.

In Rx initiated approach, it is proposed that a timer may be introduced in RLC receiver based on the t-Reordering. In this way, RLC receiver needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. The RLC advances its receiving window according to the PDCP timer t-Reordering expires or the PDCP receiving window moves. This method seems complexity for RLC to maintain these timers between two layers (PDCP and RLC) concurrently. If the UE as transmitting side could not receive the discard report from gNB timely e.g. only based on the t-Reordering timer, the retransmissions of obsolete SDUs may not be stopped and introduce resource waste.

Observation 3: In Rx initiated approach, RLC receiving side needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. It seems to be complexity for RLC to maintain these timers between two layers (PDCP and RLC) concurrently.

In the combined Tx and Rx initiated approach, the transmitting side and receiving side move forward its window by its own maintained timer. As discussed above, the timer in transmitting side is configured based on the remaining time while the timer in receiving side takes account t-Reordering in PDCP. It seems to be difficult to sync these two timers and ensure the window moving forward aligned. In case that the transmission window moves early than the receiving window, it may lead the discarding of PDU in receiving side.

Observation 4: In the combined Tx and Rx initiated approach, the timer in transmitting side is configured based on the remaining time while the timer in receiving side takes account t-Reordering in PDCP. It seems to be difficult to sync these two timers and ensure the window moving aligned.

Therefore the Tx initiated approach may be more accurate and simple to avoid the unnecessary re-transmission in RLC AM mechanism.

Proposal 1: RAN2 to adopt the Tx initiated approach as RLC AM enhancement to avoid the unnecessary re-transmission.

Proposal 2: Abandoning the retransmissions of an RLC SDU in the transmission side of RLC AM could be triggered by the indications received from PDCP layer or a new timer expired in RLC.

Proposal 3: The SN gap report from transmitting side could be used to notify the receiving RLC side for the obsolete SDUs. The RLC SN gap report may include the indications for the sets of SN or possibly SN range associated with the abandoned retransmission of RLC SDUs.

Proposal 4: To ensure that windows at Tx side and Rx side are aligned, the transmitting side abandons the retransmission of an obsolete RLC SDU or an RLC SDU segment when the ACK of an obsolete RLC SDU or an RLC SDU segment is received from the receiving side.

Proposal 5: When the receiving side of AM RLC entity receives the RLC SN gap report, it discards the buffered AMD PDUs or segment of AMD PDUs associated with SNs contained in RLC SN gap reporting and considers the positively acknowledged associated with discarded RLC SDU when constructing the STATUS report.

The indication from RLC to upper layer indicates the RLC failure when max retransmission has been reached, which may lead Radio Link Failure (RLF) and bring the interruption [2]. If the RLC AM entity discards an obsolete RLC SDU and the maximum number of retransmissions is reached, the indication from RLC to upper layer, which indicates max retransmission has been reached as RLC failure, shall not be triggered due to the retransmissions discarding.

Proposal 6: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Timely RLC retransmission

In the last meeting,  the following solutions were proposed in RAN2 to achieve the timely retransmissions.

- Option 1: Retransmission based on enhanced status report

- Option 2: Retransmission based on enhanced polling

- Option 3: Autonomous retransmission

Solutions for timely RLC retransmission

- Option 1: Retransmission based on enhanced status report

When the RLC receiving side detects the transmission failure of an AMD PDU, it waits the timer t-Reassembly expires before triggering the status report, to ensure HARQ reordering. Besides, the RLC receiving side waits the timer t-StatusProhibit expires to ensure not frequently transmit the status report. Both timer could bring the delay of the transmission of status report.

If a new timer is introduced for early transmission of status report, which may not wait for the t-Reassembly expires, it may achieve the similar effort like blind early retransmission. For XR service, the data with low PDB could be transmitted in the particular RLC entity/LCH and it could depend on the network to configure timers of RLC entity properly, e.g. set smaller value for these timers e.g. t-Reassembly  and t-StatusProhibit  to reduce the delay caused by transmitting status report.

Proposal 7: It could depend on the network to configure the timers t-Reassembly  and t-StatusProhibit properly to reduce the delay caused by transmitting status report. It is not necessary to introduce the enhancement for status report.

- Option 2: Retransmission based on enhanced polling

The polling has to wait till the number of transmitted PDU without polling is larger than the pollPDU or the total data volume of transmitted PDU without polling is larger than pollByte, or the timer t-PollRetransmit expires before it is included in AMD PDU.

Firstly , reducing the transmission delay caused by timer t-PollRetransmit could be considered:

In [3] as below, the timer t-PollRetransmit is used by the transmitting side of an AM RLC entity in order to retransmit a poll. The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling. The additional values could be introduced for short PBD of XR traffic, including [1,2,3,4]in milliseconds value, which is simple way to implement the timely retransmission.

T-PollRetransmit ::= ENUMERATED { ms1,ms2,ms3,ms4,ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms105, ms110, ms115, ms120, ms125, ms130, ms135, ms140, ms145, ms150, ms155, ms160, ms165, ms170, ms175, ms180, ms185, ms190, ms195, ms200, ms205, ms210, ms215, ms220, ms225, ms230, ms235, ms240, ms245, ms250, ms300, ms350, ms400, ms450, ms500, ms800, ms1000, ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610, ms4-v1610, spare1}

Observation 5: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 8: The additional values of the timer t-PollRetransmit could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

Secondly, reducing the delay caused by the trigger mechanism could also be considered:

In legacy, when the total number of PDUs or the total data volume of PDUs is accumulated larger than the configured value pollPDU or  pollByte, the polling could be included in AMD PDU to request the status report from receiving side. In order to reduce the polling delay for the RLC SDUs with small remaining time value, the remaining time could be considered in the polling trigger mechanism. When the remaining time below a threshold for the transmitted PDU,  a new value pollPDU or pollByte could be used to fasten the polling trigger.

Proposal 9:The remaining time could be considered to enhance the polling trigger mechanism.

Conclusion

In this contribution, we discussed the RLC enhancements related issues. We propose:

Observation 1: In Tx initiated approach, it is more natural and accurate that transmitting side of RLC AM entity considers the remaining time of an SDU from the upper layer in XR service and triggers the discard procedure timely.

Observation 2: In Rx initiated approach, the gNB as receiver could perceive the remaining time below a threshold from DSR but it is not accurate. If the UE receives the discard report early from gNB e.g. based on the inaccurate reported remaining time, and then terminate the retransmission of obsolete SDUs which may cause more results in failure.

Observation 3: In Rx initiated approach, RLC receiving side needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. It seems to be complexity for RLC to maintain these timers between two layers (PDCP and RLC) concurrently.

Observation 4: In the combined Tx and Rx initiated approach, the timer in transmitting side is configured based on the remaining time while the timer in receiving side takes account t-Reordering in PDCP. It seems to be difficult to sync these two timers and ensure the window moving aligned.

Observation 5: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 1: RAN2 to adopt the Tx initiated approach as RLC AM enhancement to avoid the unnecessary re-transmission.

Proposal 2: Abandoning the retransmissions of an RLC SDU in the transmission side of RLC AM could be triggered by the indications received from PDCP layer or a new timer expired in RLC.

Proposal 3: The SN gap report from transmitting side could be used to notify the receiving RLC side for the obsolete SDUs. The RLC SN gap report may include the indications for the sets of SN or possibly SN range associated with the abandoned retransmission of RLC SDUs.

Proposal 4: To ensure that windows at Tx side and Rx side are aligned, the transmitting side abandons the retransmission of an obsolete RLC SDU or an RLC SDU segment when the ACK of an obsolete RLC SDU or an RLC SDU segment is received from the receiving side.

Proposal 5: When the receiving side of AM RLC entity receives the RLC SN gap report, it discards the buffered AMD PDUs or segment of AMD PDUs associated with SNs contained in RLC SN gap reporting and considers the positively acknowledged associated with discarded RLC SDU when constructing the STATUS report.

Proposal 6:The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Proposal 7:It could depend on the network to configure the timers t-Reassembly  and t-StatusProhibit properly to reduce the delay caused by transmitting status report. It is not necessary to introduce the enhancement for status report.

Proposal 8: The additional values of the timer t-PollRetransmit could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

Proposal 9:The remaining time could be considered to enhance the polling trigger mechanism.

Reference

- 3GPP TS 38.322 V18.0.0 (2023-12), Radio Link Control (RLC) protocol specification (Release 18).

- 3GPP TS 38.300 V18.1.0 (2024-03), NR and NG-RAN Overall Description;Stage 2(Release 18).

- 3GPP TS 38.331 V18.1.0 (2024-03),Radio Resource Control (RRC) protocol specification(Release 18)