3GPP TSG-RAN WG2 Meeting #127bis        	                       R2-2408079

Hefei, China, October 14th-18th, 2024

Agenda item:	8.3.3

Source:	CMCC

Title:	Discussion on measurement event prediction

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

In RAN2#125bis meeting [1], RAN2 discussed the measurement event prediction, and achieved the following agreements.

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction are is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

In this contribution, we will discuss on measurement event prediction use cases and relevant performance metrics/KPIs to evaluate.

- Discussion

- 2.1 Potential benefits for measurement event prediction

- In legacy handover procedure, when a HO event is fulfilled for the duration of time-to-trigger (TTT), the UE will send the measurement report (MR) to the network. Afterwards, the network can make handover decision and send HO command to the UE. Figure 1 illustrates the basic procedure of handover taking A3 event as an example. The A3 event is met at Ta, and measurement report is sent at Tb (Ta + TTT). The gNB sends a HO command (HO CMD) after HO preparation and the UE receives HO command at Tc.

- Figure 1. An example for legacy HO

- On the one hand, there is the latency from A3 event met to receiving HO CMD (i.e. Ta to Tc, the blue arrow). We think AI/ML can be used to predict whether the A3 event can be fulfilled for the duration of TTT. And if so, the UE can send the measurement report in advance (i.e. before Tb), which will reduce the latency of handover preparation.

- On the other hand, the HO CMD may be transmitted when the UE is in a poor radio link condition of serving cell (i.e. red circle), which means the configured TTT is too long. Thus, RLF may be occurred before the HO completion, which is similar to “Too Late Handover”. In this case, AI/ML can be used to predict a optimal configuration for measurement event.

- Figure 2. Basic flow for measurement event prediction

- Based on the analysis above, we think RAN2 can study the potential benefits of measurement events prediction.

- - Reduce the latency of handover preparation by sending the measurement report in advance based on prediction

- - Optimize the measurement event configuration based on prediction

- Proposal 1: RAN2 study the following potential benefits of measurement events prediction:

- - Handover preparation latency reduction by sending the measurement report in advance based on prediction

- - Measurement events configuration optimization based on prediction

- 2.2 Input/output for measurement event prediction

- RAN2#125bis meeting agreed that at least measurement event evaluation based on RRM measurement prediction result will be studied, and direct measurement event prediction are is also allowed. The similar sub-use cases for RLF prediction were also agreed, and the following agreements were achieved in last meeting [2] can be considered for measurement event prediction.

Agreements

-	Both direct and indirect are allowed.  Companies should indicate what they used and what inputs they are using

-	Output for indirect: predicted SINR.  Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

-	Output for direct: probability of RLF within an window

-	Companies should report the prediction time window they have used in the simulations

- For indirect prediction, the output is predicted RRM measurement (e.g. RSRP, RSRQ), and whether the measurement event occurs can be determined without further AI/ML model based on predicted RSRP of the future time instances. In this case, the input can be the same as RRM measurement prediction, i.e. L1 or L3 measurement results.

- Figure 1. Indirect measurement event prediction

- Proposal 2: For indirect prediction, the input and output of AI/ML model are as follows:

- - Input: L1 or L3 measurement results

- - Output: predicted RRM measurement (e.g. RSRP, RSRQ)

- For direct prediction, the input can include at least L1/L3 measurement results and measurement event configuration (i.e. A3 event), etc. The output can be similar as which is defined for RLF prediction, i.e. probability of measurement event occurrence within an window.

- Figure 2. Direct measurement event prediction

- Proposal 3: For direct prediction, the input and output of AI/ML model are as follows:

- - Input: at least includes L1/L3 measurement results, measurement event configuration (i.e. A3 event)

- - Output: probability of measurement event occurrence within an window

- 2.3 Performance metrics/KPIs

- Considering the sub-use case of measurement event prediction is similar to RLF prediction, we think that the KPIs defined for RLF prediction in last meeting [2] can be considered for measurement event prediction.

Agreements on KPI

For indirect

-	SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction.

Direct

-	missed RLF detection, false RLF detection, F1 score.

-	Whether the RLF will happen based on the following methodology.   Above a probability threshold we assume that RLF will happen.   This is then compared with true RLF.

- For indirect prediction, measurement event is predicted based on the temporal domain serving cell and neighbor cell measurement predictions (e.g. RSRP), thus, the performance metrics/KPIs defined in RRM measurement prediction can be reused, e.g. the RSRP difference between the predicted measurement and the actual measurement.

- In addition, the occurrence time of measurement event can be derived in this case since the measurement event is derived by predicted cell quality in future time instances. Therefore, the time difference between real time and predicted time of measurement event can also be considered as KPI.

- Proposal 4: For indirect prediction, the following performance metrics/KPIs could be considered:

- - RSRP difference between the predicted measurement and actual measurement

- - Time difference between real time and predicted time of measurement event

- For direct prediction, AI/ML model can predict the measurement event probability within a time window. For both indirect and direct prediction, the prediction accuracy of the measurement event occurrence is an important KPI to evaluate the model, and the typical ML classification metrics (e.g. precision, recall or F1-score) could be used.

Refer to [3], there is a classical formula to interpret the Precision, Recall or F1-score:

Table 1

| Predicted event Baseline event | Positive | Negative |
| --- | --- | --- |
| Positive | TP | FN |
| Negative | FP | TN |

In Table 1, there are 4 cases defined as follows:

- TP (True Positives): the number of Predicted Positives that were correct, i.e. an event is predicted and an event occurs in baseline

- FN (False Negatives): the number of Predicted Negatives that were incorrect, i.e. no event is predicted but an event occurs in baseline

- FP (False Positives): the number of Predicted Positives that were incorrect, i.e. an event is predicted but no event occurs in baseline

- TN (True Negatives): the number of Predicted Negatives that were correct, i.e. no event is predicted and no event occurs in baseline

The accuracy formulas are as follows based on the above definitions:

- Precision = TP / (TP+FP)

- Recall = TP / (TP+FN)

- F1 score = 2 * Precision * Recall / (Precision+ Recall)

- As RAN2 agreed for RLF prediction, missed detection, false detection, true prediction and F1 score can be used as KPI. In our understanding, the missed detection means no measurement event is predicted but an measurement event occurs in baseline (i.e. FN in Table 1), false detection means an measurement event is predicted but no measurement event occurs in baseline (i.e. FP in Table 1), true detection means that an measurement event is predicted and an measurement event occurs in baseline (i.e. TP in Table 1) and no measurement event is predicted and no measurement event occurs in baseline (i.e. TN in Table 1).

Proposal 5: For direct and indirect prediction, the following performance metrics/KPIs could be considered:

- Missed detection, i.e. no measurement event is predicted but an measurement event actually occurs

- False detection, i.e. an measurement event is predicted but no measurement event actually occurs

- True detection, i.e. an measurement event is predicted and an measurement event actually occurs, and no measurement event is predicted and no measurement event actually occurs

- F1 score

  - 2.4 Simulation assumptions

- Based on TR 36.839 [4], we would like to provide measurement event A3 prediction specific simulation assumptions as follows, and the assumptions for RRM measurement prediction can be reused.

- Table 2: The baseline simulation assumptions for A3 Event prediction

| Parameter | Value |
| --- | --- |
| TimeToTrigger (TTT) [ms] | 40, 80, 160, 480 |
| a3-offset [dB] | -1, 0, 1, 2, 3 |
| A3 hysteresis (Hys) | 1dB |

Proposal 6: RAN2 considers the following simulation assumptions of A3 Event as the baseline for measurement event prediction evaluation.

| Parameter | Value |
| --- | --- |
| TimeToTrigger (TTT) [ms] | 40, 80, 160, 480 |
| a3-offset [dB] | -1, 0, 1, 2, 3 |
| A3 hysteresis (Hys) | 1dB |

# 3	Conclusion

Here are the observations and proposals for measurement event prediction.

Potential benefits for measurement event prediction

- Proposal 1: RAN2 study the following potential benefits of measurement events prediction:

- - Reduce the latency of handover preparation by sending the measurement report in advance based on prediction

- - Optimize the measurement events configuration based on prediction

Input/output for measurement event prediction

- Proposal 2: For indirect prediction, the input and output of AI/ML model are as follows:

- - Input: L1 or L3 measurement results

- - Output: predicted RRM measurement (e.g. RSRP, RSRQ)

- Proposal 3: For direct prediction, the input and output of AI/ML model are as follows:

- - Input: at least includes L1/L3 measurement results, measurement event configuration (i.e. A3 event)

- - Output: probability of measurement event occurrence within an window

- Performance metrics/KPIs

- Proposal 4: For indirect prediction, the following performance metrics/KPIs could be considered:

- - RSRP difference between the predicted measurement and actual measurement

- - Time difference between real time and predicted time of measurement event

Proposal 5: For direct and indirect prediction, the following performance metrics/KPIs could be considered:

- Missed detection, i.e. no measurement event is predicted but an measurement event actually occurs

- False detection, i.e. an measurement event is predicted but no measurement event actually occurs

- True detection, i.e. an measurement event is predicted and an measurement event actually occurs, and no measurement event is predicted and no measurement event actually occurs

- F1 score

Simulation assumptions

Proposal 6: RAN2 considers the following simulation assumptions of A3 Event as the baseline for measurement event prediction evaluation.

| Parameter | Value |
| --- | --- |
| TimeToTrigger (TTT) [ms] | 40, 80, 160, 480 |
| a3-offset [dB] | -1, 0, 1, 2, 3 |
| A3 hysteresis (Hys) | 1dB |

# 4	Reference

- Draft_R2_125bis_Meeting_Report_v2

- Draft_R2_127_Meeting_Report_v1

- D. J. Powers, “Evaluation: From Precision, Recall and F-measure To ROC, Informedness, Markedness & Correlation”, Journal of Machine Learning Technologies, vol. 2, no.1, pp. 37-63, 2011

- TR 36.839 Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks