3GPP TSG RAN WG2 Meeting #127bis	R2-2408082

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.9.3

Source:	CMCC

Title:	Further discussion on uplink capacity enhancement for IoT-NTN

Document for:	Discussion, Decision

Introduction

- In last meeting, there were some discussions on uplink capacity enhancements for IoT-NTN and achieved some agreements as follows [1]:

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In this contribution, we would like to provide some further discussion on EDT enhancements in IoT-NTN system.

Discussion

In RAN2 #126 meeting, we sent an LS to RAN1/4 to check whether the pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Then RAN1/4 provided feedback as following respectively:

RAN1 reply LS (R1- 2407548):

- Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to take the response into account. To RAN4: ACTION: RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

RAN4 reply LS (R4-2414114):

# 1. Overall Description: RAN4 would like to thank RAN2 for the LS R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. RAN2 asked for RAN4 following questions: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP. # 2. Actions: RAN4 respectfully asks RAN2 to take the above information into account.

Based on the feedback from RAN1/4, the pre-compensated TA could satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2. Therefore, RAN2 could confirm to continue work on a CB-Msg3 EDT-like mechanism.

Observation 1: Based on the feedback from RAN1/4, the pre-compensated TA could satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Proposal 1: RAN2 could confirm to continue work on a CB-Msg3 EDT-like mechanism.

Contention-based Msg3 transmission

- In RAN2 #126 meeting, we agreed to focus on the contention-based Msg3 transmission to complete an EDT-like transaction, however some issues are FFS, FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources. We would like to provide some analysis for the FFS issues respectively.

- Msg3 details

- In the current TS 36.300, MO-EDT is triggered when the upper layers have requested the establishment or resumption of the RRC Connection for Mobile Originated data (i.e., not signalling or SMS) and the uplink data size is less than or equal to a TB size indicated in the system information.

- Observation 2: There are two types of trigger conditions for existing MO-EDT, that is request the establishment or resumption of the RRC Connection for Mobile Originated data by upper layers and whether the the uplink data size is less than or equal to a TB size indicated in the system information.

- For contention-based Msg3 transmission for EDT in IoT-NTN, the trigger condition may at least support the establishment or resumption of the RRC Connection requested by the upper layers. Then for uplink data size whether less than or equal to a TB size indicated in the SI could be postponed until how to provide uplink resource for Msg3 transmission without Msg1/Msg2 make more detail progress.

- Proposal 2: For the trigger of contention-based Msg3 transmission in IoT-NTN, it is at least to support the establishment or resumption of the RRC Connection requested by the upper layers.

- Per the existing MO-EDT, Msg3 includes RRCEarlyDataRequest message concatenating the user data on CCCH or RRCConnectionResumeRequest message on CCCH multiplexed with uplink user data on DTCH for Control Plane CIoT EPS Optimisations and User Plane CIoT EPS Optimisations respectively. Then for the Msg3 content in contention-based Msg3 transmission for EDT in IoT-NTN, the Msg3 content in current MO-EDT could be the baseline.

- Proposal 3: For the Msg3 content in contention-based Msg3 transmission for EDT in IoT-NTN, the Msg3 content in current MO-EDT could be the baseline.

- Resource allocation

To increase the UL system capacity, solutions reducing the reliance on DL signaling could be discussed. Some existing solution such as preconfigured uplink resource (PUR) could be considered. However, the existing PUR scheme is contention free shared (CFS) and needs to reserve resources for a given UE, which only supports a limited number of pre-configured UL transmission, having an impact on the potential gains as well as network capacity available to other UEs. To resolve the issue, a contention based PUR solution could be discussed. Network broadcasts one or several shared PUR pool(s) in a cell, then all the UEs in the cell select the UL resources in the PUR pool(s) randomly. Further, with the shared PUR pool(s), UE could perform EDT procedure without msg1/RAR, that is to transmit msg3 directly to the network.

Proposal 4: For resource allocation, one or more contention-based PUR pool(s) could be configured via broadcast manner.

- Contention Resolution

However, with contention based shared PUR resource, obviously, there will still be a collision issue. And we could consider a similar contention resolution scheme in CBRA procedure. In detail, network receives the CCCH SDU included in the Msg3 and transmitted Msg4 carrying a UE Contention Resolution Identity MAC CE. If the UE Contention Resolution Identity in the MAC CE matches the 48 first bits of the CCCH SDU transmitted in Msg3, this Contention Resolution could be considered successfully.

Proposal 5: A similar contention resolution scheme in CBRA procedure (i.e. whether Msg4 carrying a UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3) could be considered as the baseline to resolve the collision issue of EDT based on CBS PUR.

- Procedural step

- Based on above, procedure combination EDT and contention-based PUR for Control Plane CIoT EPS Optimisations and User Plane CIoT EPS Optimisation are provided as following respectively:

Figure 1: Transmission using PUR for the Control Plane CIoT EPS Optimisations

- 0.	Network provides shared PUR pool(s) configuration via SIBxx for the UEs in the cell.

- 1	Same as step 1 in MO-EDT for Control Plane CIoT EPS optimisations in Figure 7.3b-1 except that the UE transmits over the shared PUR resource instead of a resource allocated in the random access response.

- If the uplink data are too large to be included in RRCEarlyDataRequest, the UE can use the shared PUR resource to transmit RRCConnectionRequest. The procedure will fall back to the legacy RRC Connection establishment procedure, a new C-RNTI can be assigned.

- After step 1, the eNB may request the UE to abort the transmission using shared PUR by sending a Layer 1 fallback indication. UE actions upon reception of Layer 1 fallback indication are left up to UE implementation.

- 2..6 Same as MO-EDT for Control Plane CIoT EPS Optimisations in Figure 7.3b-1.

- 7	If no further data are expected, the eNB can send the RRCEarlyDataComplete message including UE Contention Resolution Identity on CCCH to keep the UE in RRC_IDLE. If downlink data were received in step 6, they are concatenated in RRCEarlyDataComplete message.

Figure 2: Transmission using PUR for the User Plane CIoT EPS Optimisation

- 0.	Network provides shared PUR pool(s) configuration via SIBxx for the UEs in the cell.

- 1	Same as step 1 in MO-EDT for User Plane CIoT EPS optimisations in Figure 7.3b-2 except that the UE transmits over the shared PUR resource instead of a resource allocated in the random access response.

- If the user data are too large to be fully included in the transmission using shared PUR, the UE can use shared PUR to transmit RRCConnectionResumeRequest and a segment of the user data. The procedure will fall back to the legacy RRC Connection Resume procedure; a new C-RNTI can be assigned.

- After step 1, the eNB may request the UE to abort the transmission using shared PUR by sending a Layer 1 fallback indication. UE actions upon reception of Layer 1 fallback indication are left up to UE implementation.

- 2..7 Same as MO-EDT for User Plane CIoT EPS optimisations in Figure 7.3b-2.

- 8	Same as step 8 in MO-EDT for user Plane CIoT EPS optimisations in Figure 7.3b-2 and a UE Contention Resolution Identity is also included.

- Proposal 6: If combination EDT and contention-based PUR is supported, the above procedure could be the baseline.

- Fallback mechanism

Then a maximum number of EDT transmissions based on shared PUR resource could be defined to avoid the more serious delay caused by too many attempts. If UE attempts up to the maximum number of EDT transmissions, a fall back mechanism could be considered, i.e., to perform legacy RACH procedure to obtain UL resource.

Proposal 7: It is proposed to introduce a maximum number of EDT transmissions based on shared PUR resource to avoid the more serious delay caused by too many attempts.

Proposal 8: If UE attempts up to the maximum number of EDT transmissions, a fall back mechanism could be considered, i.e., to perform legacy RACH procedure to obtain UL resource.

- Combination of OCC and CB-Msg3

For uplink capacity enhancements, RAN1 focus on OCC for NPRACH, and the following agreements are achieved:

RAN1 #116 Agreement: For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level (including Nslot level) - Repetition-level - RV-level - Intra-symbol pre-DFT spreading OCC RAN1 #116bis Agreement: OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS. For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study: - TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs. - FFS: Detailed mapping - CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs. - FFS: Detailed mapping - Other schemes are not precluded, including combinations of the above RAN1 #117 Agreement: For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. Inter-repetition OCC for NPRACH is not studied further in RAN1. - For the time-domain DMRS pattern (including blanked DMRS, if any): - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern - For 3.75kHz single-tone - RAN1 studies - Rel-17 DMRS pattern - A new DMRS pattern - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17 The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1. RAN1 #118 Agreement: RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

Based on above agreements, RAN1 will not consider multiplexing more than 4 UEs. Applying OCC for CB-Msg3 procedure is beneficial for UL capacity improvement. However OCC enhancements is just for NB-IoT based on the WID scope, and RAN2 agreed in 125bis meeting that both NB-IoT and eMTC are within the scope of uplink capacity enhancements, which only for EDT enhancements. Therefore, maybe we could consider to combination OCC and CB-Msg3 for NB-IoT at least.

Proposal 9: Suggest RAN2 to discuss combination OCC and CB-Msg3 for NB-IoT for UL capacity improvement.

Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA)

Two interesting solutions Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) were further discussed in last meeting, some companies prefer to study the solution while some companies pointed out that the CRDSA solution is fascinating but maybe too new for R19, and RAN1 should be also involved. The discussion is postponed to this meeting to evaluate possible impacts on the specification and whether an LS to RAN1 is needed.

DSA is indicated in [3] that it could help to improve the Msg3 EDT (without msg1) throughput by about a factor of 4.5 without any impacts on the current receiver implementation at the network, and minimal specification impact overall.

Then regarding the CRDSA, it support to generate two or more replicas of the same packet at random time within a frame (as shown in figure 3), each packet includes the slot position information of the corresponding twin packet within the frame and packet collisions could be cleared up through a simple but effective iterative interference cancellation mechanism which is that to use frame composition information from the replica packet. Specifically, if one clean packet 3 in slot 5 is successfully received, then this information could be used to cancel the interference caused by packet 3 in slot 4 and packet 2 could be decoded successfully and so on. With a simple iterative interference cancellation solution, most packets could be recovered. Obviously, the main benefits of CRDSA are packet loss ratio improvement, transmission delay reduction and higher throughput. And in [3] and [4], there are simulation to prove the better throughput performance of the CRDSA compared to DSA.

Figure 3 Example CRDSATDMA frame structure for the Random Access

Observation 3: CRDSA provides a better throughput performance of the CRDSA compared to DSA.

Although CRDSA have been supported and practically demonstrated in the satellite DVB-RCS2 standard and may be beneficial for IoT-NTN [3], while for R19, such CRDSA or similar schemes may be too innovative and high packets load would also lead to dead lock issue. Therefore in R19 IoT-NTN, suggest RAN2 deprioritize the discussion of CRDSA in R19 considering the limited TUs or if majority prefer to discuss, an LS to RAN1 to make more evaluation about CRDSA or similar schemes is needed.

Proposal 10: It is proposed that deprioritize the discussion of CRDSA in R19 considering the limited TUs or if majority prefer to discuss, an LS to RAN1 to make more evaluation about CRDSA or similar schemes is needed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on EDT enhancements for IoT-NTN and have the following observations and proposals:

Observation 1: Based on the feedback from RAN1/4, the pre-compensated TA could satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Proposal 1: RAN2 could confirm to continue work on a CB-Msg3 EDT-like mechanism.

Contention-based Msg3 transmission:

- Observation 2: There are two types of trigger conditions for existing MO-EDT, that is request the establishment or resumption of the RRC Connection for Mobile Originated data by upper layers and whether the the uplink data size is less than or equal to a TB size indicated in the system information.

- Proposal 2: For the trigger of contention-based Msg3 transmission in IoT-NTN, it is at least to support the establishment or resumption of the RRC Connection requested by the upper layers.

- Proposal 3: For the Msg3 content in contention-based Msg3 transmission for EDT in IoT-NTN, the Msg3 content in current MO-EDT could be the baseline.

Proposal 4: For resource allocation, one or more contention-based PUR pool(s) could be configured via broadcast manner.

Proposal 5: A similar contention resolution scheme in CBRA procedure (i.e. whether Msg4 carrying a UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3) could be considered as the baseline to resolve the collision issue of EDT based on CBS PUR.

- Proposal 6: If combination EDT and contention-based PUR is supported, the above procedure could be the baseline.

Proposal 7: It is proposed to introduce a maximum number of EDT transmissions based on shared PUR resource to avoid the more serious delay caused by too many attempts.

Proposal 8: If UE attempts up to the maximum number of EDT transmissions, a fall back mechanism could be considered, i.e., to perform legacy RACH procedure to obtain UL resource.

Proposal 9: Suggest RAN2 to discuss combination OCC and CB-Msg3 for NB-IoT for UL capacity improvement.

Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA):

Observation 3: CRDSA provides a better throughput performance of the CRDSA compared to DSA.

Proposal 10: It is proposed that deprioritize the discussion of CRDSA in R19 considering the limited TUs or if majority prefer to discuss, an LS to RAN1 to make more evaluation about CRDSA or similar schemes is needed.

Reference

- R2-127 NR-NTN-IoT-NTN (Sergio)_ EOM

- 3GPP TS 36.300 V18.1.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- R2-2407502	Discussion on DSA and CRDSA Performance	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot	discussion	Rel-19

- E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random-Access Scheme for Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007.