3GPP TSG-RAN WG2 Meeting #127bis	R2-2408085

# Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.2

Source:	CMCC

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

Introduction

In RAN#127meeting, RAN2 reached the following agreements on on-demand SSB:

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this contribution, we provide our views on on-demand SSB Scell operation for UEs in RRC_CONNECTED mode configured with CA.

Discussion

Configuration and (de)activation of OD-SSB

In last RAN2 meeting, both RRC-based and MAC CE-based OD-SSB indication were agreed, while the detail and how they works together are not clear.

For RRC-based approach, RAN2 agreed to indicate at least the initial activation/deactivation state of OD-SSB configuration. In addition to this, we think RRC based OD-SSB transmission indication should also include OD-SSB related configuration, especially when the initial state of OD-SSB is activated. Based on RAN1’s discussion, the following parameters are related to OD-SSB transmission:

RAN1#117 meeting Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

Proposal 1: The following parameters of OD-SSB should be indicated in RRC message for initial configuration or reconfiguration:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

Besides, RAN1 discussed MAC-CE based parameter indication, i.e., periodicity of OD-SSB

RAN1#118 meeting Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Based on RAN1’s agreements, the new MAC CE can be designed to including both the OD-SSB transmission indication and periodicity of OD-SSB. But as proposed in Proposal 1, RRC siganlling can also be used to configure the periodicity of OD-SSB, there’s two understandings of RAN1’s agreements:

- If the periodicity is not configured in RRC signalling, it is indicated in MAC CE;

- If multiple OD-SSB configurations, i.e., with different periodicities are configured by RRC signalling, and MAC CE is used to indicate the actually activated configuration/periodicity.

Therefore, RAN2 should discuss whether multiple OD-SSB configuration is supported.

Proposal 2: RAN2 should discuss whether multiple OD-SSB configuration is supported.

Furthermore,  for MAC-CE based OD-SSB indication, the last RAN2 meeting agreed to indicate OD-SSB transmission, in other words, the new MAC CE is to activate OD-SSB. While for the deactivation of OD-SSB is not mentioned. In our opinion, the new MAC CE can also be used for OD-SSB deactivation. But how it works may depends on the discussion of OD-SSB transmission, since OD-SSB burst transmission is still under discussion in RAN1. If burst transmission is adopted, the end time of OD-SSB burst can be indicated with OD-SSB periodicity in MAC CE or RRC configuration, else, OD-SSB transmission can be deactivated directly by the new MAC CE.

Proposal 3: The new MAC CE can be used to indicate deactivation of OD-SSB directly or by indicating the end time of the OD-SSB burst transmission. It depends on the discussion of OD-SSB burst transmission in RAN1.

Though RAN2 agreed to introduce a new MAC CE, it is mainly for Scenario#2,  in which the new MAC CE is received before SCell activation MAC CE. While in Scenario#2A, OD-SSB transmission indication MAC CE is received when UE receives SCell activation command. Since RAN2 agreed to support Scenario#2A and anyway a new MAC CE will be designed, it’s better to including SCell activation/deactivation indication in the same MAC CE with minimum extra specification workload.

Proposal 4: To support Scenario#2A, the new MAC CE should include the SCell activation/deactivation indication with OD-SSB related indication.

OD-SSB measurement

In legacy, for a deactivated SCell, the measurement is performed by the UE referring to the value of  ‘measCycleSCell’. And taking FR1 as an example, if during the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command the UE has sent a valid measurement report for the SCell being activated and the SSB measured remains detectable, SCell is a known [3], otherwise SCell is unknown. And the activation delay for known SCell and unknown SCell is different, the activation delay of unknown SCell is longer than that of known SCell based on [3].

Observation 1: In legacy, the activation delay of unknown Scell is longer than that of known SCell.

After on-demand SSB triggering, if UE still perform measurement sampling refer to the value of ‘measCycleSCell’, which is equal to or larger than the maximum SSB period 160ms, the power will be wasted since network transmit the on-demand SSB densely while UE sample the on-demand SSB sparsely. Therefore, we think there should be different measurement configurations to match OD-SSB. When OD-SSB is activated, the OD-SSB based measurement configuration should be configured or activated.

Proposal 5: There should be OD-SSB based measurement configuration, when OD-SSB is activated, OD-SSB based measurement configuration should be configured or activated.

In last RAN1 meeting, there’s discussion of LTM issues when discussing L1 measurement based on on-demand SSB.

RAN1#118 meeting Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point

LTM is a kind of mobility triggered by cell switch command MAC CE from the network, and the network’s decision is based on UE’s L1 measurement results. In our opinion, once OD-SSB transmission is activated, it can be used for the L1 measurement for LTM. Hence, for a UE supporting both OD-SSB and LTM, OD-SSB should be activated first before LTM cell switch command.

Proposal 6: Once OD-SSB transmission is activated, it can be used for the L1 measurement for LTM.

Proposal 7: For a UE supporting both OD-SSB and LTM, OD-SSB should be activated first before LTM cell switch command.

Conclusions

In this contribution, we discussed the applicable scenario, of on-demand SSB in SCell for connected UEs, and the following proposals and observations are made.

Proposals:

Proposal 1: The following parameters of OD-SSB should be indicated in RRC message for initial configuration or reconfiguration:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

Proposal 2: RAN2 should discuss whether multiple OD-SSB configuration is supported.

Proposal 3: The new MAC CE can be used to indicate deactivation of OD-SSB directly or by indicating the end time of the OD-SSB burst transmission. It depends on the discussion of OD-SSB burst transmission in RAN1.

Proposal 4: To support Scenario#2A, the new MAC CE should include the SCell activation/deactivation indication with OD-SSB related indication.

Proposal 5: There should be OD-SSB based measurement configuration, when OD-SSB is activated, OD-SSB based measurement configuration should be configured or activated.

Proposal 6: Once OD-SSB transmission is activated, it can be used for the L1 measurement for LTM.

Proposal 7: For a UE supporting both OD-SSB and LTM, OD-SSB should be activated first before LTM cell switch command.

Observations:

Observation 1: In legacy, the activation delay of unknown Scell is longer than that of known SCell.

References

- Report of RAN2#127 meeting;

- Report of RAN1#117 meeting;

- TS 38.133.