3GPP TSG-RAN WG2 Meeting #127bis	R2-2408086

# Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.3

Source:	CMCC

Title:	Discussion on on-demand SIB1

Document for:	Discussion

Introduction

In RAN#102 meeting, a new WID on enhancements of network energy saving has been approved [1], one of the objectives is on-demand SIB1 for UEs in idle/inactive mode, the scope of this objective has been discussed and revised in RAN#105 [2], the revised objective is as following,

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we provide our views on the OD-SIB1 WUS configuration, OD-SIB1 transmission, cell selection and legacy UE access restriction.

Discussion

WUS configuration

In SI discussion on OD-SIB1, RAN2 agreed that WUS configuration is provided in Cell A’s system information, and once the UE camp on a NES cell, the UE can acquire the updated WUS configuration in the NES cell.

RAN2#126 meeting agreements[3]

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

RAN2#127 meeting agreements[4]

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

Based on the current agreements, WUS configuration can be acquired in both NES cell and Cell A. For Celll A, the WUS can be convey in SIB1 or other SIBs. In our point of view, SIB1 is mainly to indicate the necessary information for the UE to camp in one cell, it’s not rational to indicate assistant information for other cells. What’s more, RAN2 agreed to considered the cases that a NES cell with multiple Cell A or one Cell A with multiple NES cells, for the second case, if a Celll A need to support multiple NES cells and SIB1 is used for WUS configuration provision, it will bring heavy burden to Cell A’s SIB1. Therefore, we prefer to use a new SIB to indication WUS configurations for NES cell.  And it can be periodically broadcast or broadcast on-demand. What’s more, this can be applied to both NES cell and Cell A.

Proposal 1:  A new SIB can be introduced  for WUS configuration indication, and it can be periodically broadcast or broadcast on-demand.

Proposal 2: The new SIB can be broadcast in both Cell A and NES cell.

OD-SIB transmission

RAN2 assumes that the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline. And after receiving the RAR, the UE may expect to receive the corresponding SIB1 as soon as possible, similar to on-demand SI request and transmission. In the other hand, keep transmitting the SIB1 will result in a significant energy consumption from gNB side. Considering the trade-off between NES gain and reception performance on SIB1, we support to reuse the mechanism in on-demand SI procedure as baseline, i.e., introduce a time window for SIB1 transmission. The gNB can only transmit SIB1 during the certain window while the UE keeps monitoring it.

Proposal 3: If the UE receives RAR for OD-SIB1 request, it acquires the requested SIB1 immediately.

Proposal 4: Support to introduce a time window for SIB1 reception as baseline.

Cell selection considering NES cell

Besides, in RAN2#126 meeting, only cell reselection to NES-cell with SIB1 on-demand transmission is discussed. It is if UE chooses the NES cell using legacy intra-frequency/inter-frequency cell re-selection procedure (as baseline), the UE triggers WUS transmission, and then, if the UE acquire SIB1 successfully and if the NES cell is a suitable cell, the UE camps on the NES cell and then monitor paging and SI updating as legacy. But for cell selection, it’s not clear.

Regarding the issue on cell selection procedure with NES cell, in our understanding, allowing UE select to NES during cell selection procedure (especially for initial cell selection case) may result in significant extra delay, as is shown in Figure 1.

Figure 1. An example for extra delay on OD-SIB1 procedure during cell selection

Simultaneously, during the previous discussion on both RAN1 and RAN2, the latency of the initial cell selection procedure is considered to be a critical issue. Therefore, from this perspective, it is recommended that R19 NES UE shall not select to the NES cell during cell selection procedure (especially for initial cell selection case), so that it can quickly switch to a cell that is broadcasting SIB1 and the latency of cell-selection can be ensured.

Proposal 5: Support R19 NES UE not to select the NES cell during cell selection procedure (especially for initial cell selection case).

Regarding the discussion on cell reselection procedure with NES cell, it is agreed to reuse the legacy cell reselection procedure as baseline. From this perspective, R19 NES UE can request the SIB1 of all the configured NES cells according to its needs. However, if the number of UE or the number of NES cells configured in a single UE is relatively large, a significant transmission overhead will be caused by the legacy cell reselection procedure, which can result in a large extra energy consumption from both gNB and UE side. Thus, the enhancement of legacy cell reselection procedure can be considered, e.g., the R19 NES UE can preferentially reselect to non-NES cell that meet the criteria of reselection.

Proposal 6: Enhancement of legacy cell reselection procedure can be considered, e.g., the R19 NES UE can preferentially reselect to non-NES cell that meet the criteria of reselection.

To avoid UE access to a NES cell in the initial cell selection, and Besides, UE should identify whether a cell is NES cell. Besides, RAN2 agreed that if the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. In this case, UE also need to differentiate NES cell and non-NES cell.

Proposal 7: Rel-19 NES UE should be indicated whether a cell is a NES cell or not.

Access restriction to legacy UE

In last RAN2 meeting, some options were proposed to restrict legacy UE’s access to NES cell:

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

For Option 1, it may cause the case that not all legacy UEs can be barred, such as RedCap UEs or Rel-18 NES UE, which are barred based on the indication in SIB1.

Observation 1: Option 1 cannot bar all legacy UEs, such as RedCap UE. and Rel-18 NES UEs.

For Option 3, though it may avoid decoding PBCH/MIB decoding and measurement of OD-SSB NES cell by the UE, it is not flexible, since for a OD-SIB1 NES cell with on-demand SIB1 transmission deactivated, it can be accessed by the legacy UE as a normal cell when OD-SIB1 is transmitted.

Observation 2: Option 3 is not flexible considering a OD-SIB1 cell can be treated as a normal cell by legacy UEs when OD-SIB1 is transmitted.

Therefore, we prefer to use Option 2 to bar legacy UEs. What’s more, considering that it is corresponding to kSSB, which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers, and if reserved value of kSSB is used, it can be treated as an indication for Rel-19 NES UE for NES cell identification.

Proposal 8: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

Conclusions

In this contribution, we analyse the OD-SIB1 WUS configuration, OD-SIB1 transmission, cell selection and legacy UE access restriction , following are the proposals and observations:

Proposal 1:  A new SIB can be introduced  for WUS configuration indication, and it can be periodically broadcast or broadcast on-demand.

Proposal 2: The new SIB can be broadcast in both Cell A and NES cell.

Proposal 3: If the UE receives RAR for OD-SIB1 request, it acquires the requested SIB1 immediately.

Proposal 4: Support to introduce a time window for SIB1 reception as baseline.

Proposal 5: Support R19 NES UE not to select the NES cell during cell selection procedure (especially for initial cell selection case).

Proposal 6: Enhancement of legacy cell reselection procedure can be considered, e.g., the R19 NES UE can preferentially reselect to non-NES cell that meet the criteria of reselection.

Proposal 7: Rel-19 NES UE should be indicated whether a cell is a NES cell or not.

Observation 1: Option 1 cannot bar all legacy UEs, such as RedCap UE. and Rel-18 NES UEs.

Observation 2: Option 3 is not flexible considering a OD-SIB1 cell can be treated as a normal cell by legacy UEs when OD-SIB1 is transmitted.

Proposal 8: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

References

- RP-234065, New WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #102;

- RP-242354, Revised WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #105;

- Report of RAN2 #126 meeting;

- Report of RAN1 #117 meeting.