3GPP TSG-RAN WG2 Meeting #127bis	R2-2408087

# Hefei, China, Oct 14th – 18th, 2024

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#127 meeting, RAN2 reached the following agreements on inter-CU LTM [1]:

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In this paper, we further discuss the open issues of NR SA scenario and potential issues of NSA case.

- Discussion

  - Inter-CU in SA scenario

    - L3 measurement results triggered LTM

In the discussion of decision node of inter-CU LTM in last meeting, L3 measurement results triggered LTM is mentioned, which is also discussed for Rel-18 intra-CU LTM. The reason to support L3 measurement results triggered LTM is that the UE capability of LTM and L1 measurement reporting is decoupled, so that UE supporting LTM may have no L1 measurement results. Based on last meeting agreement, CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements, and there’s related LS for RAN3 for F1 enhancements [2].

For L1 measurement results triggered LTM, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

Observation 1: In current LTM procedure, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

However, when it comes to L3 measurement result triggered LTM, it’s rational that gNB-CU only indicates the target cell ID and related information but not L3 measurement results, in this case, there’s no need for the gNB-DU to send target cell ID to the gNB-CU again.

Proposal 1: For L3 measurement results triggered LTM, it’s not needed for the source gNB-DU to indicate cell ID to the source gNB-CU after it sends LTM cell switch command to the UE.

The motivation to support L3 measurement based LTM is for the UE without L1 measurement capability. But when it comes to the UE with L1 measurement capability, it’s not clear, whether the gNB-CU can request the gNB-DU to send LTM cell switch command based on L3 measurement results. If the gNB-CU can request the gNB-DU to send LTM switch command for the UE with L1 measurement capability, it may cause conflict between the gNB-CU and the gNB-DU on the target cell selection. In our opinion, LTM is to take advantage of L1 measurement, therefore, for  the UE with L1 measurement capability, the gNB-CU should not request LTM based on L3 measurement results.

Proposal 2: For the UE with capability of L1 measurement, the source gNB-CU cannot request the source gNB-DU to trigger LTM cell switch command based on L3 measurement results.

    - UE-based TA measurement

In Rel-18 intra-CU LTM, except L2 handing, the UE’s execution of UE-based TA measurement is also based on the related ID comparison of the serving cell and candidate cell. In the current spec, it’s UE implementation on how to measure TA when this way is configured by the network, and there’s no restrictions on the scenarios  in which UE can be configured with UE-based TA measurement. In other words, whether to configure UE-based TA measurement to the UE is a kind of network implementation, maybe it is decided based on the prior information in the serving cell.

Observation 2: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

When it comes to Rel-19 inter-CU LTM, we think the same principle can be applied, the serving cell can configure UE-based TA measurement to the UE. If UE cannot access to the target cell successfully, it can fallback to legacy RACH procedure.

Proposal 3: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

    - Stage 2 signalling procedure

Basic procedure has been discussed in the previous meetings, but there’re still some open issues:

- Inter-CU LTM preparation

In RAN2#126 meeting [3], RAN2 agreed that the preparation of inter-CU LTM configuration is initiated by the source gNB-CU. Since the UE can be configured with multiple LTM candidate cells, and it’s possible that some of the candidate cells belong to the same candidate gNB-CU. One open issue is whether  a single HO request can be used to initiate the preparation of  candidate cells belonging to the same candidate gNB-CU. Similar case can be found in CHO. In TS 38.300, it is described as “A CHO request message is sent for each candidate cell.”  We think this can be applied in inter-CU LTM.

Proposal 4: LTM request message(in HO request) is sent for each candidate cells, no matter whether they belong to the same gNB-CU or not.

- LTM configuration transfer

In Rel-18 intra-CU LTM, all candidate cells belong to the same CU, therefore, there’s no need to transfer LTM configurations from serving cell to target cell. However, it’s different for inter-CU LTM. The LTM related configuration should be shared among candidate cells, or at least the target cell. When LTM is successfully executed, Handover Success procedure over Xn is reused, the source gNB-CU will have knowledge that UE has accessed to the target gNB-CU. Therefore, the serving gNB-CU can transfer LTM configuration after it collects all candidate cells’ configuration or after UE’s success handover to the target gNB-CU.

Proposal 5: Serving gNB-CU transmits LTM configuration after it collects all candidate cells’ configuration or after UE’s success handover to the target gNB-CU.

- LTM data forwarding

With supporting of  Rel-19 inter-CU LTM, UE will be served with the different CUs after cell switch, and data forwarding between gNBs is needed. Based on the current LTM procedure, the data forwarding address is only exchanged in the LTM preparation by the initial serving gNB-CU with candidate gNB-CU s. Once LTM executed,  the actual serving gNB-CU has no knowledge of candidate gNB-CU’s data forwarding address. Therefore, data forwarding addresses information should be indicated to candidate gNB-CUs. Similar to the previous discussion, this can be performed in LTM preparation during which all candidate gNB-CUs will be indicated data forwarding address information or performed after every LTM cell switch and only the target gNB-CU is indicated.

Proposal 6: The serving gNB-CU needs to indicate data forwarding address information of candidate gNB-CUs, and this can be performed in LTM preparation phase or after every LTM cell switch.

  - Inter-CU LTM in NSA scenario

    - SN-initiated inter-CU SCG LTM

As mentioned above, the UE capabilities of LTM and L1 measurement results are decoupled, and LTM can be triggered based on L3 measurement results, but the discussion mainly focus on the LTM in SA scenario, it should be made clear that inter-CU LTM for SCG can also be triggered by L3 measurement results.

Proposal 7: Inter-CU LTM for SCG can be  triggered by L3 measurement results.

In last RAN2 meeting, it was agreed that only inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. In TS 37.340 [4], the procedure of SN-initiated SN change/conditional SN change  is specified and it can be taken as the baseline for SN-initiated inter-SN LTM.

Proposal 8: The procedure of SN-initiated SN change/conditional SN change specified in TS 37.340 can be taken as the baseline of SN-initiated inter-CU SCG LTM.

Referring to the current SN-initiated (conditional) SN change procedure, the source SN should indicate inter-CU SCG LTM indication and candidate SN ID(s) in SN Change Required message to MN. What’s more, SCG configuration and measurement results can also be contained in the message. And when MN initiates SN addition procedure to candidate SNs, inter-CU SCG LTM indication should also be contained in SN addition message.

Proposal 9: Inter-CU SCG LTM indication should be included in SN change Required message from the source SN to MN and SN addition message from MN to candidate SN(s).

As agreed in RAN2#127 meeting[1] that the inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. Therefore, it’s rational that MN sends RRCReconfiguration Message including inter-CU SCG LTM configuration to the UE  and then the UE replies RRCReconfigurationComplete to the MN. Once MN receive UE’s RRCReconfigurationComplete message, it can indicate the source SN to confirm SN LTM configured.

Proposal 10: It’s MN to send RRCReconfiguration message including inter-CU SCG LTM configuration to the UE and UE replies RRCReconfigurationComplete message to MN.

For SN-initiated inter-CU SCG LTM, RAN2 agreed that the LTM cell switch command MAC CE is sent by source SN. It’s not clear whether “bye message” similar to the second RRCReconfigurationComplete message send from the UE to MN in CPAC procedure is needed. In CPAC scenario, the mobility is performed by the UE itself based on the evaluation of execution conditions, and the second RRCReconfigurationComplete message is to indicate MN the selected candidate PsCell (SN) and then indicated to the source SN. While for SN-initiated SCG LTM case, since UE’s LTM is triggered by  the source SN’s LTM cell switch command, in our view, the source SN indicating MN the selected candidate PsCell to MN can also be considered, which can reduce the signalling overhead on Uu.

Proposal 11: There’s two potential solutions to indicate the selected LTM candidate PsCell to MN:

Option 1: The UE sends a second RRCReconfigurationComplete message to the MN;

Option 2: The source SN indicate the selected candidate PsCell to the MN via Xn interaction.

    - Coexistence of different types of LTM

Based on the current progress, there’re four types  LTM: intra-CU MCG LTM, intra-CU SCG LTM, inter-CU MCG LTM and SN initiated inter-CU SCG LTM.

In Rel-18 intra-CU LTM discussion, the coexistence of intra-CU MCG LTM and intra-CU SCG LTM was discussed. Intra-CU MCG LTM and intra-CU SCG LTM can be configured simultaneously to the same UE, but LTM for simultaneous PCell and PSCell change is not supported in Rel-18.

Observation 3: In Rel-18, LTM for simultaneous PCell and PSCell change is not supported.

Besides, it was agreed in RAN2#126 meeting [3]that an LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. And RAN2 also agreed that Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. Though this agreement was achieved in the discussion for LTM for PCell change, we think it can be applied to both PCell change and PsCell change.

Proposal 12: UE could be configured with subsequent (SN-initiated) inter-CU SCG LTM and subsequent intra-CU SCG LTM simultaneously.

What’s more, it’s not clear whether intra-CU MCG LTM with inter-CU SCG LTM or inter-CU MCG LTM with intra-CU SCG LTM can be configured to the UE.

For the first case, the CP of MN is not changed, for example, the security key is not changed after intra-CU MCG LTM, therefore, the configuration of inter-CU SN LTM is still valid, therefore, the simultaneous configuration of intra-CU MN LTM and inter-CU SN LTM can be supported.

Proposal 13: Intra-CU MN LTM and inter-CU SN LTM can be configured to the UE simultaneously.

As to the second case, if both inter-CU MCG LTM with intra-CU SCG LTM is configured to the same UE, once MN/MCG changed, the configuration of intra-CU SCG LTM is not valid any more, then the network need to reconfigure  the UE with new intra-CU SCG LTM configuration, and the advantage of LTM for SCG does not exist. Therefore, it is proposed to not support the simultaneous configuration of inter-CU MCG LTM with intra-CU SCG LTM.

Proposal 14: Inter-CU MN LTM and intra-CU SN LTM cannot be configured to the UE simultaneously.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: In current LTM procedure, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

Observation 2: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

Observation 3: In Rel-18, LTM for simultaneous PCell and PSCell change is not supported.

Proposals:

Proposal 1: For L3 measurement results triggered LTM, it’s not needed for the source gNB-DU to indicate cell ID to the source gNB-CU after it sends LTM cell switch command to the UE.

Proposal 2: For the UE with capability of L1 measurement, the source gNB-CU cannot request the source gNB-DU to trigger LTM cell switch command based on L3 measurement results.

Proposal 3: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

Proposal 4: LTM request message(in HO request) is sent for each candidate cells, no matter whether they belong to the same gNB-CU or not.

Proposal 5: Serving gNB-CU transmits LTM configuration after it collects all candidate cells’ configuration or after UE’s success handover to the target gNB-CU.

Proposal 6: The serving gNB-CU needs to indicate data forwarding address information of candidate gNB-CUs, and this can be performed in LTM preparation phase or after every LTM cell switch.

Proposal 7: Inter-CU LTM for SCG can be  triggered by L3 measurement results.

Proposal 8: The procedure of SN-initiated SN change/conditional SN change specified in TS 37.340 can be taken as the baseline of SN-initiated inter-CU SCG LTM.

Proposal 9: Inter-CU SCG LTM indication should be included in SN change Required message from the source SN to MN and SN addition message from MN to candidate SN(s).

Proposal 10: It’s MN to send RRCReconfiguration message including inter-CU SCG LTM configuration to the UE and UE replies RRCReconfigurationComplete message to MN.

Proposal 11: There’s two potential solutions to indicate the selected LTM candidate PsCell to MN:

Option 1: The UE sends a second RRCReconfigurationComplete message to the MN;

Option 2: The source SN indicate the selected candidate PsCell to the MN via Xn interaction.

Proposal 12: UE could be configured with subsequent (SN-initiated) inter-CU SCG LTM and subsequent intra-CU SCG LTM simultaneously.

Proposal 13: Intra-CU MN LTM and inter-CU SN LTM can be configured to the UE simultaneously.

Proposal 14: Inter-CU MN LTM and intra-CU SN LTM cannot be configured to the UE simultaneously.

- References

- Report of 3GPP TSG RAN WG2 meeting #127;

- R2-2407771; L3 measurement based LTM support over F1; Source: RAN2; To: RAN3;

- Report of 3GPP TSG RAN WG2 meeting #126;

- TS 37.340 Multi-connectivity; Stage 2; (Release 18).