3GPP TSG-RAN2 Meeting #127bis			R2-2408093

Hefei, China, 14th-18th, Oct.

Agenda item:	8.2.4

Title:	Further consideration on Random Access for Ambient IoT

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting, RAN2 discussed some details regarding CBRA and CFRA, leaving the following FFS unresolved [1].

Agreements

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

This contribution will address the random access-related issue according to both the overall design and detailed procedures.

Discussion

RAN2 currently has a general consensus on the overall process of random access, e.g., to adopt 2-step and 3-step CBRA and CFRA. However, there remain some ambiguities regarding details. Additionally, some issues not clearly related to RAN1 and RAN2 are still unresolved. Therefore, from the perspective of ensuring timely endorsement of the SI and WI, RAN2 should proactively consider feasible solutions that it can decide independently.

  - Access Round Trigger Message

Currently, certain parameters for random access can be configured within the Ambient IoT Paging message, e.g., type of RA and related resource allocation information. However, during massive (at least hundreds of) device inventory and identification processes, if only a single access round is used, some device may need to remain in a prolonged monitoring or ON state, which will consume more energy. Therefore, in case massive devices are paged, the reader should have the capability to trigger multiple access round for the same service request from CN, which can be indicated by one paging or multiple paging message. Device can then randomly choose one of the access rounds to respond while remaining OFF or Sleep mode during the other Access Rounds, as illustrated Fig. 1.

Figure 1: An example of device state with and without multiple access rounds.

Observation 1a: When massive devices are paged, reader can issue multiple access round and device can randomly select one for CBRA, which is beneficial for device energy

Thus, it is necessary to introduce an Access Round Trigger Message. This approach also offers a potential benefit by allowing devices to have longer charging times before access. If paging and the access round trigger are decoupled, the reader or power source can charge the device before the access round trigger message, making more device have sufficient energy for access. Conversely, if devices attempt to access immediately upon receiving the paging message, they may have insufficient energy to complete the access process.

Observation 1b: Decoupling A-IoT paging message and access round trigger could give enough time for device to get fully recharged.

Therefore, RAN2 should consider the introduction of a dedicated Access Round Trigger message/indication.

Proposal 1a: Introduce an explicit access round trigger message to indicate the start of an access round, which can be optional.

Proposal 1b: If there are multiple access rounds for the same service request, device could randomly select one access round for initial random access.

Regarding the issue of paging rounds, it may be outside the scope of random access; however, a similar definition could be beneficial for future discussions.

  - Access occasion selection

In slotted-ALOHA, the sender typically needs to randomly select a slot to transmit data to avoid collisions; however, slotted-ALOHA is essentially a variant of TDMA. RAN1 is also considering the use of FDMA in Ambient IoT random access, meaning that access occasions may function as time-frequency resources similar to Resource Blocks. In the case of FDMA, if a tag can only transmit data at a specific frequency point, the issue of random selection is eliminated.

In CFRA, the problem may not arise if devices do not need to select access occasions, such as when there is only one access occasion or when resources are pre-allocated by the reader.

Proposal 2a: Paged device should randomly select time-domain access occasion in CBRA.

Regarding FDMA, since it is currently uncertain whether devices can support D2R communication across the entire available frequency range, if they do support it, tags should still randomly select frequencies to reduce the probability of collisions.

Proposal 2b: If supported, paged device should randomly select frequency-domain access occasion at least in CBRA.

It may involve further RAN1 work. Should consider RAN1’s agreement

  - Failure detection and indication

The primary purpose of the failure indication is to trigger re-access or retransmission-like procedure. Thus, the decision to introduce such indication depends on whether the device requires similar functionality.

Currently, the use cases for Ambient IoT primarily involve inventory and command scenarios, where the distinction between command and inventory can be regarded as whether additional D2R or R2D upper layer data transmission occurs after random access. It is evident that random access is essential for both inventory and command use case.

The importance of upper layer data following random access is challenging to evaluated at RAN, as it is largely determined by the CN and application layer. Fundamentally, even in the absence of retransmissions and re-access, the CN can always trigger a new access round to address upper layer data transmission failures. Therefore, for now, RAN should primarily focus on resolving random access failures. To address these random access failures, it is necessary to introduce a failure indication to signal devices to prepare for potential retransmission-like procedure or re-access.

Proposal 3: Introduce an R2D failure indication in random access to indicate device for potential re-access procedure.

For devices, in order to facilitate re-access or similar retransmission-like functions, they should maintain the failure state from the last access attempt. Otherwise, the reader can only repeatedly initiate non-specific random access to trigger the previously failed devices for re-access, resulting in significant inefficiencies in both time and energy.

Proposal 4: If a device encounters a D2R failure during random access, it should retain the failure status for potential re-access procedure.

The intention to introduce feedback for data transmission after random access is reasonable. However, considering that this is the first release of Ambient IoT, it is recommended let the CN to handle upper layer data transmission failure.

  - AS layer ID for Scheduling

In 3-step CBRA, device will send a 16-bit random ID in Msg1 to the reader to identify itself and resolve contention. Considering that the device ID may be relatively long and maybe NAS encrypted, it may not be directly usable by reader.

Observation 2a: Device ID may be NAS encrypted and too long to address single device.

Therefore, an additional AS ID can be introduced for scheduling purposes. This design is similar to the RFID approach, where ReqRN commands are used to request a new random ID generated by the device for subsequent data transmission. The rationale for not directly using the random ID, i.e., RN16 in Msg1 is that other devices may generate the same random ID and store it in device’s memory, leading to potential confusion, as shown in Fig. 2.

Figure 1: Device state with and without multiple access rounds.

Observation 2b: Using random ID in Msg1 to address device may trigger other device(s) that generates and stores same random ID before to response, as in RFID.

In Ambient IoT, this issue can be optimized by allowing the reader to assign the AS ID to device. This ensures that all devices within the reader's coverage area have a unique AS ID, enabling the reader to directly address the corresponding device using the AS ID. If a device has not been assigned an AS ID, it will not respond to paging or other commands that carry the AS ID. This approach effectively avoids the issue that random ID can address undesirable device.

Proposal 5: Reader is allowed to assign AS layer ID per device for efficient addressing

After a certain period, devices may need to release their AS IDs, as they could move outside the coverage area of the previous reader. If so, the assigned AS ID should also become invalid to avoid AS ID conflicts. Therefore, the validity period of the AS ID should be discussed. It is believed there may be three options available for now.

Proposal 6: If reader is allowed to assign AS layer ID, RAN2 should consider the valid period of AS ID, as follows:

Opt. 1: Permanent, AS ID is valid until reader assigns a new AS ID.

Opt. 2: Temporary, e.g., AS ID is valid before receiving paging message for different service request from CN.

Opt. 3: Time-based, e.g., AS ID is valid within a predefined persistence time

Specific details can be confirmed in subsequent discussions, but our preliminary analysis suggests that Opt. 2 is the most reasonable choice.

  - Device support of RA type

Currently, RAN has considered both 2-step and 3-step CBRA. Given the cost and complexity constraints of Ambient IoT devices, some of them may not support all random access methods, necessitating a down selection by RAN. But at least CFRA should be supported by all devices.

Proposal 7a: CFRA should be supported by all Ambient IoT device.

Based on current observations, the 3-step CBRA is more suitable for scenarios with a larger number of devices. 2-step method is more effective with fewer devices, it may consume more energy and time due to excessive collisions when the number of tags increases. Since the disadvantages of the 3-step method primarily affect a small number of tags, it is recommend selecting the 3-step method as a mandatory option.

Observation 3: 2-step CBRA requires colliding devices send more bits in Msg1, making it less suitable for massive devices inventory use case compared to the 3-step CBRA.

Another issue is that, in the absence of prior information, if there doesn’t exist a CBRA type supported by all devices, the reader cannot guarantee the inventory of all tags within a single access round. To avoid this situation, it is advisable to ensure that at least one CBRA option is supported by all devices.

Proposal 7b: For CBRA type that device mandatorily supports, RAN can consider the following options,

Opt. 1: Support both 2-step and 3-step CBRA.

Opt. 2: Support only 3-step CBRA.

For consideration of device cost and complexity, maybe down-selection is better.

  - Device population estimation

Many simulation and theoretical derivation results have proven that the time efficiency of slotted-ALOHA does not exceed 0.368 when the parameters are appropriately set i.e., when the number of access occasions equals the number of A-IoT devices. Please note that the efficiency here is defined as the ratio of the number of devices successfully inventoried to the number of time slots used.

It can be observed that for slotted-ALOHA, when the number of slots is equal to the number of devices to be inventoried, its efficiency is maximized at 0.368. Therefore, obtaining an accurate number of devices before random access is beneficial for efficiency enhancement, especially when CN doesn’t provide related information.

Observation 7a: Device population estimation can help reader in access occasion determination and access resource allocation, especially CN doesn’t provide such information.

Furthermore, in some other scenarios, e.g., counting the number of people in a shopping mall or park, counting vehicles in a parking lot, or inventorying in a third-party warehouse, it may be necessary to protect business or personal privacy, and the reader may not be allowed to read device information e.g. EPC ID, personal ID, vehicle identification number, in such cases, the only information that can be obtained is the population size of the devices. An accurate estimation of the device population size is also essential in these use cases.

Observation 7b: In privacy concern cases, reading device ID may cause legal issues, where only ambient device population estimation can be applied.

Therefore, it is recommended to study device population estimation for the efficiency and some privacy-critical use cases.

Proposal 8: RAN2 is suggested to study reader-based device population estimation method.

RFID also provides a simple approach for population estimation.

  - Details on 2-step CBRA

Previously, RAN2 concluded that both the random ID and device ID need to be included in Msg1 of the 2-step CBRA. However, after analysis, it is believed that the random ID could be unnecessary.

In the 3-step CBRA, the random ID is sent exclusively in Msg1. If random ID is collided, reader can directly terminate the current access occasion and issue the next one. Device can also consider its RA attempt as failure.

In contrast, in the 2-step CBRA, both the random ID and device ID are sent in Msg1. Even if reader detect a collision in random ID, it cannot terminate the device's transmission of the subsequent Device ID immediately since device is currently sending Msg1 and cannot receive any R2D message. Additionally, if necessary, the reader can assign a unique AS ID, which is more efficient than having the device report a potentially duplicated AS ID with other device(s).

Some viewpoints imply a benefit of capturing Random ID in 2-step Msg 1 is to use it as subsequent D2R failure/success indications. However, failure indications do not require the participation of the random ID, as the reader may not have received the random ID at all. Success indications can be confirmed through a simple ACK in the subsequent Msg2 or by including part of Msg1's content. Alternatively, if there are other data transmissions later, confirmation can be sent during the transmission of other upper layer data.

Observation 8: Reader cannot terminate the collided Msg1 transmission because device can only receive any R2D message after Msg1. Also, success/failure indication can be achieved without random ID in 2-step CBRA, as that in 3-step CBRA

Proposal 9: Exclude random ID in 2-step CBRA Msg1, device can send device ID directly.

If possible, Msg1 in 2-step CBRA should contain device ID only, the same as Msg3 in 3-step CBRA.

  - Details on 3-step CBRA

For both CFRA and CBRA, after the device sends Msg1, the reader will send Msg2. Therefore, the existence of Msg2 itself serves as a success/failure indication. This is similar for the 2-step CBRA, which does not require the introduction of additional indications.

The situation is more specific for 3-step CFRA; during random access, the device may not receive subsequent messages after sending Msg3 to indicate whether Msg3 was transmitted successfully. Thus, we believe it is necessary to introduce an optional message following Msg3 to indicate whether it has been successfully received. The reason why this message is optional is that receiving data can also consume energy for the device. The device could choose to maintain a low-power timer, and upon timer expiration without a response, it could assume the access was either successful or failed.

Proposal 10: Introduce an explicit indication after Msg3 in 3-step CBRA as a failure or success indication, which is optional.

Failure indication is more preferred since it’s more energy efficient. After all it’s rarer.

Conclusion

This contribution discusses the general and detailed aspect on Ambient IoT random access, as follows.

Access round and its trigger message:

Observation 1a: When massive devices are paged, reader can issue multiple access round and device can randomly select one for CBRA, which is beneficial for device energy

Observation 1b: Decoupling A-IoT paging message and access round trigger could give enough time for device to get fully recharged.

Proposal 1a: Introduce an explicit access round trigger message to indicate the start of an access round, which can be optional.

Proposal 1b: If there are multiple access rounds for the same service request, device should randomly select one access round for initial access.

Access occasion selection:

Proposal 2a: Paged device should randomly select time-domain access occasion in CBRA.

Proposal 2b: If supported, paged device should randomly select frequency-domain access occasion at least in CBRA.

Failure indication:

Proposal 3: Introduce an R2D failure indication in random access to indicate device for potential re-access procedure.

Proposal 4: If a device encounters a D2R failure during random access, it should retain the failure status for potential re-access procedure.

Assigned AS ID:

Observation 2a: Device ID may be NAS encrypted and too long to address single device.

Observation 2b: Using random ID in Msg1 to address device may trigger other device(s) that generates and stores same random ID before to response, as in RFID.

Proposal 5: Reader is allowed to assign AS layer ID per device for efficient addressing

Proposal 6: If reader is allowed to assign AS layer ID, RAN2 should consider the valid period of AS ID, as follows:

Opt. 1: Permanent, AS ID is valid until reader assigns a new AS ID.

Opt. 2: Temporary, e.g., AS ID is valid before receiving paging message for different service request from CN.

Opt. 3: Time-based, e.g., AS ID is valid within a predefined persistence time

Support of RA Type:

Proposal 7a: CFRA should be supported by all Ambient IoT device.

Observation 3: 2-step CBRA requires colliding devices send more bits in Msg1, making it less suitable for massive devices inventory use case compared to the 3-step CBRA.

Proposal 7b: For CBRA type that device mandatorily supports, RAN can consider the following options,

Opt. 1: Support both 2-step and 3-step CBRA.

Opt. 2: Support only 3-step CBRA.

Device population estimation:

Observation 7a: Device population estimation can help reader in access occasion determination and access resource allocation, especially CN doesn’t provide such information.

Observation 7b: In privacy concern cases, reading device ID may cause legal issues, where only ambient device population estimation can be applied.

Proposal 8: RAN2 is suggested to study reader-based device population estimation method.

Details on CBRA:

Observation 8: Reader cannot terminate the collided Msg1 transmission because device can only receive any R2D message after Msg1. Also, success/failure indication can be achieved without random ID in 2-step CBRA, as that in 3-step CBRA

Proposal 9: Exclude random ID in 2-step CBRA Msg1, device can send device ID directly.

Proposal 10: Introduce an explicit indication after Msg3 in 3-step CBRA as a failure or success indication, which is optional.

References

[1]	Draft_RAN2_127_Meeting_Report_v1, Chairlady