3GPP TSG-RAN2 Meeting #127bis			R2-2408094

Hefei, China, 14th-18th, Oct.

Agenda item:	8.7.4.1

Title:	Consideration on LCP Enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting, RAN2 excluded LCP restriction and reached some consensus on LCP enhancements, as outlined below [1].

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

This contribution will discuss some related issue on LCP enhancement.

Discussion

The main issue with LCP enhancements revolves around the effective timing of the additional priority and the subjects it applies to. This matter was discussed in RAN2#126 but did not yield a conclusion. RAN#105 confirmed that delay-aware scheduling needs to be studied for XR Phase 3 enhancement. It is believed it is still necessary to continue discussions on this issue.

  - LCP enhancement

The premise for utilizing delay/deadline information in LCP is that the data within the LCH must be delay-critical and capable of providing delay information, such as the remaining time of PSDB. If the LCH contains a mixture of delay-critical and non-delay- critical data, then delay-aware scheduling should only apply to the delay- critical data.

According to the current definition in TS38.323, only PDCP SDUs with a remaining time below a threshold value are considered as delay-critical SDU. Therefore, we believe that the indiscriminate use of additional priority does not align with the objective of enabling high XR capacity for delay-critical data.

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

Therefore, RAN2 needs to reconsider the issue of intra-LCH priority. At the very least, the remaining time should be taken into account during PDU set discard and DSR reporting, as it seems counterintuitive to make data having varying remaining times has equal opportunity for scheduling.

Moreover, since delay-aware LCP essentially overrides legacy QoS mechanisms, it is crucial to manage its impact on QoS to ensure that the QoS for other non-delay-critical data is preserved. For example, if a lower-priority LCH exists a substantial amount of non-delay-critical data, and the configured additional priority is relatively high, the continuous activation of this additional priority may inadvertently lead to starvation for other LCHs with non-delay-critical data that fall between the two priorities of delay-critical LCH.

Observation 1: If the additional priority applies to the whole data in the delay critical LCH without any constriction, it could cause starvation issue for other LCHs whose priority is between the additional and the original priority of delay critical LCH.

Furthermore, if the activation of additional priority is unconditional, then the simplest approach would be to modify the existing priority for that LCH. It achieves the same effect without any specification impacts. Considering the potential impact of the additional priority on other LCHs, it may be possible to configure a more moderate additional priority. However, it is challenging to evaluate whether this moderate additional priority would be sufficiently effective.

Therefore, it is recommended to introduce restrictions on the activation and/or scope of the additional priority, allowing for a more aggressive additional priority to be utilized to ensure “real” delay-critical data can be sent. One situation in which the additional priority must be activated is when the remaining time for delay-sensitive data approaches the discard threshold. In such case, it is essential to elevate the priority of this data to prevent its loss.

Proposal 1: To reduce the impact on legacy QoS and other LCHs, the additional priority is not applied to non-delay-critical data.

We recognize that the previous conclusion by RAN2 was not to discuss intra-LCH prioritization. However, we believe that this conclusion may lead to more significant issues. Therefore, from the perspectives of network performance and user experience, we sincerely recommend that RAN2 revisit this conclusion.

  - Intra-UE prioritization

Another issue pertains to the internal multiplexing and assembly within the UE, particularly during resource allocation for logical channels. Given the varying remaining times of SDU within logical channels, a more rational approach would be to prioritize the allocation of resources to data with shorter remaining times. This strategy would facilitate the transmission of a greater volume of data that is more likely to be discarded before the UL grant is exhausted.

Proposal 2: RAN2 should study the enhancement on UE resource allocation for LCH to let data with less remaining time ahead of those have more remaining time.

This modification may have some implications for TS38.321. However, we believe that the complexity will not be overly too high for UE to implementation. For the UE, it suffices to ensure that the SDUs in the LCH buffer are arranged in chronological order, with the earliest SDU always being transmitted first. This approach is commonly referred to as FIFO (first-in, first-out).

Conclusion

This contribution discusses the LCP enhancement for XR, as follows.

Intra-LCH prioritization:

Observation 1: If the additional priority applies to the whole data in the delay critical LCH without any constriction, it could cause starvation issue for other LCHs whose priority is between the additional and the original priority of delay critical LCH.

Proposal 1: To reduce the impact on legacy QoS and other LCHs, the additional priority is not applied to non-delay-critical data.

Intra-UE prioritization:

Proposal 2: RAN2 should study the enhancement on UE resource allocation for LCH to let data with less remaining time ahead of those have more remaining time.

References

[1]	RP-242390, Release 19 XR enhancements - Way forward on WID scope revision, Nokia (Moderator)