3GPP TSG-RAN WG2 Meeting #127bis	                                     R2-2408107

Hefei, China, Oct 14th – 18th, 2024

Source:	TCL

Title:	Discussion on XR rate control

Agenda item:	8.7.6

Document for:	Discussion and decision

- Introduction

- According to the latest 3gpp RAN#105 meeting[1], the following objective on XR rate control are reached:

Objective: Specify uplink congestion signaling [RAN2]: - Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

In this contribution, we aim to discuss the XR rate control issue in XR, particularly focusing on the MAC layer XR rate control signaling over downlink in RAN2 for XR applications.

- Discussion

According to the conclusion of the latest RAN#105 meeting, in order to enable RAN awareness and XR cross-layer optimization framework, it was agreed to support the UL congestion information indicated by gNB based XR rate control. This contribution mainly focuses on the relevant detailed issues. When the network side perceives UL congestion, the network will indicate the UL congestion situation to the UE through RAN signaling.

Since the WID has been passed and the base station needs to indicate the uplink congestion information of per QoS flow/per DRB to the UE at the MAC layer to support faster source rate adaptation. Therefore, we further need to discuss how the base station indicates the corresponding UL congestion information and what kind of UL congestion information is indicated to the UE. For example: The frequency of the UL congestion signaling and the congestion granularity can be further discussed.

- Issue1: How to indicate the UL congestion information to UE

According to the description of WID, "Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion", it can be seen that UL congestion is indicated by the MAC layer of gNB, that is, indicated by downlink messages. DL MAC CE can be considered to support real-time and high efficiency. In addition, since it is feasible for RAN to estimate the UL congestion information per-QoS flow and per-DRB in uplink without UE impacts. For the MAC layer, it is impossible to perceive the congestion situation of each QoS flow, but it can perceive the congestion situation of data in each DRB. Therefore, it is recommended that the MAC layer of the base station feed back the UL congestion information of per DRB to the UE through DL MAC CE.

Proposal 1: It is suggested that the MAC layer of the base station feed back the UL congestion information per DRB to the UE through DL MAC CE.

- Issue2: Contents contained in the uplink congestion feedback information

Since congestion can be divided into different levels, such as general congestion, moderate congestion, and severe congestion. Under different congestion levels, the processing mechanism of the UE may be different. For example, when severe congestion occurs, the UE may need to stop continuing to send uplink packets or discard obsolete packets or give up retransmission. When general or moderate congestion occurs, the application layer of the UE may adjust the rate or coding mechanism, or give up the transmission of data with low PSI. Therefore, when considering that the base station feeds back uplink congestion information to the UE, the congestion level can be classified and reported to support the flexible processing strategy on the UE side.

Observation 1: It is recommended that when the base station feeds back uplink congestion information to the UE, the congestion level should be classified and reported to support the flexible processing strategy on the UE side.

Observation 2: When the UE receives the uplink congestion information fed back by the base station, it can adjust the transmission rate, or discard outdated packets, or discard packets with low PSI, or stop retransmission.

In addition, regarding the content contained in the UL congestion information provided by the base station, except for the congestion level discussed above, the existing flow control/congestion mechanisms such as DDDS and ECN congestion feedback information can also be referred to, such as:

- a percentage of congestion level in UL for the data radio bearer

- Desired buffer size for the data radio bearer 

- Desired Data Rate

Proposal 2: The base station can feed back UL congestion information such as congestion level in UL, Desired buffer size for the data radio bearer, Desired Data Rate, etc.

- Issue3: Trigger mechanism of uplink congestion feedback information

In order to avoid frequent UL congestion feedback from the base station, the trigger mechanism of UL congestion feedback needs to be considered. Event-triggered UL congestion feedback can be considered, such as: UE's Poll mechanism or when the base station detects that UL congestion is severe congestion; in addition, the base station can also consider periodically feeding back UL congestion to the UE. However, from the perspective of congestion control performance and signaling overhead, it is recommended to trigger UL congestion feedback based on an event triggering manner.

Proposal 3: It is recommended that the base station perform uplink congestion information feedback based on an event-triggered manner.

In addition, when the base station detects that the congestion is relieved, the base station needs to further indicate to the UE to avoid the inefficiency of UL transmission caused by the UE always transmitting UL data in a congested mechanism. Therefore, the base station also needs to directly indicate which DRB's congestion is relieved through another DL MAC CE, for example, directly indicating that the DRB's congestion is relieved through 1 bit in the DL MAC CE.

Proposal 4: When the base station detects that the UL congestion of a certain DRB is relieved, the base station needs to indicate the congestion relief information to the UE.

- Conclusion

In this contribution, we provided our observations and proposals on XR rate control as follows:

- Proposal 1: It is suggested that the MAC layer of the base station feed back the UL congestion information per DRB to the UE through DL MAC CE.

- Observation 1: It is recommended that when the base station feeds back uplink congestion information to the UE, the congestion level should be classified and reported to support the flexible processing strategy on the UE side.

- Observation 2: When the UE receives the uplink congestion information fed back by the base station, it can adjust the transmission rate, or discard outdated packets, or discard packets with low PSI, or stop retransmission.

- Proposal 2: The base station can feed back UL congestion information such as congestion level in UL, Desired buffer size for the data radio bearer, Desired Data Rate, etc.

- Proposal 3: It is recommended that the base station perform uplink congestion information feedback based on an event-triggered manner.

- Proposal 4: When the base station detects that the UL congestion of a certain DRB is relieved, the base station needs to indicate the congestion relief information to the UE.

- References

- RAN#105 meeting agreements