3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408109

# Hefei, China, 14th – 18th Octorber, 2024

Agenda item:	8.9.3

Source:	Huawei, HiSilicon, CTCC, CATT, Nokia, Nokia Shanghai Bell, Apple, Turkcell, China Southern Power Grid

Title:	Discussion on the mechanisms for UL capacity enhancement based on simulation results

Document for:	Discussion and Decision

- Introduction

According to the RAN plenary #103 meeting, the following objectives on IoT-NTN [1] were agreed:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

RAN2 has made some general agreements:

Agreements in RAN2#126:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

Agreements in RAN2#127:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In this contribution, we will analyse the candidate solutions on achieving UL capacity enhancement based on companies’ simulation results.

- Discussion

## 2.1 Background of DSA and CRDSA

In the previous meetings, new mechanisms such as Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted ALOHA (CRDSA) for MSG3 were proposed with the intention to improve the UL capacity. The basic ideas of these mechanisms are:

DSA:

- UE selects multiple random slots to send multiple duplicated packets

- NW goes through the slots and decodes the packet with no collision with any other packet. The ones that have collision with other packet(s) will not be successfully decoded.

CRDSA:

- UE selects multiple random slots to send multiple duplicated packets

- NW goes through the slots in a Time Frame, and firstly decodes the packet with no collision with any other packet. Then NW finds the slots of the duplicated packets corresponding to this decoded packet and uses this packet to perform interference cancelation within these slots.

## 2.2 Analysis on DSA and CRDSA

Scenario requirements and contradiction to the motivation

DSA/CRDSA requires that the system load is medium or low. Otherwise if there are a lot of UEs accessing the NW, duplicated transmission may lead to collision happening in every slot or there may be collisions for more than two UEs in the same slot. In this case, NW is unable to solve the collision and decoding will fail. This will result in worse performance.

On the other hand, if the system load is medium or low, there is no such strong need to do the UL capacity enhancement anyway.

Observation 1: DSA/CRDSA requires that the system load is medium or low to avoid decoding failure, but if system load is medium or low, the UL capacity enhancement is not so necessary.

Obervation from the simulation results in the Annex

From the simulation results in the Annex, it can be observed that in typical cases of GEO scenario, the throughput gains of both DSA and CRDSA are marginal compared with SA. However, with the system load increasing, the capacity of DSA and CRDSA may even be worse than SA, when the system load is above some certain level. Note that here we use GEO scenario where the NTN channel changes less rapidly than LTE/MEO case. If considering the LEO case, it is expected the performance of CRDSA will be worse.

Observation 2: In typical cases, the throughput gains of both DSA and CRDSA are marginal compared with SA. The capacity of DSA and CRDSA can be even worse than SA, when the system load becomes higher than a certain level.

Impact on UE power consumption

For DSA and CRDSA, requiring the UE to always send duplicated packets anyhow will lead to unnecessary power and resource consumption, which is critical to IoT device. In this aspect, SA has the natural advantage since the duplicated transmission is only sent if the first attempt fails. Note that latency is not that important to IoT device.

Observation 3: DSA and CRDSA will lead to unnecessary power and resource consumption, which is critical to IoT device.

Complexity in both implementation and standard

For CRDSA, in implementation, even if it is feasible, the NW needs to store in memory the baseband samples corresponding to a full RA frame duration in order to perform an iterative decoding process [2]. Specifically, the demodulator has to search for each slot for all the possible preambles, and then performs the interference cancellation by regenerating at complex baseband level by re-encoding and modulating the decoded useful bits in the current slot location. These will be a big challenge for both the algorithm implementation and storage of NW onboard.

In standard, at least the following needs to be discussed and specified:

- Simulation and performance estimation

- How to determine the Time Frame, and preamble (which should be UE specific)

- The UE behavior of slot selection, preamble selection/attachment, duplicated transmission

Observation 4: Supporting of CRDSA leads to higher complexity for NTN system and brings challenges for both the algorithm implementations and storage on satellites.

Considering the marginal performance gain versus the dramatic complexity increment and also the limited TUs for this WI, it is proposed not to support CRDSA in Rel-19 IoT NTN and further discuss DSA by taking into account its performance in the high-load scenarios.

Proposal 1a: RAN2 concludes to not support CRDSA in this release.

Proposal 1b: RAN2 discusses whether to support DSA, by taking into account its performance in the high-load scenarios.

- Conclusion

In this paper, we give some simulation results on the proposed solutions on achieving capacity enhancement for uplink and have the following proposals:

Observation 1: DSA/CRDSA requires that the system load is medium or low to avoid decoding failure, but if system load is medium or low, the UL capacity enhancement is not so necessary.

Observation 2: In typical cases, the throughput gains of both DSA and CRDSA are marginal compared with SA. The capacity of DSA and CRDSA can be even worse than SA, when the system load becomes higher than a certain level.

Observation 3: DSA and CRDSA will lead to unnecessary power and resource consumption, which is critical to IoT device.

Observation 4: Supporting of CRDSA leads to higher complexity for NTN system and brings challenges for both the algorithm implementations and storage on satellites.

Proposal 1a: RAN2 concludes to not support CRDSA in this release.

Proposal 1b: RAN2 discusses whether to support DSA, by taking into account its performance in the high-load scenarios.

- References

RP-240449, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random-Access Scheme for Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007.

3GPP TR 38.811, Study on New Radio (NR) to support non-terrestrial networks

3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

- Annex

- 5.1 Simulation from Huawei

### 5.1.1 Simulation assumptions

The following assumptions are made:

- EDT occasion is defined as the time slot to transmit Msg3 without Msg1/RAR. Specifically, in time domain, EDT occasion duration is determined by RU number NRU and repetition number NRep. While in frequency domain, each EDT occasion contains NSCEDT sub-carriers that are allowed for EDT, which can be configured by upper layer. For single-tone transmission with 3.75kHz sub-carrier spacing, the maximum number of sub-carriers is 48 within the total bandwidth of 180 kHz.

- For SA, UE can randomly choose one of the N total EDT occasions to transmit Msg3. Transmission in a non-colliding EDT occasion is regarded as successful. As shown in Figure 1, UE2, UE3, UE6, UE7 can be successfully received and decoded by NW due to no collision.

- For DSA/CRDSA, UE randomly chooses K out of the N total EDT occasions (K≤N). For each chosen EDT occasion, UE randomly choose one frequency resource to transmit the Msg3.

- For CRDSA, NW first detects a packet with no collision with other packets and uses this packet to perform interference cancelation in multiple iterations. As illustrated in Figure 3, UE1, UE3, UE4, UE6, UE7 can be successfully decoded by NW directly in iteration #1. Then the decoding results are used to perform interference cancelation in the following iterations.

Figure 1: Example of SA with N=3 EDT occasions. Sub-carrier number NSCEDT=48.

Figure 2: Example of DSA with N=3 EDT occasions. Sub-carrier number NSCEDT=48, Duplication number K=2.

Figure 3: Example of CRDSA with N=3 EDT occasions. Sub-carrier number NSCEDT=48, Duplication number K=2.

### 5.1.2 Simulation for ideal channel condition

The main simulation configurations for ideal case to evaluate the packet error rate and capacity of SA, DSA, and CRDSA are depicted in Table 1.

Table 1: Simulation configurations

| NTN Scenario | GEO / 3GPP Set-1 |
| --- | --- |
| NTN Channel | AWGN |
| Sub-carrier spacing (SCS) | 3.75 kHz |
| Number of sub-carriers ( NSCEDT) | 48 |
| Msg3-EDT TBS ( Nbitspkt ) | 408 bits |
| Number of RU per Repetition ( NRU ) | 4 |
| Repetition number per replica ( NRep ) | 4 |
| Single EDT duration ( TEDT ) | 512ms |
| FEC Coding rate ( r ) | 0.53 |
| Number of EDT occasions ( N ) | 2 or 3 or 4 |
| Number of replicas ( K ) | 2 or 3 |
| CRDSA SIC iterations ( Niter ) | 4 |
| Power Threshold | 3dB |
| Simulation number | 10000 |

Simulation results:

In simulation for the ideal case, we assume that the UE traffic model is Poisson distributed and obtain the packet error rate by using Monte Carlo Simulation.

In Figure 4, we obtain the packet error rate and throughput in different average load (which is the ratio between number of UEs sending packets and number of the total resource in all EDT occasions) for SA and DSA with replicas 2 and 3.

It can be observed that, by adopting the DSA scheme, the packet error rate of DSA with 2 replicas is lower than SA when the average load is lower than 0.48, where the gap becomes smaller and smaller with the average load increasing from 0.1 to 0.5. The packet error rate of DSA with 3 replicas is slightly better than 2 replicas when average load is lower than 0.27, and then become worse when the load increases. Similarly, regarding the system throughput, DSA with 2 replicas is slightly larger than SA when the average load is lower than 0.48, but gets smaller than SA when the average load is higher than 0.48.

Figure 4: Packet error rate and throughput of SA and DSA with 2/3 replicas

Observation A.1a: The PER of DSA (2 replicas) is better than SA when the average load is lower than 0.48, The PER of DSA (3 replicas) is better than SA when the average load is lower than 0.38.

Observation A.1b: The throughput gain of DSA is marginal compared with SA when the average load is lower than 0.48. The throughput of DSA becomes worse than SA when the average load is higher than 0.48.

In Figure 5, we further obtain the packet error rate and throughput in different average load for CRDSA with replicas 2 and 3. It can be observed that, for ideal case, CRDSA with 2 replicas and 3 replicas have advantages in PER and throughput compared with the SA.

However, for CRDSA, in reality, since the channel condition is time varying, even for the same packet, the received signal will be different in different time slots. This needs to be considered in the simulation regarding interference cancelation. So we also provide the link level simulation results in the section 2.2.3, which is closer to the reality and is more informative.

Figure 5: Packet error rate and throughput of SA, DSA, and CRDSA with 2/3 replicas, the power residual coefficient of CRDSA is 0.1

Observation A.2: In ideal case, the performance of PER and throughput of CRDSA is better than SA. However, in reality, the time varying channel condition has a large impact on the performance, so link level simulation result is more informative.

### 5.1.3 Simulation for non-ideal channel condition

The configurations of EDT occasions in time and frequency domain are the same as that for the ideal case. The LLS assumptions for EDT transmission are provided in the following table.

For the simulation based on the non-ideal channel condition, the NTN channel models according to 38.811[3] and the satellite parameters and UE characteristics according to 38.821[4] are adopted. The complete physical layer process (coding, modulation, air interface, demodulation, decoding) is included to simulate the actual scenario. In LLS, the interactions and interferences between the replicas and different UEs can be better simulated.

Table 2 Link level assumptions

| Parameter | value |
| --- | --- |
| Scenario | GEO12.5, 3GPP Set-1 |
| Channel | NTN-TDL-C The channels from different UE are independent. |
| Carrier frequency | 2 GHz |
| NPUSCH CNR | 0.58dB |
| TBS | 408 bits |
| SCS | 3.75 kHz |
| tone | single |
| Single UE PER | 0.01 |
| Power imbalance | No |
| Number of RU per Repetition | 4 |
| Number of Repetition per replica | 4 |
| Number of replica (for DSA/ CRDSA) | 2 |
| Modulation | QPSK |
| FEC coding rate | 0.53 |
| Carrier timing offset | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 0 for GEO. |
| Carrier frequency offset | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |
| Receiver algorithm | MMSE |
| Channel estimation | Real channel estimation |

The aloha scheme (SA, DSA, CRDSA) simulation assumptions are provided in the following table.

Table 3 Assumptions for SA/DSA/CRDSA

| Parameter | value |
| --- | --- |
| Number of resources per occasion | 48 |
| Number of EDT occasions | 2 |
| Duration of EDT occasion | 0.512 s |
| Interval time of EDT occasions | 0.128 s |
| Retransmission | No |
| SIC Niter (only for CRDSA) | 4 |

The simulation results are provided in Figure 6:

Figure 6: The simulation results of Packet error ratio and throughput

The throughput and PER in Link level simulation (with NTN channel) are demonstrated in Figure 6. The trend of the curves of SA and DSA are almost the same with the results for ideal case, since time varying channel condition has little impact on these two mechanisms.

However, for the CRDSA, the NTN channel changes rapidly in time domain because of the atmosphere and environment, it is difficult for the eNB to perform efficient interference cancellation in reality. There will be residual interference on the packet being decoded after the interference cancellation. And in the worst case, the interference cancellation may make the decoding harder if it turns out to be adding interference. This is why in the link level simulation, the CRDSA’s performance gain is marginal compared to SA. Note that here we use GEO scenario where the NTN channel changes less rapidly than LTE/MEO case. If considering the LEO case, it is expected the performance of CRDSA will be worse.

- Observation A.3: In simulation based on the non-ideal channel condition of GEO case, the performance gain of CRDSA is marginal compared to SA, due to the impact of time varying channel condition in reality.

- In 3GPP system, DMRS can be used by the physical layer to estimate the channel condition. However, DMRS cannot be used here for CRDSA. For single tone NB-IoT, there is only one DMRS and for multi-tone NB-IoT there are two DMRS. In both cases, DMRS is far from enough to make sure different UEs use different DMRS in case of collision. In addition, even if the number is enough, same issue exists on how to make sure different UEs use different DMRS and NW knows which UE uses which.