3GPP TSG-RAN WG2 Meeting #127bis	R2-2408110

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.9.4

Source:	Huawei, HiSilicon, Turkcell, China Southern Power Grid

Title:	Support of PWS for NB-IoT

Document for:	Discussion and Decision

- Introduction

In RAN plenary #105 meeting, the objective regarding the support of PWS for NB-IOT was approved [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide our consideration on how to support PWS for NB-IoT.

- Discussion

According to the current TS 36.331, the following system information is supported in LTE (eMTC included) for PWS:

      - SystemInformationBlockType10 contains an ETWS primary notification

      - SystemInformationBlockType11 contains an ETWS secondary notification

      - SystemInformationBlockType12 contains an CMAS primary notification

However, there is no such system information for NB-IoT UEs. To support public emergency warning including ETWS and CMAS for NB-IoT UEs, NB-IoT PWS system information should be introduced. The structures of the three existing system information can be used as a baseline for NB-IoT system information.   Therefore, we propose to introduce SystemInformationBlockType10-NB for ETWS primary notification, SystemInformationBlockType11-NB for ETWS secondary notification, and SystemInformationBlockType12-NB for CMAS notification. There should be no restriction that this system information can only be broadcast in IoT-NTN.

Proposal 1a: Introduce SystemInformationBlockType10-NB for ETWS primary notification, SystemInformationBlockType11-NB for ETWS secondary notification, and SystemInformationBlockType12-NB for CMAS notification.

Proposal 1b: The newly introduced NB-IoT system information can also be sent in TN.

Besides, in the current procedural text, since PWS is not supported for NB-IoT, there are multiple places where NB-IoT is excluded such as below highlighted text. To support PWS for NB-IoT, these procedural restrictions need to be removed.

- The UE: - Monitors a Paging channel and/ or System Information Block Type 1 contents to detect system information change, for ETWS capable UEs, ETWS notification, and for CMAS capable UEs, CMAS notification (not applicable for BL UEs, UEs in CE and NB-IoT UEs);

Proposal 2: Remove the restrictions throughout the specification that prevent supporting PWS for NB-IoT.

Regarding whether to support the indication of intended service area for ETWS, it is under discussion in Rel-19 NR NTN. An LS was sent to SA1 during the last meeting, and RAN2 is waiting for the feedback from SA1. Since the motivation is the same in IoT-NTN as in NR NTN. To avoid duplicated discussion, IoT NTN can simple wait for the conclusion in NR NTN.

Proposal 3: Whether to support the indication of intended service area for ETWS in IoT NTN should waits for the conclusion in NR NTN.

- Conclusion

This contribution discussed the support of PWS for NB-IoT. Proposals are summarized as follows:

Proposal 1a: Introduce SystemInformationBlockType10-NB for ETWS primary notification, SystemInformationBlockType11-NB for ETWS secondary notification, and SystemInformationBlockType12-NB for CMAS notification.

Proposal 1b: The newly introduced NB-IoT system information can also be sent in TN.

Proposal 2: Remove the restrictions throughout the specification that prevent supporting PWS for NB-IoT.

Proposal 3: Whether to support the indication of intended service area for ETWS in IoT NTN should waits for the conclusion in NR NTN.

- References

RP-242397, Revised WID: Non-Terrestrial Networks (NTN) for IoT Phase 3.