3GPP TSG-RAN WG2 Meeting #127bis		R2-2408119

Hefei, China, 14th – 18th Oct. 2024

Source:	vivo

Title:		Discussion on conditional LTM

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

In RAN#105 meeting, the following objective was agreed related to conditional LTM[1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we will discuss the issues related to general conditional LTM procedures, including:

- Early DL/UL synchronization;

- Conditional LTM event evaluation;

- Conditional LTM execution and completion.

- Discussion

The following Figure 1 is a simplified signaling procedure for conditional LTM.

Figure 1. Example of signalling procedure for conditional LTM

Similar to R18 LTM procedure, the R19 conditional LTM procedures can consist of following phases:

- Phase 1: LTM preparation

- Phase 2: Early sync

- Phase 3: Conditional LTM event evaluation and execution

- Phase 4: Conditional LTM completion

And we would like to discuss the conditional LTM related issues based on these phases.

  - Early DL/UL synchronization

In Rel-18 LTM, the UE performs early DL synchronization, i.e. the TCI states of the LTM candidate cells can be activated in advance before any of those cells become the serving cell, and this facilitates a faster cell switch to one of those cells when cell switch is triggered[2]. However, in conditional LTM, as the L1 measurement result is used by the UE itself for conditional LTM evaluation and the cell switch will be triggered by the UE itself, it seems that the network does not have necessary measurement information to determine whether the TCI state of any conditional LTM candidate cells should be activated or not in advance. Therefore, we understand early DL synchronization could be unworkable in conditional LTM.

Observation 1: In conditional LTM, network does not have measurement information to determine whether the TCI state of any conditional LTM candidate cells should be activated or not in advance.

Proposal 1: Early DL synchronization is not performed in conditional LTM, i.e. network does not activate the TCI state in advance for candidate cell in conditional LTM.

On the other hand, for early UL synchronization, more discussions are needed.

In our understanding, early TA acquisition procedure which makes RACH-less LTM cell switch possible is an important feature to make full use of the latency reduction advantages of LTM. RACH-less procedure helps with facilitating a faster cell switch compared with legacy L3 mobility, and we think it should be supported for conditional LTM as well. On the other hand, RACH-based conditional LTM is supported anyway, so we propose:

Proposal 2: Both RACH-based and RACH-less conditional LTM are supported.

In Rel-18 LTM, to support RACH-less procedure, early TA acquisition procedure is triggered by PDCCH order or realized through UE-based TA measurement as configured by RRC[2]. We understand at least the UE-based TA measurement procedure can be reused, so that the UE can apply the TA value measured by itself and performs RACH-less LTM upon conditional LTM execution. If UE-based TA measurement is supported, at least configured grant resource should be configured for the first UL transmission at RACH-less conditional LTM, considering there may be no early TCI state activation and in this case the dynamic grant may not be available for conditional LTM case.

However, for PDCCH order-based or early RACH with RAR-based TA acquisition procedure, it can be further discussed.

For PDCCH-order based TA acquisition, it is questionable when should the network send the PDCCH order to UE, considering the conditional LTM is evaluated and executed by the UE itself, we think maybe UE could trigger the early TA acquisition by itself.

For early RACH with RAR-based TA acquisition, it is more motivated to support it in conditional LTM and this is why an FFS was left on this in RAN2 #126 meeting.

RAN2 #126 Agreement: RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Therefore, we propose:

Proposal 3: For early UL synchronization, UE-based TA measurement is supported, i.e. the UE may perform UL synchronization with conditional LTM candidate cell(s) before execution. FFS whether early-RACH based TA acquisition is supported.

Proposal 4: If UE-based TA measurement is supported, CG resources should be configured for the first UL transmission at RACH-less conditional LTM.

  - Conditional LTM event evaluation

In RAN2 #127 meeting, it was agreed for event-triggered L1 measurement report that:

RAN2 #127 Agreements: 6. MAC layer handles the event evaluation and measurement report triggering.

For conditional LTM, we think it is also straightforward to make the MAC layer responsible for the event evaluation for conditional LTM. So, a simple proposal is given:

Proposal 5: MAC layer handles the event evaluation triggering for conditional LTM.

Before we discuss the conditions for events used in conditional LTM, we think it is appropriate to start with whether we should support cell level measurement result or not. Cell level measurement has been discussed for several meetings for event-triggered L1 measurement report, and it finally concludes in RAN2 #127 that:

RAN2 #127 Agreements: Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

Cell level measurement means that we should consider multiple beams in order to derive the proper cell quality to trigger conditional LTM execution. In our understanding, for L1 measurement reporting, beam level may be enough as anyway whether to trigger LTM is based on network implementation and the network can have the whole picture for all beams, while the cell level measurement event is needed in conditional LTM as the handover is triggered by UE itself and the event may need to consider measurement results for multiple beams to derive the cell quality, otherwise the UE may e.g. switch to a new serving cell with only one good beam, which will cause the frequent cell switch and is not robust.

Observation 2: Relying on beam-level measurement result for conditional LTM can make UE switch to a new serving cell e.g. with only one good beam.

Therefore, we prefer to support cell level L1 measurement for conditional LTM event evaluation.

Proposal 6: Trigger event for conditional LTM is based on cell level L1 measurement result which is derived from measurement results of one or multiple beams associated to one cell.

As for how to derive the cell level L1 measurement result, there can be different options. E.g.:

- Option 1: Multiple beams (all or best N) should all satisfy the conditional event respectively at the same time to trigger conditional LTM;

- Option 2:  Similar to L3 measurement cell quality derivation, the average quality of multiple beams should be used to trigger conditional LTM.

Taking the measurement model for L3 as the example[2]:

Figure 9.2.4-1: Measurement Model

According to the above figure, option 1 means that we use the beam quality at point A1 for conditional LTM event evaluation, e.g. comparing each beam of neighbour cell(s) with current beam. Option 2 means that we use the beam quality at point B for conditional LTM event evaluation. There are some benefits and drawbacks for both option 1 and 2:

Table 1. Pros and Cons of different options to derive cell level L1 measurement result

| Option | Pros | Cons |
| --- | --- | --- |
| Option 1 (Multiple beams satisfy the conditional event respectively at the same time to trigger conditional LTM) | May reuse the events defined for event triggered L1 measurement reporting | Complicated on e.g. how to combine different comparing results for different beams |
| Option 2 (Similar to L3 measurement cell quality derivation, the average quality of multiple beams should be used to trigger conditional LTM) | Can reuse the legacy L3 cell quality derivation framework | Quite similar to legacy L3 framework (without L3 filter) |

In order to simplify the define and reuse legacy framework, we think it is preferable to go for option 2.

Proposal 7: Similar to L3 measurement cell level quality derivation, the average quality of multiple beams (i.e. measurement result at point B in Figure 9.2.4-1 of TS38.300) is used for conditional LTM event evaluation.

Then, the conditional LTM events can be, e.g.:

- Event LTMC3: Candidate cell quality (based on multiple beams of one candidate cell) becomes amount of offset better than serving cell quality (based on multiple beams of serving cell, e.g. multiple beams of activated TCI state);

- Event LTMC4: Candidate cell quality (based on multiple beams of one candidate cell) becomes better than absolute threshold;

- Event LTMC5: Serving cell quality (based on multiple beams of serving cell) becomes worse than absolute threshold1 AND candidate cell quality (based on multiple beams of one candidate cell) becomes better than another absolute threshold2.

Therefore, we propose:

Proposal 8: Similar events as LTM 3/4/5 is defined for conditional LTM, e.g.:

- Event LTMC3: Candidate cell quality (based on multiple beams of one candidate cell) becomes amount of offset better than serving cell quality (based on multiple beams of serving cell, e.g. multiple beams of activated TCI state);

- Event LTMC4: Candidate cell quality (based on multiple beams of one candidate cell) becomes better than absolute threshold;

- Event LTMC5: Serving cell quality (based on multiple beams of serving cell) becomes worse than absolute threshold1 AND candidate cell quality (based on multiple beams of one candidate cell) becomes better than another absolute threshold2.

  - Conditional LTM execution and completion

For execution phase, there are multiple questions to be answered, e.g.

- Whether/how to define the supervisor timer for conditional LTM?

- For RACH-based procedure, how can UE acquire the CFRA resource for the initial access in target cell?

- For RACH-based and RACH-less procedure, how to determine the conditional LTM is successfully complete?

- What should be the UE behavior upon conditional LTM failure?

For second question, which is about the CFRA resources, it is quite different to legacy LTM as legacy LTM switch command MAC CE can include the CFRA information such as Random Access Preamble index, which UL carrier to transmit the PRACH, SS/PBCH index, PRACH Mask index, and Repetition number[3]. For conditional LTM, if we would like to support CFRA, the CFRA resource may only be included in the candidate cell configurations of conditional LTM. However, it is questionable whether to reserve those resources considering the conditional LTM may never be executed for a certain cell. We understand this may be left to candidate cell implementation, and if CFRA resource is included, CFRA can be performed by the UE.

Proposal 9: For RACH-based conditional LTM procedure, CFRA is performed when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

For other questions, we understand we should reuse the legacy LTM as much as possible. LTM supervisor timer was discussed in Rel-18 and T304 is finally extended to cover the LTM case, as follows[4]:

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T304 | <omitted...> Also, for the MCG and SCG upon an indication from lower layer that an LTM cell switch procedure is triggered and, for the MCG, upon performing an LTM cell switch procedure following cell selection performed while timer T311 is running. | Upon successful completion of random access on the corresponding SpCell. Upon receiving an indication from lower layers of successful completion of Rach-less handover, or upon receiving an indication from lower layers of successful completion of an LTM RACH-less cell switch. <omitted...> | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, or an LTM cell switch procedure, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. <omitted...> |

In our understanding, T304 can be simply reused for conditional LTM, i.e. same UE behavior as R18 LTM.

For stop condition of T304, which is how to determine the conditional LTM is successfully complete, it seems that legacy LTM can also be reused, as follows[2]:

8. The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

And for UE behavior upon conditional LTM failure, we think it is also reasonable to support fast recovery to a candidate cell which is a conditional LTM candidate cell. Generally speaking, almost all the UE behaviors for conditional LTM execution and completion are quite similar, because the main difference for conditional LTM and legacy LTM is on the configuration and evaluation phase before LTM cell switch is triggered.

Therefore, we propose:

Proposal 10: T304 is reused as the supervisor timer for conditional LTM, i.e. the timer starts upon conditional LTM execution, stops upon successful conditional LTM procedure completion, and expiry with RRC re-establishment procedure by the UE.

Proposal 11: For RACH-based conditional LTM, the UE considers the conditional LTM procedure is successfully completed when the RACH is successfully completed.

Proposal 12: For RACH-less conditional LTM, the UE considers the conditional LTM procedure is successfully completed when the UE determines that the network has successfully received its first UL data.

Proposal 13: For conditional LTM failure, fast recovery to a candidate cell of conditional LTM is supported. Details are FFS.

- Conclusion

In this contribution, we discussed the issues related to conditional LTM. We have the following Observations and Proposals:

Observation 1: In conditional LTM, network does not have measurement information to determine whether the TCI state of any conditional LTM candidate cells should be activated or not in advance.

Observation 2: Relying on beam-level measurement result for conditional LTM can make UE switch to a new serving cell e.g. with only one good beam.

Proposal 1: Early DL synchronization is not performed in conditional LTM, i.e. network does not activate the TCI state in advance for candidate cell in conditional LTM.

Proposal 2: Both RACH-based and RACH-less conditional LTM are supported.

Proposal 3: For early UL synchronization, UE-based TA measurement is supported, i.e. the UE may perform UL synchronization with conditional LTM candidate cell(s) before execution. FFS whether early-RACH based TA acquisition is supported.

Proposal 4: If UE-based TA measurement is supported, CG resources should be configured for the first UL transmission at RACH-less conditional LTM.

Proposal 5: MAC layer handles the event evaluation triggering for conditional LTM.

Proposal 6: Trigger event for conditional LTM is based on cell level L1 measurement result which is derived from measurement results of one or multiple beams associated to one cell.

Proposal 7: Similar to L3 measurement cell level quality derivation, the average quality of multiple beams (i.e. measurement result at point B in Figure 9.2.4-1 of TS38.300) is used for conditional LTM event evaluation.

Proposal 8: Similar events as LTM 3/4/5 is defined for conditional LTM, e.g.:

- Event LTMC3: Candidate cell quality (based on multiple beams of one candidate cell) becomes amount of offset better than serving cell quality (based on multiple beams of serving cell, e.g. multiple beams of activated TCI state);

- Event LTMC4: Candidate cell quality (based on multiple beams of one candidate cell) becomes better than absolute threshold;

- Event LTMC5: Serving cell quality (based on multiple beams of serving cell) becomes worse than absolute threshold1 AND candidate cell quality (based on multiple beams of one candidate cell) becomes better than another absolute threshold2.

Proposal 9: For RACH-based conditional LTM procedure, CFRA is performed when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

Proposal 10: T304 is reused as the supervisor timer for conditional LTM, i.e. the timer starts upon conditional LTM execution, stops upon successful conditional LTM procedure completion, and expiry with RRC re-establishment procedure by the UE.

Proposal 11: For RACH-based conditional LTM, the UE considers the conditional LTM procedure is successfully completed when the RACH is successfully completed.

Proposal 12: For RACH-less conditional LTM, the UE considers the conditional LTM procedure is successfully completed when the UE determines that the network has successfully received its first UL data.

Proposal 13: For conditional LTM failure, fast recovery to a candidate cell of conditional LTM is supported. Details are FFS.

- References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN meeting #105, Melbourne, Australia, Sep 9-12, 2024;

- 3GPP TS 38.300 V18.3.0 (2024-09);

- 3GPP TS 38.321 V18.3.0 (2024-09);

- 3GPP TS 38.331 V18.3.0 (2024-09);