3GPP TSG-RAN WG2 Meeting #127bis		R2-2408121

Hefei, China, 14th – 28th Oct. 2024

Source:	vivo

Title:	Discussion on LCP enhancements for XR

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

- Introduction

In RAN#105, the WID of R19 XR was updated, and there were the following descriptions regarding LCP enhancement [1]:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In this contribution, the LCH priority enhancements on handling using delay/deadline information of packets are discussed.

- Discussion

In RAN2#126 [2], there were the following agreements regarding LCP enhancements:

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In RAN2#127 [3], there were the following agreements regarding LCP enhancements:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In the following, the open issues for LCP enhancements are discussed respectively for LCH priority enhancements.

In RAN2#126 meeting, there is an FFS on whether the additional priority should be applied only for the delay-critical data of the LCH or the whole LCH. In the following, this issue is discussed in two aspects:

- If the additional priority is only applied to the delay-critical data, one LCH may have two applicable priorities during one MAC PDU construction. Upon one MAC PDU construction, i.e. the additional priority of an LCH is determined due to the delay-critical data existence of this LCH and later on the delay-critical data of this LCH is included in the MAC PDU according to this additional priority. Afterwards, the MAC PDU may still have the remaining capacity to accommodate more data after including all delay-critical data of the LCH into the MAC PDU, the original priority has to be applied for the LCH for allocating resources for the non-delay-critical data for the LCH. This means that the UE has to evaluate the prioritization between LCHs twice in correspondence to one MAC PDU construction, which causes considerable complexity at the UE side.

- If the additional priority is applied to the whole LCH, for an LCH configured with additional priority, whether additional priority or original priority should be applied needs to be determined per MAC PDU basis, i.e. only single priority (either original priority or the additional priority) can be applied for one LCH during one MAC PDU construction. Once the additional priority is determined for one LCH with delay-critical data upon one MAC PDU construction, the UE includes the delay-critical data and non-delay-critical data of this LCH into the MAC PDU according to the additional priority. In such way, the UE only evaluates the prioritization between LCHs once for one MAC PDU construction.

- Applying additional priority only for delay-critical data for an LCH may require the UE to evaluate the priorities among LCHs twice for the LCH corresponding to one MAC PDU construction.

Based on the above analysis and the already conclusion that high complexity solution for LCP enhancement should be avoided, we prefer that the priority should be applied to the whole LCH.

- Once the additional priority is determined for an LCH, the additional priority should be applied to all data of the LCH, including both delay critical data and non-delay critical data.

- Whether additional priority or original priority is applied should be determined for a LCH upon a MAC PDU construction, the resulted priority (either original priority or additional one) should be applied for the whole MAC PDU construction without change.

For a first LCH with delay-critical data, the gNB may issue UL grants to speed up the delay-critical data transmissions. After receiving the UL grants, the UE may prioritize the resource allocation for this LCH according to the additional priority of this LCH. However, when the UE has a large amount of delay-critical data for the first LCH, after the UE has allocated radio resource up to Bj for the first LCH, there are still remaining delay-critical data for the first LCH. According to existing multiplexing procedure, the UE will allocate the remaining resource for the second LCH of lower priority. After radio resource allocation for the second LCH, there may be no remaining capacity or the remaining capacity of the MAC PDU can be not enough to contain all the remaining delay-critical data of the first LCH.

In another instance, when there can be certain scheduling restriction, e.g. due to measurement gap (MG) overlapping with the PDB/PSDB window of a frame burst, the actual available time period(s) for transmission of the frame burst after DSR triggering could be further reduced due to the overlapping between the MG and the remaining time period. In this case, even when the UE get enough UL grants providing enough capacity for the delay-critical data of a LCH, the UE may not be able to include all delay-critical data of this LCH into the corresponding MAC PDUs due to the restriction of the Bj of this LCH and some capacity of the UL grants intending for the delay-critical data of this LCH are allocated to transmit the non-delay-critical data of other LCHs instead according to the existing multiplexing mechanism.

- For MAC PDU construction, the LCH with delay-critical data may not be able to be allocated enough radio resource due to its buckets (Bj) exhausted.

To speed up the delay-critical data transmission, the UE should be temporarily allowed to allocate radio resources exceeding the rate limit (Bj limit).

- In addition to LCH priority adaptation, RAN2 should also consider the option to allow UE to temporarily raise the rate limit (e.g. Bj) of the LCH with delay-critical data.

There is still one further open issue on whether to reuse the existing remaining time threshold or introduce a new threshold to determine the delay-critical data of an LCH for LCH priority enhancements. As far as our understanding, that the UE request UL grant via DSR is the last method to speed up the delay-critical data transmission. If the UE has a large amount of delay critical data, after DSR reception, the gNB may not be able to issue enough UL grants for the UE so that the UE can transmit all delay-critical data.

To reduce the risk that there is a large amount of delay-critical data (in accordance with the remaining time threshold of DSR) when DSR is triggered, a separate remaining time threshold should be introduced to determine the delay-critical data for LCH priority enhancements. With a separate remaining time threshold, the gNB can configure a relatively larger remaining time threshold for delay-critical data determination for additional LCH priority application, which enable the UE to speed up the delay-critical data transmission earlier than DSR triggering.

- If a separate remaining time threshold for LCH priority adaptation larger than the DSR triggering remaining time threshold is configured, the risk of the occurrence of large amount of delay-critical data upon DSR triggering can be reduced.

- Introduce a separate remaining time threshold for determining the delay critical data, for which the additional LCH priority is to be applied.

Yet there is another issue on whether or how to handle the additional priority impact on the intra-UE prioritization. It is well known that intra-UE prioritization is specified to prioritize the transmission of URLLC service which is specific for industry wireless. Firstly, it is doubtful whether it is a reasonable assumption that one UE could have concurrent URLLC service and XR service. Furthermore, we need also think that whether the gNB can ensure desired intra-UE prioritization procedure without specification impact even if one UE “could” have the concurrent XR service and the URLLC service. As far as our understanding, the following assumptions should be considered:

- In the first aspect, in previous discussions, there has been no convincing evidence to indicate that it is a reasonable application scenario for one XR UE to have concurrent XR service and URLLC service, i.e. it is less likely that one UE would have simultaneous URLLC service and XR service.

- In the second aspect, even if one UE may have concurrent URLLC service and XR service, the gNB should be able to ensure that both the additional LCH priority and the original LCH priority for XR service data should be lower than that of the URLLC service data in the specific case. With this assumption, the URLLC service data transmission would not be blocked by the additional LCH priority of XR data due to the URLLC service data always has higher priority than the XR service data no matter whether the original LCH priority or additional priority is applied for the XR service data.

- It is less likely that one UE has concurrent URLLC service and XR service.

- Even if one XR UE has concurrent URLLC service and XR service, by proper additional LCH priority configuration, gNB can ensure that URLLC service data always has higher priority than the XR service data, regardless of whether the additional LCH priority or the original LCH priority is applied for the XR data.

Based on the above assumption, no special enhancement is required for the intra-UE prioritization procedure due to introducing the additional LCH priority.

- No enhancement is needed for intra-UE prioritization procedure due to the additional LCH priority.

- Conclusion

In this contribution, we have discussed the LCP enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	Applying additional priority only for delay-critical data for an LCH may require the UE to evaluate the priorities among LCHs twice for the LCH corresponding to one MAC PDU construction.

Observation 2	For MAC PDU construction, the LCH with delay-critical data may not be able to be allocated enough radio resource due to its buckets (Bj) exhausted.

Observation 3	If a separate remaining time threshold for LCH priority adaptation larger than the DSR triggering remaining time threshold is configured, the risk of the occurrence of large amount of delay-critical data upon DSR triggering can be reduced.

Observation 4	It is less likely that one UE has concurrent URLLC service and XR service.

Observation 5	Even if one XR UE has concurrent URLLC service and XR service, by proper additional LCH priority configuration, gNB can ensure that URLLC service data always has higher priority than the XR service data, regardless of whether the additional LCH priority or the original LCH priority is applied for the XR data.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	Once the additional priority is determined for an LCH, the additional priority should be applied to all data of the LCH, including both delay critical data and non-delay critical data.

Proposal 2	Whether additional priority or original priority is applied should be determined for a LCH upon a MAC PDU construction, the resulted priority (either original priority or additional one) should be applied for the whole MAC PDU construction without change.

Proposal 3	In addition to LCH priority adaptation, RAN2 should also consider the option to allow UE to temporarily raise the rate limit (e.g. Bj) of the LCH with delay-critical data.

Proposal 4	Introduce a separate remaining time threshold for determining the delay critical data, for which the additional LCH priority is to be applied.

Proposal 5	No enhancement is needed for intra-UE prioritization procedure due to the additional LCH priority.

- References

- RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

- RAN2-126, Chair notes

- RAN2-127, Chair notes