3GPP TSG-RAN WG2 Meeting #127bis		R2-2408122

Hefei, China, 14th – 28th Oct. 202

Source:	vivo

Title:	Discussion on DSR enhancements for XR

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

- Introduction

In RAN#105, the WID of R19 XR was updated, and there were the following descriptions regarding DSR enhancement[1]:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In this contribution, the DSR related enhancements for multiple pairs of remaining time and buffer size for an LCG are discussed.

- Discussion

In RAN2#126 [2], there were the following agreements regarding DSR enhancements:

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In RAN2#127 [3], there were the following agreements regarding DSR enhancements on multiple pairs of remaining time and buffer size:

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In the following, the DSR enhancements for multiple pairs of remaining time and buffer sizes for an LCG were discussed based the open issues corresponding to the FFS issues concluded in previous meetings.

It was already agreed that single DSR triggering threshold will be supported.  As in R18, a single DSR triggering threshold was already specified. In our understanding, the basic usage for DSR in R18 is to request the uplink grant for timely transmitting delay-critical data. We don’t think that the motivation for DSR has been changed due to introducing multiple pairs of remaining time and buffer sizes per LCG. From this perspective, there is no need to introduce a new DSR triggering threshold for reporting multiple pairs of remaining time and buffer sizes for one LCG. In the meanwhile, in R18, the single DSR triggering threshold was used for delay-critical data determination. Considering the motivation of DSR is the same as in R18, the DSR triggering threshold being reused for delay-critical data identification and the delay-critical data volume calculation should be inherited for R19 DSR enhancement.

- The existing DSR triggering remaining time threshold should be one of the DSR reporting thresholds.

- The DSR triggering remaining time threshold should be reused for the delay-critical data volume calculation for DSR report with multiple pairs of remaining time and buffer size.

For an LCG configured with multiple pairs of remaining time and buffer sizes, the DSR reporting thresholds other than the DSR triggering threshold can be either higher or lower than the DSR triggering threshold. For a DSR reporting threshold higher than the DSR triggering threshold, it is used to configure a UE to report a pair of remaining time and buffer size for non-delay-critical data; while for an DSR triggering threshold lower than the DSR triggering threshold, it is used to configure a UE to report an additional pair of remaining time and buffer size for delay-critical data. We think it should be up to gNB to configure the multiple reporting thresholds are either higher or lower than the DSR triggering threshold or both, which keeps the network flexibility.

- DSR reporting thresholds for one LCG are configured by gNB, wherein the DSR reporting thresholds can be either higher or lower than the DSR triggering threshold.

In the following figure of example, one LCG is configured with 3 reporting thresholds, comprising one DSR triggering threshold, one high DSR reporting threshold higher than the DSR triggering threshold and one low DSR reporting threshold lower than the DSR triggering threshold, wherein the UE can report up to 3 pairs of remaining time/buffer sizes for this LCG (the first pair and the second pair for delay-critical data and the third pair for non-delay-critical data).

Fig.1 An example of DSR with multiple pairs of remaining time and buffer size for an LCG

For an LCG configured with N (N>1) pairs of remaining time/buffer sizes, it is not likely that this LCG always has N pairs of remaining time/buffer sizes to be reported once DSR for this LCG has been triggered. If always N pairs of remaining time/buffer sizes are reported for this LCG once DSR has been triggered by any LCH of this LCG, some pairs may have to be reported with zero buffer sizes in case there is no data for these pairs, which increases the DSR overhead. To avoid reporting the pairs with zero buffer size for an LCG, it is necessary to introduce a pair presence indicator per LCG to indicate which pairs are reported in the DSR for this LCG.

- New DSR format should comprise the pair presence indicator for DSR with multiple pairs of remaining time/buffer sizes for one LCG.

For new DSR format with multiple pairs of remaining time/buffer sizes for one LCG, no matter always reporting all configured pairs of remaining time/buffer sizes for the LCG, or only reporting the pairs of remaining time/buffer sizes with non-zero buffer size for the LCG, the overhead of new DSR format would be larger than the legacy DSR format.

- New DSR format for multiple pairs of remaining time/buffer size per LCG has larger overhead than legacy DSR format.

As the overhead of the new DSR format is larger than the legacy one in principle, to save the overhead, it is beneficial to allow a UE to select the legacy DSR format or new DSR format conditionally according to whether there is any LCG having multiple pairs of remaining time/buffer size to be reported or not. For instance, if none of the LCGs with pending DSRs has more than one pairs of remaining time/buffer size to be reported, the legacy DSR format should be selected by the UE, regardless of whether any one of these LCGs has been configured with multiple pairs of remaining time/buffer size or not. Otherwise, i.e. if any of the LCGs with pending DSRs has more than one pairs of remaining time/buffer size to be reported, the new DSR format should be selected.

- Only when any LCG is configured with multiple pairs of remaining time/buffer sizes and has multiple pair remaining time/buffer sizes to be reported, new DSR format should be selected; Otherwise (i.e. non of the LCGs with pending DSRs has more than one pairs of remaining time/buffer size to be reported), legacy DSR format should be selected.

Regarding whether PSI information is reported in DSR, it depends on whether per PDU set delay status should be supported or not, i.e. whether one pair of remaining time/buffer size per PDU set is supported or not. While currently, for no matter the single pair remaining time/buffer size per LCG specified in R18 or multiple pairs of remaining time/buffer size per LCG based on multiple DSR reporting thresholds in R19 being discussed, the data are categorized to the corresponding pair according to the remaining time to the remaining time threshold mapping, i.e. neither R18 DSR nor R19 DSR enhancement is designed for per PDU set per LCG delay status report. With this, there is no foundation to support per PDU set importance report in DSR in R19.

- Per PDU set importance report in DSR is not supported in R19.

- Conclusion

In this contribution, we have discussed the DSR enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	New DSR format for multiple pairs of remaining time/buffer size per LCG has larger overhead than legacy DSR format.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	The existing DSR triggering remaining time threshold should be one of the DSR reporting thresholds.

Proposal 2	The DSR triggering remaining time threshold should be reused for the delay-critical data volume calculation for DSR report with multiple pairs of remaining time and buffer size.

Proposal 3	DSR reporting thresholds for one LCG are configured by gNB, wherein the DSR reporting thresholds can be either higher or lower than the DSR triggering threshold.

Proposal 4	New DSR format should comprise the pair presence indicator for DSR with multiple pairs of remaining time/buffer sizes for one LCG.

Proposal 5	Only when any LCG is configured with multiple pairs of remaining time/buffer sizes and has multiple pair remaining time/buffer sizes to be reported, new DSR format should be selected; Otherwise (i.e. non of the LCGs with pending DSRs has more than one pairs of remaining time/buffer size to be reported), legacy DSR format should be selected.

Proposal 6	Per PDU set importance report in DSR is not supported in R19.

- References

- RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

- RAN2-126, Chair notes

- RAN2-127, Chair notes