3GPP TSG-RAN WG2 Meeting #127bis		R2-2408123

Hefei, China, 14th  – 18th Oct. 2024

Source:	vivo

Title:	Discussion on RLC enhancement for XR

Agenda Item:	8.7.5

Document for:	Discussion and Decision

- Introduction

In RAN2#127, the following agreements were made for RLC enhancement [1]:

- Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. - In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. - In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In this paper, we will discuss the enhancements to ensure timely RLC retransmission and to avoid unnecessary retransmission, and evaluate the pros and cons of each proposed solutions for these two directions.

- Discussion

  - Enhancements to ensure timely RLC retransmissions

- Approach 1: Autonomous retransmission

Autonomous retransmission is a kind of proactive retransmission, allowing the transmitting AM RLC entity to autonomously perform RLC retransmissions without waiting for the RLC NACK feedback. In this approach, the transmitting AM RLC entity may decide to perform blind retransmissions based on traffic needs (e.g., the delay budget of the RLC SDU) or the HARQ feedback from the physical layer (e.g., the number of HARQ retransmission failures).

In XR, one of the common cases of late PDUs is congestion. Thus, the congestion scenario is a primary scenario that requires optimization. In a congested scenario, autonomous retransmission may have a negative impact on the system and worsen the congestion, as the transmitting RLC entity may perform unnecessary RLC retransmission, leading to the wastage of radio resources, even if the packet has already been successfully transmitted.

Observation 1: Autonomous retransmission may make the congestion worse.

For autonomous retransmission based on HARQ feedback, it is possible for the network to perform DL RLC autonomous retransmission based on the HARQ feedback. However, in the case of UL, there is no HARQ feedback and the UE is unable to determine whether the HARQ transmission is successful or the network just gives up the HARQ retransmission when the NDI is toggled. As a result, it is not feasible for the UE to perform UL autonomous RLC retransmissions based on the HARQ feedback.

Observation 2: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Considering the negative effects of autonomous retransmission and the relatively large specification impact. We think there is no need to support the autonomous retransmission in Rel-19.

Proposal 1: Autonomous retransmission is not supported in Rel-19.

- Approach 2: Enhance the status report or polling mechanism to speed up the RLC retransmission

In this approach, several solutions have been proposed to reduce the time for status reporting, so that the transmitting RLC entity can initiate retransmission for the NACK RLC SDU timely.

Solution 1: Retransmission based on enhanced status report

Since we already agreed that RAN2 focuses on the RLC enhancements for UL traffic, the status reporting enhancement is not needed because the network can trigger the status reporting based on implementation. For example, the network can set a short t-reassembly timer to achieve fast status reporting to ensure timely RLC retransmissions on the UE side.

RAN2#126 - RAN2 focuses on the enhancements for UL traffic

Proposal 2: Enhancement on status report triggering is not needed for UL traffic.

Solution 2: Retransmission based on enhanced polling

In this solution, the polling mechanism is enhanced to guarantee timely reception of status report for the delay-critical RLC SDUs.

Regarding enhancements to the polling mechanism, one option is to define new polling triggering mechanism (e.g., triggering polling based on the delay budget of the RLC SDU). Another option is to reuse the existing polling triggering mechanism and configure smaller values for the existing timer and counters (e.g., t-PollRetransmit, pollPDU, and pollByte) to reduce the time for triggering status reporting. In the latter option, whether smaller values with finer granularity for the existing timer and parameters need to be introduced is FFS.

In our view, both the two options of polling enhancement can effectively ensure timely retransmission. However, the second solution is preferred due to its simplicity. Given that only the header and ACK_SN are mandatory present in the STATUS PDU, more frequent STATUS reporting for the second solution is not expected to introduce too much radio resources wasting. Details on whether smaller values with finer granularity for the existing timer and parameters need to be introduced could be further discussed after this approach proceeds.

Comparation﻿ between Solution 1 and Solution 2:

The pros and cons of the above solutions for ensuring timely RLC retransmission(s) are summarized in the following table:

Table 1: Pros and Cons of different approaches to ensure timely RLC retransmission(s)

| Approach description | Pros | Cons |
| --- | --- | --- |
| Autonomous retransmission | - Fast RLC retransmission can be achieved | - May introduce unnecessary retransmission and make the congestion worse - Retransmission based on the HARQ feedback is not feasible for UL - Large spec impact when introducing a new retransmission mechanism |
| Enhance the status report or polling mechanism | - Fast RLC retransmission can be achieved - Little specification impact | - Smaller values with finer granularity for polling related timer or counters may need to be introduced to match with XR traffic requirement |

According to the analysis, we prefer to reuse the current status report and polling mechanism to ensure timely RLC retransmission. Whether smaller values with finer granularity for polling related timer (e.g., t-PollRetransmit) or counters (e.g., pollPDU and pollByte) need to be introduced for fast STATUS report triggering can be discussed later.

Proposal 3: Proper configuration and current mechanism for triggering status report or polling is enough to ensure timely RLC retransmission. Whether smaller values with finer granularity for polling related timer (e.g., t-PollRetransmit) or counters (e.g., pollPDU and pollByte) need to be introduced for faster STATUS report triggering is FFS.

  - Enhancements to avoid unnecessary retransmissions

For the enhancement to avoid unnecessary retransmission, three approaches have been identified in RAN2 and some progress on the procedures are agreed as follows.

Table 2: Approaches to avoid unnecessary retransmissions

| Approach | Procedure |
| --- | --- |
| Tx initiated approach | - The Tx side notifies the Rx side about the obsolete SDUs - Tx side stops retransmitting obsolete SDUs - Rx side updates state variables according to the information from the Tx side |
| Rx initiated approach | - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the Rx side to indicate abandoned SDUs to the Tx side. - Tx side just processes the status report as in the legacy. - Rx side abandons the missing SDUs like already done by PDCP, i.e. based on a timer. |
| Combined Rx and Tx approach | - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach |

In the following, we will discuss the details of these approaches and analyze the pros and cons of each approach.

- Approach 1:Tx initiated approach

In this approach, the Tx side determines whether RLC retransmission for certain packets is unnecessary and could be skipped. The Tx side RLC entity may make the decision based on the delay information of each packet, such as the remaining time, or based on the indication from the PDCP layer at Tx side. Given that the delay-related information is maintained at the Tx side, it is natural and straightforward for the Tx to make the decision.

Observation 3: Delay-related information for packets (e.g., remaining time) is maintained at the Tx side, it is natural and straightforward for the Tx side to decide whether to stop RLC retransmission for specific packet(s).

Regarding the window synchronization between the Tx side and the Rx side, RAN2 has agreed that the Rx window advances before the Tx window advances is the baseline, and the Tx window advances first is FFS.

RAN2#127 - Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.

For the case of Rx window advances before Tx window advances, the procedure works as follows:

  - The Tx side notifies the Rx side about the obsolete SDUs

  - The Tx side stops retransmitting obsolete SDUs

  - Rx side updates state variables according to the information from the Tx side

  - Rx side sends status report with ack for the obsolete SDUs

  - Tx side updates state variables according to the status report

For the case of the Tx window advances before the Rx window advances, the procedure works as follows:

- The Tx side notifies the Rx side about the obsolete SDUs

- The Tx side stops retransmitting obsolete SDUs and updates state variables

- Rx side updates state variables according to the information from the Tx side

- Rx side sends status report with ack for the obsolete SDUs

In our view, the Tx and Rx window synchronization can be guaranteed even if the Tx side advances the window first, as long as the notification about the obsolete SDUs sent from the Tx side is successfully received by the Rx side.

Observation 4: The Tx and Rx window synchronization can be guaranteed in Tx initiated approach, no matter advancing Rx window first or advancing Tx window first.

Since the Tx side first knows the obsolete SDUs in Tx initiated approach, advancing the Tx side window first shows benefit than advancing the Rx side window first from latency and procedure points of view.

According to the AM data transfer mechanism, when the Tx side window advances, new RLC SDUs from PDCP can be delivered to the RLC layer so that these RLC SDUs can be transmitted timely. While for the case of the Rx side window advances first, the Tx side window can only be advanced after receiving feedback from the Rx side, which can lead to a waste of transmission opportunities because the transmission window is stuck and new packets cannot be transmitted before receiving the feedback from Rx side.

Considering that the motivation of RLC enhancement for XR is to avoid unnecessary retransmission for obsolete SDUs and ensure that the PDB/PSDB of subsequent packets is fulfilled, advancing Tx side window first should be applied to Tx initiated approach.

Proposal 4: In Tx initiated approach, the Tx window advances first, and the Rx window advances after receiving the notification about the obsolete SDUs from the Tx side.

- Approach 2:Rx initiated approach

In this option, the Rx side decides whether the packets in the transmitter should stop RLC retransmission and notify the Tx side. However, since the delay information of packets is maintained at the Tx side and only the Tx side knows whether the delay for a PDU/PDU set will meet the PDB/PSDB requirements or is about to exceed the PDB/PSDB requirements, it is difficult for the receiver to decide which packets should skip retransmission and when to skip.

Observation 5: Since the packet status is maintained at the transmitter, it is difficult for the receiver to determine whether an RLC SDU is obsolete or not.

In the previous meetings, some companies proposed introducing a local timer at the receiver and the receiver will notify the transmitter to stop RLC retransmission for the packets when the timer expires. However, the solution is not feasible because the receiver does not know when the packet arrives at the transmitter buffer and it is hard to set the length of the timer and to decide when to start the timer. Besides, introducing a new timer at the receiver will bring a huge specification impact and is very complex.

Observation 6: The Rx side has no information of when the packet arrives at the Tx side buffer, so it is not feasible to decide the length of the timer and decide when to start the timer for Rx initiated approach.

- Approach 3: Combined approach

According to the agreements from RAN2#127 meeting, the combined Rx and Tx approach was identified, which means that both Tx initiated approach for stopping RLC SDUs retransmission and Rx initiated approach for stopping RLC SDUs receiving are applied. However, it is not clear how to ensure the Tx and Rx window synchronization for the combined Rx and Tx approach.

From our understanding, there are mainly 4 alternatives on how to ensure the Tx and Rx window synchronization for the combined Rx and Tx approach.

Table 3: Alternatives for combined Rx and Tx approach

| Alternatives | Procedures |
| --- | --- |
| Alt 1: Both notifications about the obsolete SDUs from Tx to Rx and Rx to Tx are provided. | - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach; - Tx side notifies the Rx side about the obsolete SDUs and Rx side updates state variables according to the information from the Tx side; - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach; - Rx side notifies the Tx side about the obsolete SDUs and Tx side just processes the status report as in the legacy; |
| Alt 2: No notification about the obsolete SDUs from Tx to Rx or Rx to Tx is provided | - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach; - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach; - No notification about obsolete SDUs are provided from the Tx side to Rx side or from the Rx side to Tx side |
| Alt 3: Only notification about the obsolete SDUs from Rx to Tx is provided | - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach; - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach; - Rx side notifies the Tx side about the obsolete SDUs and Tx side just processes the status report as in the legacy; |
| Alt 4: Only notification about the obsolete SDUs from Tx to Rx is provided | - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach; - Tx side notifies the Rx side about the obsolete SDUs and Rx side updates state variables according to the information from the Tx side; - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach ; |

Considering the 4 alternatives of combined approaches above, Alt 1 can best ensure the Tx side and Rx side window synchronized because information about the obsolete SDUs is fastly shared between the Tx side and Rx side, and this ensures that the Tx window and Rx window are always synchronized.

For Alt 2, no information about the obsolete SDUs is exchanged between the Tx side and the Rx side. In this case, the Tx and Rx window may not be synchronized in time, because the Tx side and Rx side decide obsolete SDUs based on different mechanisms and there will be timing difference for a packet for the Tx side to stop transmission and for Rx side to stop receiving. This kind of window unsynchronization will cause additional transmission delay for the subsequent packets.

For Alt 3, only the Rx side provides information about the obsolete SDUs for the Tx side to update state variables, so it has a similar problem with Alt 2. If the Tx side stops retransmission for obsolete SDUs and advances the Tx window first, the Rx side may need to wait for the local timer to expire and stop receiving the corresponding obsolete SDUs on the Tx side. During this time, new packets sent from the Tx side cannot be received by the Rx side because the Rx window is stuck without the notification from the Tx side.

For Alt 4, only the Tx side provides information about the obsolete SDUs for the Rx side to update state variables. In this case, if the Rx side stops retransmission for obsolete SDUs and advances the Rx window first, the Tx side may need to wait for the packet to expire on the Tx side. During this time, new packets cannot be sent from the Tx side because the Tx window is stuck without the notification from the Rx side.

Observation 7: For the combined approach, information about obsolete SDUs both from Tx side to Rx side and from Rx side to Tx side needs to be provided to ensure the Tx side and Rx side window are always synchronized.

From the analysis above, alternative 1 for combining Rx and Tx side approach is a good compromise since it can ensure timely Rx and Tx window synchronization.

Proposal 5: For combined Rx side and Tx side approach, the following procedure is applied:

- Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach;

- Tx side notifies the Rx side about the obsolete SDUs and Rx side updates state variables according to the information from the Tx side;

- Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach;

- Rx side notifies the Tx side about the obsolete SDUs and Tx side just processes the status report as in the legacy.

The pros and cons for the above solutions and the corresponding specification impacts to avoid unnecessary RLC retransmission(s) are summarized in the following table:

Table 3: Pros and Cons of different approaches to avoid unnecessary RLC retransmission(s)

| Approach description |  | Pros | Cons | Spec impacts |
| --- | --- | --- | --- | --- |
| Tx initiated approach |  | - Packet status is maintained at the Tx side and Tx side can accurately know whether an RLC SDU is obsolete. | - No cons | - Notification about obsolete SDUs from Tx side to Rx side is needed |
| Rx initiated approach |  | - Status report can be reused for Rx side to indicate Tx side about the obsolete SDUs | - Rx side does not know the arrival time of packets and it is hard to decide whether and when a packet is obsolete. | - A new local timer for rx side to decide whether packets are obsolete needs to be introduced |
| Combined Rx and Tx approach | Alt 1: Both notifications about the obsolete SDUs from Tx to Rx and Rx to Tx is provided. | - Can ensure timely window sync between Tx side and Rx side | - No cons | - A new local timer for rx side to decide whether packets are obsolete needs to be introduced - Notification about obsolete SDUs from Tx side to Rx side is needed |
|  | Alt 2: No notification about the obsolete SDUs from Tx to Rx or Rx to Tx is provided | - No information about Obselte needs to be exchanged between Tx side and Rx side | - Cannot ensure window sync and may bring additional delay for packet transmission | - A new local timer for rx side to decide whether packets are obsolete needs to be introduced |
|  | Alt 3: Only notification about the obsolete SDUs from Rx to Tx is provided | - No information about obsolete SDU from Tx side to Rx side needs to be exchanged | - Cannot ensure window sync and may bring additional delay for packet transmission | - A new local timer for rx side to decide whether packets are obsolete needs to be introduced |
|  | Alt 4: Only notification about the obsolete SDUs from Tx to Rx is provided | - No information about obsolete SDU from Rx side to Tx side needs to be exchanged | - Cannot ensure window sync and may bring additional delay for packet transmission | - A new local timer for rx side to decide whether packets are obsolete needs to be introduced - Notification about obsolete SDUs from Tx side to Rx side is needed |

- Conclusion

In this contribution, we discuss the two directions of RLC AM enhancements, and we have the following observations and proposals:

Enhancements to ensure timely RLC retransmission(s)

Observation 1: Autonomous retransmission may make the congestion worse.

Observation 2: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Proposal 1: Autonomous retransmission is not supported in Rel-19.

Proposal 2: Enhancement on status report triggering is not needed for UL traffic.

Proposal 3: Proper configuration and current mechanism for triggering status report or polling is enough to ensure timely RLC retransmission. Whether smaller values with finer granularity for polling related timer (e.g., t-PollRetransmit) or counters (e.g., pollPDU and pollByte) need to be introduced for faster STATUS report triggering is FFS.

Enhancements to avoid unnecessary retransmissions

Observation 3: Delay-related information for packets (e.g., remaining time) is maintained at the Tx side, it is natural and straightforward for the Tx side to decide whether to stop RLC retransmission for specific packet(s).

Observation 4: The Tx and Rx window synchronization can be guaranteed in Tx initiated approach, no matter advancing Rx window first or advancing Tx window first.

Observation 5: Since the packet status is maintained at the transmitter, it is difficult for the receiver to determine whether an RLC SDU is obsolete or not.

Observation 6: The Rx side has no information of when the packet arrives at the Tx side buffer, so it is not feasible to decide the length of the timer and decide when to start the timer for Rx initiated approach.

Observation 7: For the combined approach, information about obsolete SDUs both from Tx side to Rx side and from Rx side to Tx side needs to be provided to ensure the Tx side and Rx side window are always synchronized.

Proposal 4: In Tx initiated approach, the Tx window advances first, and the Rx window advances after receiving the notification about the obsolete SDUs from the Tx side.

Proposal 5: For combined Rx side and Tx side approach, the following procedure is applied:

- Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach;

- Tx side notifies the Rx side about the obsolete SDUs and Rx side updates state variables according to the information from the Tx side;

- Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach;

- Rx side notifies the Tx side about the obsolete SDUs and Tx side just processes the status report as in the legacy.

- References

- R2_127_ChairNotes.